3GPP TSG-SA WG2 Meeting #158	S2-2309393

Goteborg, Sweden, 21-25 August 2023

Title:                       Reply LS on NSAC procedure for counting of UEs with at least one PDU session/PDN connection

Response to:         LS on Clarification on NSAC procedure for counting of UEs with at least one PDU session or PDN connection (S2-2308306/C4-232530)

Release:                 Rel-18

Work Item:	eNSAC

Source:	SA2

To:	CT4

Cc:	CT3

Contact person:

#### Name:	Peng Tan

#### E-mail Address:	v-tanpeng@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks CT4 for the LS in S2-2308306/C4-232530. SA2 discussed the questions in the LS and agreed on the following answers to CT4 questions.

Question 1: Which service operation (NumOfUEsUpdate or NumOfPDUsUpdate) should be used by the SMF/SMF+PGW-C to trigger the counting of the number of UEs registered with at least one PDN connection / PDU session?

SA2 Answer:

Clause 5.15.11.5a of TS 23.501 specifies both service operations, Nnsacf_NSAC_NumOfUEsUpdate_Request and Nnsacf_NSAC_NumOfPDUsUpdate_Request, could be used by the SMF/SMF+PGW-C to trigger the counting of the number of UEs registered with at least one PDN connection/PDU session.

As further specified in this clause, A PLMN can deploy one of the above two options for a slice when EPS counting is required for a network slice and NSACF is configured with maximum number of UEs with at least one PDU Session/PDN Connection.

Question 2: If the answer to Q1 is the NumOfUEsUpdate, should the SMF/SMF+PGW-C send 2 requests, one for NumOfUEsUpdate and one for NumOfPDUsUpdate, to check the number of UEs with at least PDN connection and the max number of PDUs, or only one request (i.e. NumOfUEsUpdate) to check both criteria?

SA2 Answer:

To check both criteria, SMF/SMF+PGW-C should send two requests, one for NumOfUEsUpdate, and the other for NumOfPDUsUPdate.

Question 3: Should the NF service consumer of NSACF exposure service (e.g. NEF, AF):

- explicitly subscribe to the event for the number of registered UEs, or the event for the number of UEs with at least one PDN connection / PDU session implying that it is aware of which of two mechanisms is applied to the S-NSSAI, or

- subscribe to both events (number of registered UEs and number of UEs registered with at least one PDN connection/PDU session) and the NSACF will notify the appropriate event based on the mechanism (registered UEs or registered UEs with at least one PDN connection/PDU session) that is configured for the corresponding S-NSSAI?

SA2 Answer:

There is no event, named “the number of UEs with at least one PDU connection/PDU session” in Nnsacf_SliceEventExposure service operations.

As specified in Clause 5.2.21.4 of TS 23.502, the NF service consumer of NSACF exposure service should explicitly subscribe to the event for the number of UEs registered with a network slice by invoking Nnsacf_SliceEventExposure_Subscribe.

In Nnsacf_SliceEventExposure_Notify service operation, the Event Reporting information parameter provides the network slice status information. The network slice status information may include information to indicate whether this is number of UE with at least one PDU Session/PDU Connection.

Two relevant service operations are provided below as reference.

Nnsacf_SliceEventExposure_Subscribe service operation is used by the consumer NF to subscribe or modify a subscription. It requires Event ID as input. The Event ID defines whether notify the number of UEs registered with a network slice or the number of PDU Sessions established on a network slice.

The other required input parameter, Event Filter, provides the option to report both the current number of UEs registered for a network slice and the current number of PDU Sessions established on a network slice.

# 2	Actions

To CT4

ACTION:

SA2 kindly asks CT4 to take above into account.

# 3	Dates of next TSG SA WG 2 meetings

SA2#159	9-13 October 2023	Xiamen, CN

SA2#160	13-17 November 2023	Chicago, US