# 3GPP TSG-SA WG2 Meeting #158	S2-2309724

August 21 - 25, 2023, Gothenburg, Sweden	(Revision of S2-2309599)

Title:	Reply LS on INACTIVE eDRX above 10.24sec and SDT

Response to:	LS on INACTIVE eDRX above 10.24sec and SDT (S2-2308320/R3-233347)

Release:	Rel-18

# Work Item:	     NR_REDCAP_Ph2, NR_redcap_enh-Core, NR_MT_SDT-Core

Source:	SA2

To:	RAN3, CT4

Cc:	RAN2

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 4770/4710, TS 23.502 CR 4322/4425

1. Overall Description:

SA2 thanks RAN3 for the LS (R3-233347) on “INACTIVE eDRX above 10.24sec and SDT” with the following bullets.

- RAN3 has agreed to include the RAN configured Paging Time Window (PTW) in the MT COMMUNICATION HANDLING REQUEST message, when the NG-RAN sends this message to request CN applying HLCOM.

- Following SA2 and CT4 updates, RAN3 has agreed to include the QFI, PPI, ARP and PDU session ID of the QoS Flow in the DL DATA NOTIFICATION message from AMF to NG-RAN requesting connection resume. Concerning signalling 5QI, RAN3 is still discussing it and would like to check with SA2, if the signalled 5QI can be different than the one associated with the stored QFI in the RAN UE context.

- Furthermore, RAN3 has discussed the need to signal the DL data size in the DL DATA NOTIFICATION message to help NG-RAN making decision if MT-SDT paging can be performed based on the QFI associated with the configured SDT radio bearer. RAN3 would like to ask SA2 and CT4 whether it is feasible that DL data size can be signalled to the NG-RAN (e.g. for QoS flows that have buffered data in the CN) along with the paging differentiation parameters to assist NG-RAN for MT-SDT paging decision.

For the information in bullet 1 of in the overall description related to NG-RAN provides of PTW parameter for RRC_INACTIVE eDRX to CN. SA2 has agreed the attached CRs (TS 23.501 CR 4770 and TS 23.502 CR 4322).

For the two questions in bullet 2 and 3, SA2 discussed the issues and provided the following responses:

Answer 2: When NGAP DL DATA NOTIFICATION is triggered by downlink data on user plane, the signalled 5QI is the same as the one stored in RAN UE context. For the case of NGAP DL DATA NOTIFICATION is triggered by MT signalling, SA2 will continue the discussion and provide further response.

Answer 3: SA2 discussed the issue and considers it’s feasible for UPF/SMF to provide the DL data size of the first downlink packet to NG-RAN via AMF as the agreed solution indicated in attached CRs (TS 23.501 CR 4710 and TS 23.502 CR 4425).

2. Actions:

To RAN3, CT4

ACTION: 	SA2 kindly asks RAN3 and CT4 to take above information into consideration.

3. Date of Next SA2 Meeting:

SA2#159   			October     09 - 13, 2023 				Xiamen, CN

SA2#160   			November 13 - 17, 2023 				Chicago, US