SA WG2 Meeting #S2-159	S2-2310077

09 - 13 October, 2023, Xiamen, P.R. China

SA WG2 Meeting #S2-158	S2-2308299

21 - 25 August, 2023, Goteborg, Sweden

3GPP TSG CT WG3 Meeting 128	C3-232510

Bratislava, Slovakia, 22nd - 26th May, 2023

Title:	LS on Support of multiple UEs in Northbound APIs

Response to:	NA

Release:	Rel-18

Work Item:	NBI18

Source:	CT3

To:	SA2, SA6

Cc:

Contact person:	Narendranath Durga Tangudu

n.tangudu@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

CT3 is currently working under NBI18 WID (which is a stage 3 only WI), on specifying the technical enhancements to the 3GPP Northbound and Application Layer interfaces and APIs. In this regard, CT3 is discussing the support of list of UE(s) in various northbound APIs like NEF APIs in TS 29.522, SCEF APIs in TS 29.122, SEAL APIs in TS 29.549 etc.

Currently, various northbound APIs only support either single UE information (external Identifier, MSISDN, Gpsi etc) or group of UEs (external group identifier, VAL group Identifier, etc). The northbound function (NEF, SCEF, Edge Enabler Server, SEAL server, etc) receiving the API request, may use the UE(s) information to further invoke the downstream 3GPP network services to fulfil the AF’s request.

When the AF has to invoke a northbound API targetting multiple UE(s) that do not belong to same group (identified by a group identifier like external group identifier), then the AF can either invoke the multiple northbound API individually for each UE or modify/create an external group for this set of UE(s) and invoke the northbound API for the created/modified group. CT3 has also observed that other APIs allow a third option, namely providing a list of UE(s) as the target of the request (instead of a GPSI or a group id).

Question 1: Do SA2 and SA6 believe that the addition of this option (i.e. the option to provide a list of UE(s) as the target) to any API can be implemented by stage 3 as a signalling optimization or are there pros and cons that render it necessary to determine the existence (or not) of this option in stage 2 on a case by case basis? The implications on the E2E procedures (whether downstream APIs, e.g. 5GC APIs, should also be updated to support this list of UE(s) mechanism) should also be considered.

# 2	Actions

To SA2, SA6 group

ACTION: 	CT3 kindly asks SA2 and SA6 to clarify on the above question and update the specifications accordingly (if required).

# 3	Dates of next TSG CT WG 3 meetings

CT3#129	21st – 25th August 2023 		Goteborg, Sweden

CT3#130	9th – 13th October 2023 		Xiamen, China