SA WG2 Meeting #S2-159	S2-2310091

09 - 13 October, 2023, Xiamen, P.R. China

3GPP TSG-CT WG1 Meeting #143	C1-236448

# Goteburg, 21– 25 August 2023

# Title:	LS on 5G ProSe UE-to-UE relay discovery

# Response to:

# Release:	Rel-18

# Work Item:	5G_ProSe_Ph2

Source:	CT1

To:	SA2

Cc:

Contact Person:

Name:	Haorui Yang

Tel. Number:

E-mail Address:	yanghaorui@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT1 has the following questions regarding 5G ProSe UE-to-UE relay discovery, for which CT1 would like to seek clarifications and guidance from SA2:

- According to clause 6.3.2.1 of TS 23.304, during 5G ProSe direct discovery procedures, the announcing UE/ discoverer UE/ discoveree UE "includes the relay_Indication if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Discovery".

However, the user info ID, used in the 5G ProSe UE-to-UE relay discovery, cannot be obtained by the 5G ProSe UE-to-UE relay UE during the 5G ProSe direct discovery. So the 5G ProSe UE-to-UE relay UE cannot identify the same 5G ProSe end UE who sent the relay indication by using this user info ID in the 5G ProSe UE-to-UE relay discovery. This makes the relay indication mechanism cannot work. Then:

Question 1: How can this issue be resolved?

- According to clause 6.3.2.4.4 of TS 23.304, for candidate 5G ProSe UE-to-UE Relay Discovery to support the negotiated Relay reselection, the discoverer End UE discovers a candidate 5G ProSe UE-to-UE Relay without sending the RSC and the User Info ID of the discoveree 5G ProSe End UE. Then:

Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly?

Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message?

- According to clause 5.8.4.2, 6.4.3.7.1 and 6.4.3.7.4 of TS 23.304, the user info of the target 5G ProSe end UE (i.e. discoveree end UE) is mandatory in 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) and Direct Communication Request message for 5G ProSe UE-to-UE Relay Communication; while it is optional in the Direct Communication Request message for 5G ProSe UE-to-UE relay communication with integrated discovery. Then:

Question 4: Why are different approaches applied to the user info of the target 5G ProSe end UE in the above procedures?

2. Actions:

To SA2 group.

ACTION: 	CT1 kindly asks SA2 to provide answers to the above questions.

3. Date of Next CT1 Meetings:

CT1#144	9th   – 13th October 2023 		China

CT1#145	13th   – 17th November 2023 		Chicago