SA WG2 Meeting #S2-159	S2-2310096

09 - 13 October, 2023, Xiamen, P.R. China

3GPP TSG-CT WG1 Meeting #143	C1-236521

# Goteborg, 21– 25 August 2023

# Title:	LS on providing a new 5G-GUTI in the REGISTRATION REJECT message to the UE

# Response to:

# Release:	Rel-18

# Work Item:	5WWC_Ph2

Source:	CT1

To:	SA3

Cc:	SA2

Contact Person:

Name:	Mohamed Amin Nassar

E-mail Address:	mohamed.a.nassar@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

CT1 has observed that SA3 WG has agreed CR 1622 on TS 33.501 (see S3-233272) as part of the security requirements for 5WWC_Ph2, where the CR states the following in clause 7.2.1 of TS 33.501:

19. The AMF shall send a Registration Reject message via N3IWF to the UE as defined in step 17 in clause 4.12.2.2 of TS 23.502[8]. The Registration Reject message is ciphered and integrity protected, and a new 5G-GUTI is provided to the UE.

And similar requirement is specified in clause 7A.2.1 of TS 33.501, due to the same CR mentioned above.

Currently in NAS specifications in TS 24.501 for Rel-18 (and also in all previous releases), the REGISTRATION REJECT message doesn't contain any 5G-GUTI information element. CT1 has some concerns about the above requirement from SA3 since it is CT1 understanding that a 5G-GUTI can be provided to the UE after a successful registration and a 5G-GUTI cannot be provided to the UE in the registration rejection case. This understanding from CT1 is supported by stage-2 specification, where the following is specified in clause 5.3.2.4 in TS 23.501:

"….The 5G-GUTI is assigned upon a successful registration of the UE, and is valid over both 3GPP and Non-3GPP access to the same PLMN for the UE…"

Hence CT1 would like to highlight the above concern to SA3 and to ask them to do any modification in the specification if needed.

2. Actions:

To SA3 group:

ACTION: 	CT1 would like to highlight the above concern to SA3 and to ask them to do any modification in the specification if needed.

3. Date of Next CT1 Meetings:

CT1#144	9th   – 13th October 2023 		China

CT1#145	13th   – 17th November 2023 		Chicago