3GPP TSG-SA WG2 Meeting #159	S2-2310234

# Xiamen, China, October 09 – 13, 2023								          (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0358 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of ENs having RAN dependency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following ENs with RAN dependency can be removed as no SA2 impact is identified, and reference to RAN specification is added. Editor's note: Mobility of 5G ProSe Layer-2 Remote UE between Direct and Indirect Network communication path will be defined by RAN WGs and alignment work (if any) will be made by SA2 based on RAN WGs conclusions. Editor's note: The procedures for Layer-2 Remote UE in CM-CONNECTED state switching the communication path to another Layer-2 UE-to-Network Relay will be aligned with RAN WG specifications. Editor's note: Alignment with RAN WGs is to be done. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the ENs with RAN dependency and add reference to RAN specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs with RAN dependency are unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.5.2.3	Path switching between direct network communication path and indirect network communication path via 5G ProSe Layer-2 UE-to-Network Relay

Xn based (as defined in clause 4.9.1.2 of TS 23.502 [5]) and N2 based (as defined in clause 4.9.1.3 of TS 23.502 [5]) Handover procedure is used for inter-gNB indirect-to-direct and inter-gNB direct-to-indirect path switching for 5G ProSe Layer-2 Remote UE in CM-CONNECTED with RRC_CONNECTED state.

* * * * 2nd Change * * * *

6.5.5.2	Procedures for Communication Path Switching between UE-to-Network Relays

This clause specifies the communication path switching procedures between two UE-to-Network Relays.

When performing the path switching between two 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, or between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-2 UE-to-Network Relay, or between 5G ProSe Layer-3 UE-to-Network Relay without N3IWF and 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the application layer procedures are used for service continuity support which is out of scope of this specification.

Path switching between two 5G ProSe Layer-3 UE-to-Network Relay with N3IWF is performed using the mechanism specified for Local mobility anchor within untrusted non-3GPP access networks using MOBIKE in clause 6.2.9 of TS 23.501 [4].

When a 5G ProSe-enabled UE as a 5G ProSe Layer-2 Remote UE in CM-CONNECTED state switches the communication path to Layer-3 UE-to-Network Relay with N3IWF, 5G ProSe-enabled UE's PDU session(s) can handover to the indirect path via Layer-3 UE-to-Network Relay with N3IWF by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

When a 5G ProSe-enabled UE as a 5G ProSe Layer-3 Remote UE switches the communication path from Layer-3 UE-to-Network Relay with N3IWF to an indirect path via Layer-2 UE-to-Network Relay, 5G ProSe-enabled UE's PDU session(s) can handover to the indirect path via Layer-2 UE-to-Network Relay by reusing the procedures specified in clause 4.9.2 of TS 23.502 [5].

Procedure on Path switching between two 5G ProSe Layer-2 UE-to-Network Relays is specified in TS 38.300 [12].

* * * * 3rd Change * * * *

6.9.3	Multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay

Procedure on Multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay is specified in TS 38.300 [12].

* * * * End of Changes * * * *