3GPP TSG-WG SA2 Meeting #159	S2-2310326

Xiamen, China, Oct 9 – 13, 2023	(revision of SP-2301103)

# Source:	Huawei (Moderator)

# Title:	New WID on Study on enhancement of Timing Resiliency, TSC&URLLC, and LAN

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on enhancement of Timing Resiliency, TSC&URLLC, and LAN

Acronym: FS_eTRS_URLLC_LAN

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  | X |
| Don't know |  |  | X |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820019 | Enhancement of Ultra-Reliable Low-Latency Communication support in the 5G Core network | Related Work Item in Release 16 (SA2) |
| 900008 | Support of Enhanced Industrial IIoT | Related Work Item in Release 17 (SA2) |
| 970024 | Timing Resiliency and URLLC enhancements | Related Work Item in Release 18 (SA2) |
| 970069 | Generic group management, exposure and communication enhancements | Related Work Item in Release 18 (SA2) |

Dependency on non-3GPP (draft) specification:

None

3	Justification

Time Resilience related:

SA1 specified requirements for 5G System to remain resilient if there is GNSS failure and for 5G System to act as a backup and offer wireless and indoor-capable time synchronization service for other applications (e.g. financial, power grid systems) in Rel-18. These requirements were targeting to position the 5G timing synchronization services can be resilient to loss of GNSS and can be trusted as a time source for critical infrastructure.

For high accuracy time service consumers, only some NG-RAN nodes may be upgraded to support the service due to propagation delay compensation budget limit. Hence, impact on connected mode mobility due to time synchronization service among RAN nodes with different time synchronization accuracies needs to be considered.

In addition to the ongoing resiliency study for time synchronization service in Rel-18 and its impacts for monitoring and reporting the service status in the specification, SA2 has introduced time synchronization service support in the previous releases. SA2 specified solution for time synchronization support using gPTP in Rel-16, AF requested time synchronization as a service (based on access stratum time distribution, called ASTI, or based on user plane packets using (g)PTP) support in Rel-17 and introduced the ability for subscription control and/or AF to activate / deactivate ASTI/PTP time synchronization service in a certain coverage area in Rel-18. However, focusing on ASTI service, the current specification still assumes that the reference time information (RTI) is broadcasted by the RAN based on OA&M mainly, thus the UE moving into a time synchronization coverage area (where it is supposed to receive time synchronization services) may not receive 5G RTI and the RAN may also be broadcasting RTI in cells/gNBs even if it is not part of the time synchronization coverage area resulting in wasted radio resources. That is, it remains unspecified in Rel-18 how the time synchronization coverage area can be ensured for UEs in RRC_INACTIVE/IDLE state.

Besides, cooperation between multiple mobile robots/AGVs is commonly needed in industry environment. In order to support the time synchronization service for one or more multiple ad-hoc cooperative groups, the 5GS needs to support the management of the establishment/update of the ad-hoc cooperative groups and its grand master clock.

URLLC related:

3GPP specifies replication mechanisms since Rel-16 for URLLC however the drawback is that the URLLC replication is either done for all traffic or no traffic for a given application. However, the current replication mechanism assumes that application layer protocols are used to control the traffic over multiple user plane paths for high reliability, with no control available within 3GPP system for operators.

Besides, capacity utilization is a key point for supporting demanding applications and services such as URLLC applications and XR services. ). But currently the network can only support limited terminals (due to URLLC devices requiring more recourse). It is noticed 5GS has transferred many unnecessary bits: For example, Ethernet has a minimum frame size of 64 Bytes, comprising an 18-Byte header and a payload of 46 Bytes. The application-layer packet payload of industrial applications is usually small, e.g. 20 Bytes. There are lots of duplicated packets in the industry scenario, the content (except the sequence number) in the payload of the packets does not change (unless there is a status update, such as an alarm).

UE-UE communication is widely used in industry scenario. However some features, e.g. BAT adjustment, Survival Time, cannot be used in UE-UE communication. For BAT adjustment in the UE2UE scenario, if the RANs provides the feedback (e.g. BAT offset) independently, the gap between the two RANs may not be synchronized, and the adjustment cannot be performed correctly. For Survival Time in the UE2UE scenario, it is unclear how the RAN nodes coordinate the packet loss, especially the packet is lost in the second air interface.

GMEC related:

The connections between different sites and private/public clouds can be controlled via e.g. SD-WAN or E/IP-VPN technologies in campus network deployment, there is a need to support 5G VN integration with legacy LAN network in scenarios like bank branches, chain stores, factory areas, enterprise campuses, and electric grids etc..

This corresponds to the leftover SA1 requirement in TS22.261, clause 6.24: The Ethernet transport service shall support the transport of Ethernet frames between a UE that an Ethernet device is connected to and an Ethernet network in DN (Data Network).

In addition to the ongoing GMEC study for configured connectivity between UPFs in GMEC work for cross SMF 5G VN group communication in Rel-18, continue resolving the existing challenges to enable support for dynamic routing among multiple SMF domains is required.

Further QoS monitoring enhancements is needed to enlarge the functionalities of QoS monitoring (includes features, additional monitored parameters) and provision of 5G communication services end-to-end within 5GS is required: Request, modification, and release of communication services must not be contingent on knowledge of internal 5G system parameters/architecture/setup.

4	Objective

Following are the objectives for this study:

WT-1: Time Synchronization enhancements

WT-1.1:   How to support to activate/deactivate Access Stratum based time synchronization via SIB9 based on cell level Area of Interest.

WT-1.2    Impact on connected mode mobility due to time synchronization service among RAN nodes with different time synchronization accuracies, i.e. support for service continuity between different clocks during HO.

WT-1.3    How to support time synchronization service for a group of UEs.

NOTE 1: This WT does not consider Clock synchronization to a group of UEs via ProSe direct/indirect communication.

WT 1.4 Void

WT-1.5: Study how to enable selective reference time delivery based on subscription for WB-EUTRA. It assumes the accuracy will not be changed in E-UTRAN.

- EPS Subscription enhancements for control of reference time delivery to the UE

- MME and eNB enhancements to inform eNBs about need for reference time delivery for the UE

NOTE 1a: WT-1.5 only requires alignment work in RAN3 (no work needed in RAN2) since delivering reference time  to individual UEs in dedicated RRC is already supported for WB-EUTRA.

NOTE 2: The solution defined in Rel-18 for selective reference time delivery based on subscription in 5GS can be taken as a baseline.

WT-1.6: Study how to enable clock quality information delivery based on subscription for WB-EUTRA

- EPS Subscription enhancements for control of clock quality delivery to the UE

- MME and eNB enhancements to provide clock quality information to UEs

NOTE Y: The solution defined in Rel-18 for delivery of clock quality information in 5GS can be taken as a baseline.

WT-2: General URLLC enhancements

WT-2.1 Support for 3GPP system-controlled E2E replication/elimination for redundancy for both Ethernet and IP type PDU Sessions, i.e. without relying on upper layer protocols to manage the replication and elimination of redundant packets/frames over the duplicate paths. The user plane for replication and elimination is based on IEEE FRER specifications for Ethernet and IETF DetNet specifications for IP.

The scenarios could be:

- The terminal and network side replication and elimination endpoint,

  - (WT-2.1.1.1) may be within the 5GS and 3GPP network and leverage 3GPP control plane signalling for session setup;

  - (WT-2.1.1.2) may be outside of the 5GS, and IEEE or IETF protocols may be used to carry the required FRER or DetNet configuration.

- The two PDU Sessions carrying the redundant traffic

  - (WT-2.1.2.1) May be for a single UE,

  - (WT-2.1.2.2) May be for two UEs that are independently registered.​

- The trigger for setting up the two PDU Sessions carrying the redundant traffic

  - (WT-2.1.3.1) may be pre-configured in the terminal;

  - (WT-2.1.3.2) may be triggered by the 5GS.

- The controller for setting up the replication and elimination

  - (WT-2.1.5.1) may be within the 5GS;

  - (WT-2.1.5.2) may be outside of the 5GS, with an interface to the required 5GS network functions.

The work task addresses architecture and protocol aspects for the 3GPP network operators on the above scenarios:

- Ability to trigger and support replication and elimination for redundancy, protocol aspects to replicate/eliminate.​

- Extended capability exposure framework to allow Application Function to discover, request or influence selective duplication, 5GS reliability and redundancy functionality.

WT 2.2 Support for Ethernet paddings removal to improve efficiency of transmission capacity.

WT 2.3 Void

WT 2.4 Support BAT adjustment based on BAT offset in UE-to-UE scenario considering two legs of the service.

WT 2.5 Support survival time in UE-to-UE scenario considering two legs of the service.

WT-3: Void

WT-4: Generic group management, exposure and communication enhancements

WT-4.1: How to support integration of 5G VN and legacy Ethernet LAN (device(s) behind UE  or DN)

- Integration architecture to support cross-site tunnel (between UPFs, between UPF and router in local DC) establishment and release, QoS provisioning and monitoring for cross-site tunnel, intelligent traffic steering on cross-site tunnel, exposure for reporting and configuration mapping.

- How to support for dynamic routing among multiple SMF domains,

- Enhancements in order to accommodate for multiple DN side devices and multiple UE side devices as 5G VN group members in order to enable the 5GC to be aware of the existence of the devices in the legacy LAN so to control the traffic routing inside the 5GS for such devices in the group communication

- Enhancements supporting dynamic addition of VN members, large numbers of VLANs.

- Enhancements supporting loop prevention/spanning tree protocols

- Study whether and how to support configuration by netconf/YANG within 5GS.

WT-4.2 Void

WT-4.3 Void.

WT-4.4 Void.

WT-4.5 Enhancements for provisioning of communication services  and for QoS Monitoring exposure to avoid being contingent on knowledge of internal 5G system parameters/architecture/setup.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 3 | 2.25 | Yes | WT#1 is self-contained |
| WT#1.1 | 1 | 0.5 |  |  |
| WT#1.2 | 0.5 | 0.5 |  |  |
| WT#1.3 | 0.5 | 0.25 |  |  |
| WT-1.5 | 0.5 | 0.5 |  |  |
| WT-1.6 | 0.5 | 0.5 |  |  |
| WT#2 | 3 | 2.5 | Maybe | WT#2 is self-contained |
| WT#2.1 | 1 | 1 |  |  |
| WT#2.2 | 1 | 0.5 |  |  |
| WT#2.4 | 0.5 | 0.5 |  |  |
| WT#2.5 | 0.5 | 0.5 |  |  |
| WT#4 | 2 | 2 | No | WT#3 is self-contained |
| WT#4.1 | 1.5 | 1.5 |  |  |
| WT#4.5 | 0.5 | 0.5 |  |  |

Total TU estimates for the study phase: 8

Total TU estimates for the normative phase: 6.75

Total TU estimates: 8 +6.75 = 14.75

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.abc | Study on enhancement of Timing Resiliency, TSC&URLLC, and LAN |  |  |  |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

7	Work item leadership

SA2

8	Aspects that involve other WGs

Security aspects are considered by SA3.

RAN aspects are considered by RAN WGs.

9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

Samsung

NICT

China Unicom

SIA