3GPP TSG-WG SA2 Meeting #159	S2-2310350

# Xiamen, China, October 09 – 13, 2023	(was S2- S2-2308580)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4736 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for QoS monitoring and network exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Lenovo, Tencent, Tencent Cloud, LG Electronics, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The existing Round trip delay measurement over multiple QoS flows actually includes two aspects: - Round trip delay for one service data flow, the UL traffic and DL traffic are separated into two QoS flows, as defined in 5.37.6. - Round trip delay for two service data flows, one UL direction service data flow and one DL direction service data flow, which are over two QoS flows respectively. Besides, in case of AF requested measurement, AF doesn’t know and care it is over one or multiple QoS flow(s). QoS flow mapping is within 5GS and transparent to AF. The aspects need further correction. Also, as requested by LS on Issues on QoS Monitoring parameters for XR service(S2-2310102), the QoS Monitoring request for QoS parameter needs to be clarified, e.g. reporting period, and periodic and event triggered reporting frequencies. Also, as requested by LS on Data rate monitoring (C4-233573/S2-2310108), monitoring averaging window for data rate needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Refine the Round trip delay measurement. - Other editorial changes. - Clarify QoS Monitoring request for QoS parameter. - Clarify monitoring averaging window for data rate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The desciption for QoS monitoring and network exposure is not sufficient clear and correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.4, 5.45.1, 5.45.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

5.37.4	Network Exposure of 5GS information

5GS and XR/media services cooperate to provide a better user experience using External Network Exposure.

Based on the AF request, the 5GS can expose the following information based on the QoS Monitoring as defined in clause 5.33.3 and/or clause 5.45:

-	The UL and/or DL congestion information monitoring (see clause 5.45.3)

Based on the PCC rule from PCF, the SMF requests the NG-RAN to report the information via GTP-U header to PSA UPF This NG-RAN reported information is common to support congestion information exposure and to support ECN marking for L4S in PSA UPF as described in clause 5.37.3.3. In the case of congestion information exposure, the PSA UPF exposes the UL and/or DL congestion information via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18. It can be applied to a Non-GBR or GBR QoS flow.

-	The UL and/or DL Data rate information (see clause 5.45.4):

Based on the PCC rule from PCF, the SMF requests the PSA UPF to measure and report the information. They may be exposed to the AF directly

by PSA UPF via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18.

-	The round trip delay for one service data flow or two service data flows.

In case the one service data flow are over two QoS flows (the UL traffic and DL traffic are separated into two QoS flows  see clause 5.37.6), or two service data flow(s) are over two QoS Flows of the XR service (e.g. UL direction service data flow and DL direction service data flow are bound to separate QoS flows respectively, see clause 5.37.6) in the same PDU Session, it is determined by triggering the QoS Monitoring for packet delay of individual QoS Flows as described in clause 5.33.3 and clause 5.45.2. The PCF derives the separate QoS monitoring policies for each direction in PCC rules respectively based on AF request. The PCF provides the two QoS Monitoring policies in the PCC rules for the XR service data flow(s). The PSA UPF reports the delay information per QoS Flow to the SMF. The SMF reports to PCF. The PCF derives round trip delay information based on the two direction’s packet delay result for the XR service data flow(s) and exposes the information to the AF directly or via NEF.

NOTE:	How PCF calculates the requested round trip delay for multiple QoS flows from delays of individual QoS Flows is not defined in this specification.

The AF may provide the Alternative QoS parameter set requirements and Averaging Window to the NEF/PCF for the GBR QoS Flow as specified in clause 4.15.6.6 of TS 23.502 [3].

Estimated bandwidth for 5QI may be exposed by NWDAF (according to information described in clause 6.9.2 of TS 23.288 [86]) to AF.

# * * Next change * * * *

5.45	QoS Monitoring

5.45.1	General

QoS monitoring comprises of measurements of QoS parameters and reports of the measurement result for a QoS Flow and can be enabled based on 3rd party application requests and/or operator policies configured in the PCF. The reports of the measurement result by PCF is specified in clause 6.1.3.18 of TS 23.503 [45] and the reports of the measurement result by UPF is specified in clause 5.8.5.12 and clause 5.8.2.18.

The AF may request measurements for one or more of the following QoS parameters, which may trigger QoS monitoring for service data flow(s):

-	UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

-	Packet Delay Variation, see clause 5.37.7.

-	Round trip packet delay when UL and DL are on different QoS flows, see clause 5.37.4.

The PCF may generate the authorized QoS Monitoring policy for a service data flow based on the QoS Monitoring request received from the AF (as described in clause 6.1.3.21 of TS 23.503 [45]) or based on PCF local policy or configuration reasons, such as PCF awareness of dynamic satellite backhaul connection. The PCF includes the authorized QoS Monitoring policy in the PCC rule and provides it to the SMF.

The QoS parameter(s) that can be measured by means of QoS monitoring are listed below. The QoS monitoring policy in PCC rule (described in clause 6.3.1 of TS 23.503 [45]) may include the following:

-	UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

The SMF configures the UPF to perform QoS monitoring for the QoS Flow and to report the monitoring results as described in clause 5.8.2.18 with parameters determined by the SMF based on the authorized QoS Monitoring policy received from the PCF or local configuration or both.

The SMF may also configure RAN to measure the QoS parameters based on the authorized QoS Monitoring policy received from the PCF and/or local configuration.

-    The QoS Monitoring request for packet delay is as described in clause 5.33.3, including QoS Monitoring indication and QoS Monitoring reporting frequency, as described as clause 4.3.3.2 of TS 23.502 [3];

-    With regard to the other QoS parameter (i.e. congestion information), the QoS Monitoring request for QoS parameter, provided by the SMF to the RAN, includes the following:

-	QoS parameters to be measured (i.e. congestion information);

-	Reporting frequency (event triggered, periodic):

-	if the reporting frequency is event triggered:

-	the corresponding reporting threshold to each QoS parameter;

-	minimum waiting time between subsequent reports;

-	the reporting period.

The following clauses describe the QoS parameters which can be measured and any specific actions or constraints for their measurement.

NOTE:	The QoS parameter which can be measured are parameters which describe the QoS experienced in the 5GS by the application, i.e. they are not restricted to the 5G QoS Parameters defined in clause 5.7.2.

5.45.2	Packet delay monitoring

QoS Monitoring for packet delay allows for the measurement of UL packet delay, DL packet delay or round trip packet delay between UE and PSA UPF. The details of the QoS Monitoring for packet delay are described in clause 5.33.3.

If the AF requests the QoS parameter(s) listed below, it can be derived by the PCF based on packet delay monitoring results of QoS flows.

-	Packet Delay Variation, see clause 5.37.7.

-	Round trip packet delay when UL and DL are on different QoS flows, see clause 5.37.4.

NOTE:	The round trip delay over two QoS flows can applies for one or two service flow(s).

5.45.3	Congestion information monitoring

The NG-RAN may be required to provide the UL and/or DL QoS Flow congestion information (i.e. a percentage of congestion level for exposure). The UPF may be required to monitor the UL and/or DL QoS Flow congestion information reported from the NG-RAN.

QoS monitoring request to the NG-RAN and NG-RAN reporting for UL and/or DL QoS Flow congestion information to PSA UPF is as defined in 5.37.3. The PSA UPF reports the received UL and/or DL QoS Flow congestion information to the target NF as instructed by the QoS Monitoring request from the SMF.

Only one of ECN marking for L4S (in the case of ECN marking for L4S in RAN as described in clause 5.37.3) or QoS monitoring of congestion information may be requested to NG-RAN for a QoS Flow. They are mutually exclusive, therefore, measurements of Congestion information on a QoS Flow are not provided in QoS Monitoring reports if SMF enables ECN marking for L4S in RAN (see clause 5.37.3).

5.45.4	Data rate monitoring

The QoS Monitoring for data rate allows the measurement of the UL and/or DL data rate per QoS flow at UPF and it can be applied to a Non-GBR or GBR QoS flow. The data rate is measured over a monitoring averaging window with a standardized value (i.e. 2000ms as the Default Averaging Window of the QoS flow as described in clause 5.7.4).

According to the QoS Monitoring request for UL and/or DL data rate from SMF, UPF is required to initiate data rate measurement for a QoS Flow and to report the measured data rate as instructed.

# * * * * End of changes * * * *