SA WG2 Meeting #S2-159	S2-2310372

09 - 13 October, 2023, Xiamen, P.R. China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0937 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve ENs on Pending Notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 7.4.4 and 8.2.4 FFS Editor’s Note on additional cause of Pending Notification need to be resolved. Since there are no use case requiring additional cause of pending notification at current stage, the FFS Ens can be directly removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the FFS ENs on Pending Notification in clause 7.4.4 Nnwdaf_DataManagement_Notify service operation and clause 8.2.4 Ndccf_DataManagement_Notify service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved FFS ENs in the frozen Rel-18 TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.4, 8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

### 7.4.4	Nnwdaf_DataManagement_Notify service operation

Service operation name: Nnwdaf_DataManagement_Notify.

Description: NWDAF notifies the consumer instance of the requested data, or notifies of the availability of previously subscribed Data when delivery is via an NWDAF. The NWDAF may also notify the consumer instance when Data or Analytics is to be deleted.

Inputs, Required: Notification Correlation ID, time stamp representing time when NWDAF completed preparation of the requested data.

Inputs, Optional:

-	Requested Data with timestamp;

-	Fetch Instructions;

-	Termination Request;

-	Data or Analytics Deletion Alert.

NOTE 1:	If the NWDAF has received the notifications from another source without a timestamp, then the NWDAF adds itself a timestamp based on the time it received the notification.

Fetch Instructions indicate whether the data are to be fetched by the Consumer. If the data are to be fetched, the fetch instructions include an address from which the data may be fetched, one or more Fetch Correlation IDs and a deadline to fetch the data (Fetch Deadline).

Termination Request indicates that NWDAF requests to terminate the data management subscription, i.e. NWDAF will not provide further notifications related to this subscription.

Pending Notification Cause indicates the cause of the pending notification of the stored unsent data, e.g., the data cannot be collected any more due to UE moved out of NWDAF serving area.

Data or Analytics Deletion Alert is described in clause 5B.1.

NOTE 2:	Data provided in notifications are processed and formatted according to the Processing and Formatting Instructions provided by the Consumer in Nnwdaf_DataManagement_Subscribe.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Data or Analytics Retrieval Indication.

The Data or Analytics Retrieval Indication may be sent if the notification contains a Data or Analytics Deletion Alert. The Data or Analytics Retrieval Indication indicates if the consumer will retrieve stored data or analytics prior to deletion (see clause 6.2B.3)

# *** 2nd Change ***

### 8.2.4	Ndccf_DataManagement_Notify service operation

Service operation name: Ndccf_DataManagement_Notify

Description: DCCF notifies the consumer instance of the requested data or analytics (which is regarded as a kind of data) according to the request, or notifies of the availability of previously subscribed Data or Analytics when data delivery is via the DCCF. The DCCF may also notify the consumer instance when Data or Analytics is to be deleted.

Inputs, Required: Notification Correlation Information, time stamp representing time when DCCF completed preparation of the requested data.

Inputs, Optional: Requested Data with timestamp or Analytics with timestamp, Fetch Instructions, Termination Request, Data or Analytics Deletion Alert.

NOTE 1:	If the DCCF has received the notifications from another source (e.g. NWDAF) without a timestamp, then the DCCF adds itself a timestamp based on the time it received the notification.

Fetch Instructions indicate whether the data or analytics are to be fetched by the Consumer. If the data or analytics are to be fetched, the fetch instructions include an address from which the data may be fetched, one or more Fetch Correlation IDs and a deadline to fetch the data (Fetch Deadline).

Termination Request indicates that the DCCF requests to terminate the data management subscription, i.e. DCCF will not provide further notifications related to this subscription.

Pending Notification Cause indicates the cause of the pending notification of the stored unsent data, e.g. the data cannot be collected any more due to UE moved out of DCCF serving area.

Data or Analytics Deletion Alert is described in clause 5B.1.

NOTE 2:	Data or Analytics provided in notifications are processed and formatted according to the Processing and Formatting Instructions provided by the Consumer in Ndccf_DataManagement_Subscribe.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Data or Analytics Retrieval Indication.

The Data or Analytics Retrieval Indication may be sent if the notification contains a Data or Analytics Deletion Alert. The Data or Analytics Retrieval Indication indicates if the consumer will retrieve stored data or analytics prior to deletion (see clause 6.2B.3).

*** End of Changes ***