3GPP TSG-SA WG2 Meeting #159	S2-2310411

# Xiamen, China, 09 – 13 October 2023								  	    (Revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0365 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PDU session establishment or modification for path switching from PC5 to Uu |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to clause 5.16, when switching between PC5 path and Uu path, the UEs first prepare the target path to communicate with each other and then perform the path switching. In case of path switching from PC5 to Uu, the concept seems just applied for UE-2 since the NOTE 1 in clause 6.8.2 states that step 4b can be performed before step 3 (i.e. UE-2 can initiate the PDU session establishment or modification procedure before sending the Path Switch Response message to UE-1). If the concept in clause 5.16 is applied for UE-1, it is also feasible for UE-1 to initiate the PDU session establishment or modification procedure before sending the Path Switch Request message so that the Path Switch Request message can be set based on the result of the established/modified PDU session(s) on UE-1. Thus, this CR is to clarify that UE-1 can initiate the PDU session establishment or modification procedure before sending the Path Switch Request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.8.2, the statement in NOTE 1 are modified as “Steps 4a and 4b can be executed in parallel, step 4a can be performed before step 2 and step 4b can be performed before step 3”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of path switching from PC5 to Uu, UE-1 can only initiate PDU session establishment/modification procedure after receiving the Path Switch Response message from UE-2. To this end, UE-2 may start traffic transfor over Uu path before the PDU session(s) on UE-1 is ready, and thus leading to risk of data missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 6.8.2	Procedure for communication path switching from PC5 reference point to Uu reference point

Figure 6.8.2-1 depicts the procedure for communication path switching from PC5 reference point to Uu reference point.

Figure 6.8.2-1: Procedure for communication path switching from PC5 reference point to Uu reference point

1.	UE-1 and UE-2 communicate with each other via PC5 path, i.e. by using the established PC5 unicast link between them.

2.	UE-1 decides to path switch ProSe service(s) from PC5 path to Uu path, e.g. if the PC5 signal strength of the unicast link with UE-2 reaches or below certain configured signal strength threshold.

UE-1 determines whether and which ProSe service(s) can be switched to Uu path based on e.g. the availability of the Uu path, the path selection policy, etc.

UE-1 sends a Path Switch Request message to UE-2 to negotiate the ProSe service(s) to be switched. UE-1 includes the ProSe service(s) that can be switched to Uu path in the Path Switch Request message.

Optionally, UE-1 can decide the Uu QoS parameters of each UE based on PC5 QoS parameters for each ProSe service. UE-1 may include Uu QoS parameters used for UE-2's Uu path for the ProSe service(s) that can be switched to a Uu path in the Path Switch Request message.

3.	UE-2 responds with a Path Switch Response message.

UE-2 determines whether and which ProSe service(s) can be switched to Uu path based on e.g., the path selection policy, availability of Uu path, etc.

If UE-2 accepts the path switch request from UE-1, UE-2 includes the accepted ProSe service(s) to be switched to Uu path in the Path Switch Response message from those indicated by UE-1. Otherwise, UE-2 rejects the path switch request with the appropriate cause (e.g. path switching to Uu path for the ProSe service(s) is not permitted, Uu path is not available) in the Path Switch Response message.

4a.	UE-1 may establish or modify PDU Session by using the PDU session establishment procedure as specified in clause 4.3.2 or the PDU session modification procedure as specified in clause 4.3.3 of TS 23.502 [5]. UE-1 may set the Requested QoS as the Uu QoS parameters used for UE-1's Uu path for the specific accepted ProSe service in the PDU Session Modification Request. The PDU Session needs to support QoS requirements for the ProSe service(s) to be switched from PC5 path.

4b.	UE-2 may establish or modify PDU Session by using the PDU session establishment procedure as specified in clause 4.3.2 or the PDU session modification procedure as specified in clause 4.3.3 of TS 23.502 [5]. UE-2 may set the Requested QoS as the Uu QoS parameters used for UE-2's Uu path for the specific accepted ProSe service in the PDU Session Modification Request. The PDU Session needs to support QoS requirements for the ProSe service(s) to be switched from PC5 path.

NOTE 1:	Steps 4a and 4b can be executed in parallel, step 4a can be performed before step 2 and step 4b can be performed before step 3.

5.	Traffic for the accepted ProSe service(s) is switched from PC5 path to Uu path.

6.	The PC5 unicast link may be released, e.g. if no more ProSe services over the PC5 unicast link.

NOTE 2:	The UEs can agree to maintain the PC5 unicast link after path switching from PC5 path to Uu path. In this case, the UEs can determine to switch back to PC5 path from Uu path based on e.g. the path selection policy, the PC5 signal level of the maintained unicast link (e.g. the measured link quality is good).

NOTE 3:	When the UEs cannot successfully exchange Path Switch Request/Response due to, e.g. the PC5 unicast link suddenly breaks, whether to perform path switching to Uu path is left to UE implementation.

END OF CHANGES