SA WG2 Meeting #159	S2-2310460

# October 9th – 13th, 2023; Xiamen, CN			(revision of S2-)

Source: 	CATT, Thales

Title: 	FS_5GSAT_ARCH_Ph3_Key issue for WT#2

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a KI for WT#2 of FS_5GSAT_ARCH_Ph3.

1	Discussion

The Work Task#2 of the FS_5GSAT_ARCH_Ph3 SID is:

“WT2: Store and Forward

WT-2.1: Study, and if applicable, define the parameters needed to characterize and support S&F Satellite operation from a data service perspective, both for NR NTN (5GS) and IOT NTN (EPS).

WT-2.2: Study, and if applicable, define the control plane and user plane enhancements, including the minimum necessary set of Core Network functions to be embedded in the satellite, to support S&F Satellite data operation, both for NR NTN (5GS) and IoT-NTN (EPS). Co-ordinate with SA3 LI if needed.

NOTE 2:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

”

Based on the WT, it is clear that the architecture to support S&F for delay tolerant services (e.g., SMS, small data for CIoT and some UDP based data services) needs to be studied first.

Besides the architecture, we also need to study following issues based on WT description:

-	Authentication of activating S&F operation for a UE;

-	Parameters to characterize and support S&F operation on satellite from a data service perspective.

Due to the movement of NGSO satellites, the satellite providing S&F service to a UE may change dynamically. In this case, the UL/DL user data would be stored in one or more satellite, so we proposed to study:

-	S&F operation on satellites, including:

-	How to store UL/DL data into one or more satellites;

-	How to forward UL/DL data stored in one or more satellites.

2 Proposal

It is proposed to update TR 23.700-29 on FS_5GSAT_Ph3 as follows

5	Key Issues

5.X	Key Issue #X:  Support of store and forward

5.X.1	Description

Store and Forward (S&F) operation is an operation mode of a 5G system with satellite-access, where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently or temporarily unavailable, e.g. to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment.

To support S&F operation for delay tolerant services, it is proposed to study the following items:

-	Architectural enhancement for 5GS/EPS, including:

-	Minimum necessary set of core network functions onboard the satellite(s), to support S&F operation;

-	Which network function will be used to store UL/DL data.

-	Negotiation between UE and the network to activate S&F operation;

-	Parameters to characterize and support S&F operation on satellite from a data service perspective following SA1 requirements;

-	S&F operation on satellites, including:

-	How to store UL/DL data into one or more satellites;

-	How to forward UL/DL data stored in one or more satellites.

NOTE:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.