3GPP TSG-WG2 Meeting #159	S2-2310535

# Xiamen, China, October 09 – 13, 2023                                               (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1171 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on interaction between AF and PCF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DetNet |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in current specification, the information required for setting up an AF session with support for DetNet is transported from AF to PCF, as following: 5.3.1 Interactions between PCF and AF Npcf and Naf enable transport of application level session information and Ethernet/IP port management information from AF to PCF. Such information includes, but is not limited to: - information required to enable setting up an AF session with support for Deterministic Networking (DetNet) as defined in clause 6.1.3.23b. However, current procedure is TSCTSF creates an AF session with the PCF based on the information (e.g. user-plane node ID, port number) provided by PCF. For the 5GS DetNet node configuration procedure, the TSCTSF constructs the TSCAC based on the traffic requirement provided by DetNet controller, and then sends the mapped parameters to PCF on AF session. In the whole procedure, the AF is not involved, and per the architecture for Deterministic Networking (specified in clause 4.4.8.4 of TS 23.501), there is not an interface between AF and PCF. Therefore, the description related to information required for setting up an AF session with support for DetNet between PCF and AF is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To remove the description related to information required for setting up an AF session with support for DetNet in clasue 5.3.1 Interactions between PCF and AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

5.3.1	Interactions between PCF and AF

Npcf and Naf enable transport of application level session information and Ethernet/IP port management information from AF to PCF. Such information includes, but is not limited to:

-	IP filter information or Ethernet packet filter information to identify the service data flow for policy control and/or differentiated charging;

-	media/application bandwidth requirements for QoS control;

-	In addition, for sponsored data connectivity:

-	the sponsor's identification;

-	optionally, a usage threshold and whether the PCF reports these events to the AF;

-	information identifying the application service provider and application (e.g. SDFs, application identifier, etc.);

-	information required to enable Application Function influence on traffic routing as defined in clause 5.6.7 of TS 23.501 [2];

-	information required to enable Application Function influence on Service Function Chaining as defined in clause 5.6.16 of TS 23.501 [2];

-	information required to enable setting up an AF session with required QoS as defined in clause 6.1.3.22;

-	information required to enable setting up an AF session with support for Time Sensitive Networking (TSN) as defined in clause 6.1.3.23.

-	information required to enable setting up an AF session with support for Time Sensitive Communication as defined in clause 6.1.3.23a.

-	information required to enable setting up an AF session with support for eXtended Reality and Interactive Media Services (XRM) as defined in clause 6.1.3.27.

Npcf also enables the AF to request to influence Access and Mobility related policies for a UE and enables the AF to provide guidance for UE URSP rule determination. Npcf and Naf enable the AF subscription to notifications on PDU Session events, i.e. the events requested by the AF as described in clause 6.1.3.18 and the change of DNAI as defined in clause 5.6.7 of TS 23.501 [2].

The N5 reference point is defined for the interactions between PCF and AF in the reference point representation.

>>>> End of Changes <<<<