3GPP TSG-SA2 Meeting #159	S2-2310582

# Xiamen, China,  09 - 13 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4981 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Congestion handling when a UE accesses an SNPN for Localized Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When a UE accesses an SNPN for Localized Services, if the SNPN is under congestion control condition, based on the existing mechanism in clause 5.19.7, the AMF of the SNPN can select a Mobility Management back-off time value to the UE to defer the UE’s request in this SNPN. However, the Mobility Management back-off timer may exceed the duration/time period of the time validity that allows the UE for accessing Localized Services in that SNPN. Therefore, the UE may stay in that SNPN but cannot access any services until the expiration of the Mobility Management timer. Based on the above issue, the CR proposes to solve it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes to resolve the above issue: - When NAS level congestion control is activated at AMF as specified in clause 5.19.7.2, to prevent a UE staying in an SNPN for Localized Services but not able to get services from the SNPN due to congestion, the AMF of the SNPN can select the Mobility Management back-off time value so that the UE can still access Localized Services in the SNPN when the Mobililty Management back-off timer expires or the AMF of the SNPN can reject the UE with proper cause to prevent the UE from automatically selecting and accessing the SNPN again |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot access any Localized Services in an SNPN for Localized Services sue to congestion control |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

5.30.2.5	Network access control

If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a self-assigned NID and there is no subscription for the UE, then the AMF shall reject the UE with an appropriate cause code to temporarily prevent the UE from automatically selecting and registering with the same SNPN.

If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a coordinated assigned NID and there is no subscription for the UE, then the AMF shall reject the UE with an appropriate cause code to permanently prevent the UE from automatically selecting and registering with the same SNPN.

NOTE:	The details of rejection and cause codes is defined in TS 24.501 [47].

If a UE performs the registration in an SNPN using credentials from a Credentials Holder (i.e. the CH is the PLMN/SNPN that owns the UE subscription and controls the access) and the Credentials Holder does not authorize the UE to access that specific SNPN due to access authorization based on subscription data or invalid time for accessing an SNPN that provides access to localized services, then the UDM, in the Credentials Holder, can reject the UE which results in AMF rejecting the registration request from the UE with an appropriate cause code to prevent the UE from automatically selecting and registering with the same SNPN using credentials from the Credentials Holder as described in TS 24.501 [47].

In order to prevent access to SNPNs for authorized UE(s) in the case of network congestion/overload, Unified Access Control information is configured per SNPN (i.e. as part of the subscription information that the UE has for a given SNPN) and provided to the UE as described in TS 24.501 [47].

When the NAS level congestion control is activated at AMF as specified in clause 5.19.7.2, to prevent a UE staying in an SNPN for accessing for Localized Services but not able to get services from the SNPN due to the congestion, the AMF can select the Mobility Management back-off time value so that the UE can still access Localized Services in the SNPN when the Mobility Management back-off time is expired or the AMF can reject the UE with a proper cause to prevent the UE from automatically selecting and accessing the SNPN again.

**** END OF CHANGES ****