SA WG2 Meeting #S2-159	S2-2310773

09 - 13 October, 2023, Xiamen, P.R. China

# Title:	Avoiding Packet loss due to deactivated MBS session

# Source:	Nokia; Nokia Shanghai-Bell

# Document for:	Discussion

Agenda Item:	8.9

Abstract:	This contributions provides explanations for the CR in S2-2310774

Introduction:

At SA2#157, SA2 has treated LS S2-2306284 of SA6. SA6 raised concerns related to possible packet loss when an MBS session is deactivated by the network after a silence period and reactivated when the first packets arrive. The LS indicated Rel-18 as target release as SA6 is performing work related to public safety usage of MBS in that release.

SA2 then reacted with LS S2-2307982 with follow-up questions towards SA6 and RAN2, but postponed related CRs and a resolution of the original issue raised by SA6.

SA2 now obtained replies from those groups:

- RAN2 informs in S2-2310119 that RAN may move UEs to RRC-Inactive state when temporary receiving no MBS multicast data and that a subsequent transitioning to RRC-Connected state (as required in Rel-17) may require about 1 sec; however, in Rel-18 a UE may be able to receive downlink Rel-18 multicast data in RRC_INACTIVE state.

- SA6 informs in S2-2310142 about mission critical service KPIs and requirements in 3GPP TS 22.179:

  - [R-6.15.3.2-012] For group calls where no acknowledgement is requested from affiliated MCPTT group members, the MCPTT Service shall provide an MCPTT Access time (KPI 1) less than 300 ms for 95% of all MCPTT Request.

  - [R-6.15.3.2-013] For MCPTT Emergency Group Calls and Imminent Peril Calls the MCPTT Service shall provide an MCPTT Access time (KPI 1) less than 300 ms for 99% of all MCPTT Requests.

  - [R-6.15.3.2-015] The MCPTT Service shall provide a Mouth-to-ear latency (KPI 3) that is less than 300 ms for 95% of all voice bursts.

  - [R-6.15.3.2-016] There shall be no (0 ms) initial lost audio at receiving user.

Discussion

Meeting the SA6 requirements for no packet loss after a speech pause can probably be best accomplished if an MBS session is not deactivated (to prevent that UEs transition to RRC_Idle state). Also, RRC-inactive transmission mode will be required for RRC-Inactive UEs.

The delay requirements are for a percentage of MCPTT requests. How well those requirements can be met strongley depends on the characteristics of the MCPTT session (frequence and duration of speech pauses).

It thus seems desirable that the AF can control whether an MBS session can be deactivated by the network.

What is the best way of an AF to inform the 5GC that an MBS session shall not be deactivated?

A 5QI contains information related to delay and packet loss requirements during a data transmission in general, but no specific information about requirements for the start of data transmission. It does not express at all how long expected transmission pauses can be.

Observations:

- A 5QI contains delay and packet loss requirements for an ongoing data transmission but no information about expected duration of pauses

A dedicated flag thus seems preferable.

Annex: Status in TS 23.247 and related observations:

TS 23.247 describes the active and inactive states of an MBS multicast session as follows:

## 4.3	Multicast session state model

…

-	Active state: Multicast MBS session is established and MBS data can be transmitted to the UEs that have joined th Multicast MBS session. Radio resources for the Multicast MBS session are established. To receive multicast MBS session data, UEs that joined the Multicast MBS session shall be in CM-CONNECTED state for receiving data of the Multicast MBS session. UEs are allowed to join the Multicast MBS session (subject to authorization check). 5GC resources and radio resources for the Multicast MBS session are reserved for UEs that joined the Multicast MBS session.

-	Inactive state: Multicast MBS session is established but no MBS data is transmitted to the UEs that have joined the Multicast MBS session. Radio resources for the Multicast MBS session are released, and the UEs that joined the Multicast MBS session may be in CM-CONNECTED or CM-IDLE state. UEs are allowed to join the Multicast MBS session (subject to authorization check).

Observations:

- The Inactive state of the MBS sessions enables resource savings and UE power savings by allowing UEs to transition to CM-IDLE state. However, a subsequent activation that involves paging of idle UEs is required.

- The Inactive MBS session state is suited well for longer data transmission interruptions.

- UEs can be in RRC inactive state even while ab MBS session is in active state (also in Rel-17), e.g. when there are pauses in the data transmission, depending on RAN implementation and policies.

- RAN paging after transmission pauses can occur irrespective of the MBS session state. However, RRC-Inactive UEs are assumed to be reachable faster than RRC-Idle UEs.

TS 23.247 describes the MBS multicast session activation as follows

#### 7.2.5.2	MBS session activation procedure

The following can trigger the MBS session activation procedure:

-	AF requests MB-SMF to activate the MBS session;

-	MB-UPF receives the multicast data and notifies MB-SMF.

Figure 7.2.5.2-1: MBS session activation procedure

In this procedure, steps 11 to 15 are executed if the MB-SMF finds out there are shared tunnel established. Steps 11 to 15, if needed, are executed in parallel with steps 2 to 10.

1.	The procedure may be triggered by the following events:

-	When the MB-UPF receives downlink data for a multicast MBS session, based on the instruction from the MB-SMF (as described in clause 7.2.5.3), the MB-UPF sends N4mb Notification (N4 Session ID) to the MB-SMF for indicating the arrival of DL MBS data.

-	The AF sends MBS Activation request (TMGI) to the MB-SMF directly or via NEF.

2.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session state = Active) to SMF(s).

The SMF sets the related multicast MBS session state to Active and finds out the list of UEs that joined the multicast MBS session identified by the related TMGI. If the SMF determines the user plane of the associated PDU session(s) of the UE(s) with respect to the TMGI are activated already, steps 3-8a will be skipped for those UE(s), i.e. executed from step 8b.

3.	The SMF invokes Namf_MT_EnableGroupReachability Request (List of UEs, [PDU Session ID of the associated PDU Sessions], TMGI, [UE reachability Notification Address], [most demanding ARP, 5QI of all MBS QoS Flow within MBS session])) to AMF(s). When later UE is reachable, the UE reachability Notification Address is used by the AMF to identify and notify the related SMF.

After receiving the request, for each UE in the list, the AMF determines CM state of the UE: see steps 4 - 7.

4a.	If there are UEs involved in the multicast MBS Session and in CM-CONNECTED state, the AMF indicates those UEs to the SMF, using Namf_MT_EnableGroupReachability Response (UE list). Otherwise, the response does not include UE list.

4b.	For each UE in the UE list included in step 4a, if the QoS profile(s) for associated PDU Session has not yet been provided, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information (PDU Session ID, MBS Session ID, [QoS profile(s) for associated QoS flow(s)], [mapping information between the unicast QoS flow and multicast QoS flow])) to the AMF for the UE which is identified in step 4a. The associated unicast QoS Flow(s) as well as the mapping information between the unicast QoS Flow(s) and multicast QoS Flow(s) are included to support the 5GC Individual MBS traffic delivery.

The procedure continues at step 9.

5.	[Conditional] If AMF determines that there are UEs in CM-IDLE state and involved in the multicast MBS Session, the AMF figures out the paging area covering all the registration areas of those UE(s), which need to be paged. The AMF may apply paging differentiation as specified in clause 6.12. The AMF sends a Multicast Group paging request message to the NG-RAN node(s) belonging to this Multicast Paging Area with the involved UE list and TMGI as the identifier to be paged if the related NG-RAN node(s) support MBS. If the AMF knows that the NG-RAN node(s) do not support MBS based on configuration, the AMF sends Paging message(s) to the NG-RAN node(s) per UE as described in step 4b in clause 4.2.3.3 of TS 23.502 [6].

NOTE 1:	In addition to the paging in clause 6.12, other paging strategies is up to AMF implementation.

NOTE 2:	The details of the paging are specified by the RAN WGs.

6.	Receiving the paging, the UE(s) in CM-IDLE state sends Service Request message to the AMF, see clause 4.2.3 of TS 23.502 [6].

NOTE 3:	Step 6 for a UE can be parallel to step 5 for another UE(s), which has not received any paging yet.

7a/7b.	After receiving the Service Request sent by the UE(s),

-	Either based on the received PDU Session ID in step 3, the AMF identifies the related SMF and invokes Nsmf_PDUSession_UpdateSMContext request. The procedure continues at step 9; or:

-	Based on the received UE reachability Notification Address in step 3, the AMF identifies and notifies the related SMF of the UE(s), which are reachable now and the Location Information, by using the Namf_MT_UEReachabilityInfoNotify message. In this case, it can be a separated notification or combined with step 8.

8a.	For UE(s) that do not respond to paging, the AMF informs the SMF of the paging failure in Namf_MT_UEReachabilityInfoNotify.

8b.	For UE(s) that is indicated as reachable via the Namf_MT_UEReachabilityInfoNotify message, or user plane of the associated PDU session is activated already but the QoS profile(s) for associated QoS flow(s) needs to be provided for the PDU session, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information ()) to the AMF same as described in step 4b.

9.	The AMF sends N2 request message (N2 SM information ()) to the RAN node.

NOTE 4:	A joined UE is not able to receive MBS data if NG-RAN rejects the PDU Session Resource setup request (due to implementation specific reasons, e.g. activation of user plane fails due to the number of UEs reaching a limit).

10a.	If the shared tunnel has not been established before, the shared tunnel is established at this step, as defined in clause 7.2.1.4. The NG-RAN configures UE with RRC messages if needed.

10b.	Steps 8 to 12 defined in clause 7.2.1.3 are performed. If 5GC Individual MBS traffic delivery is used, the SMF configures the UPF for individual delivery and if necessary, requests the MB-SMF to configure the MB-UPF to send multicast data to the UPF.

11.	If the MB-SMF finds out there are shared tunnel established, step11-15 are performed. The MB-SMF invokes Namf_MBSCommunication_N2MessageTransfer Request (TMGI, N2 SM Information (Activation, TMGI)) to the AMF for those NG-RAN nodes, which have shared tunnel with MB-UPF. This step may be performed in parallel with step 2.

NOTE 5:	The messages in steps 10a, 11 and 12 are MBS-specific and it is possible that the AMF(s) in steps 10a, 11 and 12 are not associate to any UEs involved in the multicast MBS Session.

12.	The AMF sends NGAP activation request message (N2 SM Information ()) to the NG-RAN nodes. For those UEs that have joined in the MBS Session and are in RRC_INACTIVE state, the RAN nodes perform RAN paging as specified in TS 38.300 [9].

13.	The NG-RAN nodes responses to AMF by NGAP activation response message. The NG-RAN nodes establish radio resources to transmit multicast MBS session data to the UE(s). The NG-RAN shall not release the radio connection of a UE that has joined into the Multicast MBS session only because no unicast traffic is received for the UE.

14.	AMF to MB-SMF: Namf_MBSCommunication_N2MessageTransfer Response ().

15.	The MB-SMF sends N4mb Session Modification Request to the MB-UPF to forward the received MBS packets. The MB-UPF responds to the MB-SMF with N4mb Session Modification Response acknowledging the MB-SMF request.

NOTE 6:	Depending on implementation, step 15 can be executed after the first successful response in step 14 to shorten the activation time, or buffering at the MB-UPF can be applied sufficiently long for the majority of RAN nodes to activate the MBS session to reduce packet loss.

Observations:

- While an MBS session is deactivated, buffering at the MB-UPF for incoming packets is applied to reduce packet loss during MBS session activation

- The MB-SMF does not know when UEs in the multicast session are again reachable:

  - The MB-SMF is not aware of when UEs react to paging.

  - When the MB-SMF contacts RAN nodes for activation, the RAN nodes do not need to wait until radio resources for multicast transmission are established before replying to the MB-SMF

  - New RAN nodes that the MB-SMF is not yet aware of may be added to the MBS multicast session during the activation procedure.

- It is up to MB-SMF implementation when to terminate a buffering during the activation.

  - There is a trade-off between delay in the data transmission and reduced risk of packet loss.

- A deactivation of the MBS session comes with a risk of packet loss and/or delayed packet transmission during the activation even in buffering is used.

- There is a trade-off between advantages and disadvantages of MBS session deactivation that may depend on the sensitivity of and application on delay and packet loss, and on the expected length of transmission pauses. The AF may have the best knowledge about the related characteristics of an application

- Allowing the AF to prevent a deactivation of a MBS session by the 5GC allows to prevent both delay and the risk of packet loss during that comes with an MBS session activation.