3GPP TSG-SA2 Meeting #159	S2-2310781

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0335 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 3 Allignment regarding MBS Group Msg Delivery service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Group Message Correlation ID is assigned by the NEF during the Nnef_MBSGroupMsgDelivery_Create service operation (i.e. is a mandatory output parameter of the service operation), whereas the External Group ID is provided by the AF as mandatory input parameter of the Nnef_MBSGroupMsgDelivery_Create service operation. For the Nnef_MBSGroupMsgDelivery_Update service operation and Nnef_MBSGroupMsgDelivery_Delete service operations, both the Group Message Correlation ID and the External Group Identifier are defined as mandatory input parameters. However, the External Group Identifier is redundant in those service operations and could lead to contradictions with the Group Message Correlation ID. For service based operations it is preferable to use a parameter assigned by the NF service provider (such as the Group Message Correlation ID) as resource identifier. The External Group IdentifierID is also not supported in stage 3 TS 29.522 for the Nnef_MBSGroupMsgDelivery_Update service operation and Nnef_MBSGroupMsgDelivery_Delete service operations. In TS 29.522 Figure 5.29.2.1-1 the following related resource model is defined: The ”delRef” parameter in stage 3 corresponds to the Group Message Correlation ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The External Group IdentifierID is removed as mandatory input parameter from the Nnef_MBSGroupMsgDelivery_Update service operation and Nnef_MBSGroupMsgDelivery_Delete service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Contradiction related to Parameters of the Nnef_MBSGroupMsgDelivery_Update service operation and Nnef_MBSGroupMsgDelivery_Delete service operations between stage 2 and stage 3, and unnecessary redundant parameters in stage 2 that complicate a service-based design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.4.3, 9.4.4.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.4.4.3	Nnef_MBSGroupMsgDelivery_Update service operation

Service operation name: Nnef_MBSGroupMsgDelivery_Update

Description: This service is used by the NF Service Consumer to replace group message delivery.

Inputs, Required: Group Message Correlation ID.

Inputs, Optional: Group Message Payload, MBS service area, Group Message Delivery Start Time, Stop Time.

Outputs, Required: Acceptance Status

Outputs, Optional: Cause, service area without MBS capability.

#### 9.4.4.3a	Nnef_MBSGroupMsgDelivery_Delete service operation

Service operation name: Nnef_MBSGroupMsgDelivery_Delete

Description: This service is used by the NF Service Consumer to recall (or cancel) group message delivery.

Inputs, Required: Group Message Correlation ID.

Inputs, Optional: None.

Outputs, Required: Acceptance Status.

Outputs, Optional: Cause.