3GPP TSG-WG SA2 Meeting #159	S2-2310796

# Xiamen, China, Oct 9-13, 2023		(revision of S2-23)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0060 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to procedures for Ranging/SL Positioning service exposure through PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Since the work scope of NR_pos_enh2 has been updated in RP-232670, the following assumptions will be followed for R18: - Only unicast Ranging/SL Positioning session is supported. - Partial coverage scenario is not supported, i.e. either the UEs are all out of coverage or all in coverage. - When LMF is involved, it is assumed that the same LMF serves all the Ranging/SL Positioning UEs for the eLCS procedures and Ranging/SL Positioning procedures, i.e. the LMF can communicate to each UE directly. For Network-based Operation, inter-PLMN Ranging/SL Positioning is not supported. Hence, it is not necessory for the SL Positioning Client UE to check the status of the UEs. Especially, it should be the Target UE to determine whether to trigger SL-MO-LR procedure or UE-only operation procedure based on its own NAS connection status: - If it has the NAS connection, SL-MO-LR procedure is initiated - Otherwise, UE-only operation procedure is initiated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the procedure to be aligned with RAN2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unalignment to RAN2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.7.1-1.	Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.2 with following updates:

-	When SL positioning service is triggered, the client UE tries to discover the target UE and/or a SL Reference UE. When Model B discovery used, in the Solicitation message, target UE's user info is included and role of target UE and role of SL Reference UE are indicated in the RSPP metadata.

-	When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2. When Model B discovery used, in the Solicitation message, UE1's user info and UE2's user info are included and role of target UE is indicated in the RSPP metadata.

During discovery for SL positioning service, the information of SL Reference UE is acquired.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes a PC5 connection with the discovered UE.

3.	The SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE and info of SL Reference UE for SL positioning, and info of SL Positioning Client UE, info of UE1 and info of UE2 for Ranging.

4.	Authorization check of the client UE for the service request may be performed via SL Positioning Server UE or 5GC or internally by UE1 or target UE.

5.	The UE which received SL Positioning Service Request (here UE1) determines whether to initiate SL-MO-LR procedure as defined in 6.20.1 of TS 23.273 [8] or procedures of Ranging/Sidelink Positioning control defined in clause 6.8 based on its own NAS connection status.

6.	The Ranging/SL Positioning result is provided to the SL Positioning client UE by UE1.

# * * * * End of changes * * * *