3GPP SA WG2 Meeting #159	S2-2310891

09-13 Oct 2023, Xiamen, China                 	(revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0373 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Mobility Restrictions for 5G ProSe UE-to-Network Relaying |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.4.3, for the forbidden area the following description is given: - Forbidden Area: - In a Forbidden Area, a 5G ProSe enabled UE is not allowed to perform the Relay operation. If the 5G ProSe UE-to-Network Relay operates in a Forbidden Area of the 5G ProSe Remote UE, the 5G ProSe Remote UE is not allowed to access the network via this 5G ProSe UE-to-Network Relay. It is proposed to describe the same scenario for the service area restriction: the behavior of the 5G ProSe Layer-2 Remote UE when the 5G ProSe Layer-2 UE-to-Network Relay operates in a Non-Allowed Area of the 5G ProSe Layer-2 Remote UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description regarding the behavior of the 5G ProSe Layer-2 Remote UE when the 5G ProSe Layer-2 UE-to-Network Relay operates in a Non-Allowed Area of the 5G ProSe Layer-2 Remote UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description regarding the behavior of the 5G ProSe Layer-2 Remote UE when the 5G ProSe Layer-2 UE-to-Network Relay operates in a Non-Allowed Area of the 5G ProSe Layer-2 Remote UE is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Mirror CR to S2-2310890 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.4.3	Mobility Restrictions for 5G ProSe UE-to-Network Relaying

The handling of Mobility Restrictions for 5G ProSe enabled UE follows the principles as specified in clause 5.3.4.1 of TS 23.501 [4] with the following additions and clarifications:

-	Forbidden Area:

-	In a Forbidden Area, a 5G ProSe enabled UE is not allowed to perform the Relay operation. If the 5G ProSe UE-to-Network Relay operates in a Forbidden Area of the 5G ProSe Remote UE, the 5G ProSe Remote UE is not allowed to access the network via this 5G ProSe UE-to-Network Relay unless the 5G ProSe Remote UE is attempting for emergency service.

-	Service Area Restriction:

-	Service Area Restriction is not applicable to 5G ProSe Layer-3 Remote UE.

-	Service Area Restriction is not applicable to 5G ProSe enabled MCX-subscribed UEs as defined in clause 5.3.4.1 of TS 23.501 [4].

NOTE 1:	It is expected that all 5G ProSe enabled Public Safety UEs are MCX-subscribed as defined in clause 5.16.6 of TS 23.501 [4].

-	In a Non-Allowed Area, a 5G ProSe enabled UE follows the principles of a UE in limited service state as specified in clause 5.9. It cannot perform the Relay operation as 5G ProSe Layer-2 UE-to-Network Relay or 5G ProSe Layer-3 UE-to-Network Relay based on the conditions described in clause 5.9 except for emergency service from the 5G ProSe Remote UE as specified in clause 5.4.4.

-	In a Non-Allowed Area, a 5G ProSe Layer-2 Remote UE follows the principles as specified in clause 5.3.4.1 of TS 23.501 [4], for communication with the network via the 5G ProSe Layer-2 UE-to-Network Relay.

-	If a 5G ProSe Layer-2 UE-to-Network Relay operates in a Non-Allowed Area of a 5G ProSe Layer-2 Remote UE, the 5G ProSe Layer-2 Remote UE follows the principles as specified in clause 5.3.4.1 of TS 23.501 [4], for communication with the network via the 5G ProSe Layer-2 UE-to-Network Relay.

-	Core Network type restriction, RAT type restriction:

In these cases, a 5G ProSe enabled UE is not able to register in the network for normal service and stays in limited service state and the principles in clause 5.9 shall apply. The 5G ProSe Layer-2 Remote UE is allowed to register to the network via the 5G ProSe UE-to-Network Relay for emergency service.

NOTE 2:	Closed Access Group information is not specified for 5G ProSe.

END OF CHANGES