3GPP TSG-WG SA2 Meeting #159	S2-2310926

# Xiamen, China, Oct 9 – 13, 2023		(revision of S2-2309090)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0336 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on Relay discovery for emergency service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For a 5G ProSe Layer-2 UE-to-Network Relay to advertise its support of emergency service, the serving NG-RAN support of emergency services is required as the Layer-2 Remote UE may select a different PLMN from the Layer-2 Relay. To advertise its support of emergency service means the model A discovery, and the model B needs to follow the same principle. But the corresponding model B part is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The serving NG-RAN support of emergency services is required for L2 Relay, when performing both model A and model B relay discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.9.3, 5.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.3.9.3	5G ProSe Layer-2 UE-to-Network Relay

In addition to the common 5G ProSe UE-to-Network Relay functions defined in clause 4.3.9.1, 5G ProSe Layer-2 UE-to-Network Relay supports the following functions to enable connectivity to the network:

-	5G ProSe Direct Communication via 5G ProSe Layer-2 UE-to-Network Relay as specified in clause 6.5.2, for the communication with the 5G ProSe Layer-2 Remote UEs for the relay operations;

-	QoS handling for 5G ProSe Layer-2 UE-to-Network Relay and end-to-end QoS treatment as defined in clause 5.6.2.3.

-	For a 5G ProSe Layer-2 UE-to-Network Relay to participate in the 5G ProSe UE-to-Network Relay Discovery procedure for emergency service, the serving NG-RAN support of emergency services is required as the Layer-2 Remote UE may select a different PLMN from the Layer-2 Relay.

# * * * * Second change * * * *

#### 5.4.4.2	Emergency service from 5G ProSe Remote UE via via 5G ProSe Layer-2 UE-to-Network Relay

For a 5G ProSe Layer-2 UE-to-Network Relay to participate in the 5G ProSe UE-to-Network Relay Discovery procedure for emergency service (i.e., sending the UE-to-Network Relay Discovery Announcement message or UE-to-Network Relay Discovery Response message) , the serving NG-RAN support of emergency services is required, indicated by it broadcasting ims-EmergencySupport in system information as specified in TS 38.300 [12] and TS 38.331 [16]. The 5G ProSe Layer-2 Remote UE may select a different PLMN from the 5G ProSe Layer-2 UE-to-Network Relay. The 5G ProSe Layer-2 Remote UE may select any PLMN within the RRC Container (see clause 5.8.3.3) obtained from any 5G ProSe Layer-2 UE-to-Network Relay(s) which has announced the RSC dedicated for emergency service during 5G ProSe UE-to-Network Relay Discovery in clause 6.3.2.3.

NOTE:	When emergency support is indicated by RAN, 5G ProSe Layer-2 UE-to-Network Relay can participate in the 5G ProSe UE-to-Network Relay Discovery procedure for emergency service even if its serving PLMN does not support emergency service.

# * * * * End of changes * * * *