3GPP TSG SA2#159	S2-2311054

9 - 13 Oct, 2023, Xiamen, China

# Source:	ZTE

# Title:	TEI-19 on Enhancement of Network Slicing Phase 4

# Document for:	Information

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	TEI-19 on Enhancement of Network Slicing Phase 4

Acronym:	TEI19_eNS_Ph4

Unique identifier:	TBD

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

3	Justification

Network Slicing has been evolved over a number of 3GPP releases and now it has reached maturity from feature richness standpoint. However, there are still some improvements that are needed to ensure existing features are more deployable and make operators task of running a network where network slicing is deployed easier.

NSSAA has been supported in 5GS since Rel-16.After NSSAA procedure the AAA server stores the NSSAA result and may revoke the NSSAA result at any time. It is benefit for AAA server to be notified when the UE is deregistered from the network so it does not revoke the NSSAA result if the UE has been deregistered from the network slice.

The NG-RAN today can provide indication of what network slices are supported in which TA. In Rel-18 the AMF needs to be configured area of network slice availability at cell level. Vendor specific OAM mechanisms seem to be required and this may operationally represent a challenge. It is benefit to define a mechanism by which the NG-RAN can indicate the network slice support per cell level.

Currently the UE-Slice-MBR is statically subscribed in UDM. The network slice consumer may want to upgrade the UE-Slice-MBR for short time to meet new requirements and downgrade the UE-Slice-MBR after the short time. The mechanism to support dynamical Slice MBR provisioning from the network slice consumer needs to be specified.

The network slice mapping information between VPLMN and HPLMN is only provided from NSSF to AMF in the Nnssf_NSSelection_Get service operation. When the mapping information is updated in the NSSF(e.g. the VPLMN network slice is not available), it is not possible to notify the AMF in time about the change. A mechanism to support slice mapping information notification from NSSF needs to be specified.

4	Objective

The following aspects will be specified in this WID:

- Support the UE deregistration status awareness in the NSSAAF

- Support open mechanisms to indicate information on availability of network slices at cell level to AMF.

- Support dynamical Slice MBR provisioning from the network slice consumer.

- Support slice mapping information notification from NSSF

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or it depends on the completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 |  | 0.25 | No |  |
| WT#2 |  | 0.25 | No |  |
| WT#3 |  | 0.25 | No |  |
| WT#4 |  | 0.25 | No |  |

Total TU estimates for the study phase:

Total TU estimates for the normative phase: 1

Total TU estimates: 1

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Enhancement of system architecture of 5GC | SA#103 (March 2024) | SA#104 (June 2024) |
| TS 23.501 | Enhancement of procedure of 5GC | SA#103 (March 2024) | SA#104 (June 2024) |

6	Work item Rapporteur(s)

name, company, company dot com

7	Work item leadership

SA2

8	Aspects that involve other WGs

RAN impacts covered by RAN WGs (RAN2, RAN3).

Security aspects covered by SA3.

OAM and charging aspects covered by SA5.

9	Supporting Individual Members

Supporting IM name

ZTE