3GPP TSG-WG SA2 Meeting #159 	S2-2311190

Xiamen, China, 9-13 October 2023

# Title:	[Draft] Reply LS on 5G ProSe UE-to-UE relay discovery

# Response to:	LS on 5G ProSe UE-to-UE relay discovery (C1-236448/ S2-2310091)

# Release:	Release 18

# Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR0321, CR0382 to TS 23.304;

1. Overall Description:

SA2 would like to thank CT1 for its LS on 5G ProSe UE-to-UE relay discovery (C1-236448/ S2-2310091).

Regarding the questions from CT1, SA2 would like to provide the following answers:

- According to clause 6.3.2.1 of TS 23.304, during 5G ProSe direct discovery procedures, the announcing UE/ discoverer UE/ discoveree UE "includes the relay_Indication if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Discovery".

However, the user info ID, used in the 5G ProSe UE-to-UE relay discovery, cannot be obtained by the 5G ProSe UE-to-UE relay UE during the 5G ProSe direct discovery. So the 5G ProSe UE-to-UE relay UE cannot identify the same 5G ProSe end UE who sent the relay indication by using this user info ID in the 5G ProSe UE-to-UE relay discovery. This makes the relay indication mechanism cannot work. Then:

Question 1: How can this issue be resolved?

SA2 answer to Q1: SA2 had discussed this matter, and agreed the attached CR0321. The "relay_indication" was removed from the description in clause 6.3.2.1. A 5G ProSe End UE only uses the procedures defined in clause 6.3.2.4 for 5G ProSe UE-to-UE Relay discovery.

- According to clause 6.3.2.4.4 of TS 23.304, for candidate 5G ProSe UE-to-UE Relay Discovery to support the negotiated Relay reselection, the discoverer End UE discovers a candidate 5G ProSe UE-to-UE Relay without sending the RSC and the User Info ID of the discoveree 5G ProSe End UE. Then:

Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly?

Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message?

SA2 answer to Q2 and Q3: SA2 had discussed this issue and agreed the attached CR0382.

The procedure defined in clasue 6.3.2.4.4 was meant to be used by the "responding 5G ProSe End UE", as in clause 6.7.4.2 step 4 or clause 6.7.4.3 step 7, and should terminate at the candidate relay UE.

Therefore, for Question 2, based on the attached CR0382, the relay UE determines to skip the step 2 and 3 in clasue 6.3.2.4.4, based on the absence of discoveree 5G ProSe End UE User Info ID, and the matching of its own User Info ID with the candidate 5G ProSe UE-to-UE Relay User Info ID in the received message.

For Question 3, the updated procedure has the RSC included in the solicitation message.

- According to clause 5.8.4.2, 6.4.3.7.1 and 6.4.3.7.4 of TS 23.304, the user info of the target 5G ProSe end UE (i.e. discoveree end UE) is mandatory in 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) and Direct Communication Request message for 5G ProSe UE-to-UE Relay Communication; while it is optional in the Direct Communication Request message for 5G ProSe UE-to-UE relay communication with integrated discovery. Then:

Question 4: Why are different approaches applied to the user info of the target 5G ProSe end UE in the above procedures?

SA2 answer to Q4: The procedure in clasue 6.4.3.7.4 was meant for the use with Integrated Discovery, i.e. there was no prior standalone 5G ProSe UE-to-UE Relay discovery. Therefore, it was model after the procedure in clause 6.4.3.1, with both the UE oriented option and ProSe Service oriented option supported. That was the reason for an optional User Info for the target 5G ProSe End UE.

On the other hand, the procedure in 5.8.4.2 and 6.4.3.7.1 were for the operation with a separate 5G ProSe UE-to-UE Relay discovery before the link establishment. In such case, the target 5G ProSe End UE is always known. Therefore, the User Info would be mandatory for these procedures.

2. Actions:

To CT1:

ACTION: 	SA2 would like to ask CT1 to take the above into account and develop the corresponding stage 3 specifications.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160	13-17 November 2023	Chicago, USA

TSG-SA2 Meeting #Ad Hoc	21-26 January 2024 	Electronic