3GPP TSG-SA2 Meeting #159	S2-2311193

# Xiamen, P.R. China, 09th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0383 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU session handling with SSC mode 3 for UE to NW Relay connection. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDU session supporting 5G ProSe U2N Relay will be established with SSC mode. But, there is no clear description how the session continuity per SSC mode is satisfied at the Remote UE with a 5G ProSe Layer-3 ProSe UE-to-NW Relay connection case. For SSC mode 3, new PDU session is established before old PDU session is released and new IP address is assigned per IP anchor change. For IPv6 case, new IP prefix shall be assigned to the remote UE also. But, there is no description how it can be achieved. CT1 addressed this issue (C1-233309) but conclusion was deferred for lack of stage2 level decision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For PDU session supporting 5G ProSe UE-to-NW Relay, it is clarified that 5G ProSe Layer-3 UE-to-NW Relay informs the Remote UE that new IP prefix is available when PDU session anchor changes for IPv6 type PDU session with SSC mode 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Session continuity is not supported for 5G ProSe Layer-3 ProSe UE-to-NW relay connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.5	IP address allocation

### 5.5.1	General

#### 5.5.1.1	IP address allocation for unicast mode of 5G ProSe direct communication

For unicast mode of 5G ProSe direct communication, the following mechanism for IP address/prefix allocation may be used:

a)	DHCP-based IPv4 address allocation with one of the two UEs acting as a DHCP server.

b)	IPv6 Stateless Address auto configuration specified in RFC 4862 [17] for assignment of IPv6 prefix, with one of the two UEs acting as IPv6 default router.

NOTE:	Which UE acts as a DHCPv4 server or IPv6 default router is negotiated during secure layer-2 link establishment by exchanging the IP Address Configuration as described in clause 6.4.3.

c)	IPv6 link-local addresses as defined in RFC 4862 [17] are formed by UEs locally. The IPv6 link-local addresses are exchanged during the establishment of a secure layer-2 link over PC5. The UEs shall disable duplicate address detection after the layer-2 link is established.

#### 5.5.1.2	IP address allocation for broadcast and groupcast modes of 5G ProSe direct communication

For broadcast and groupcast modes of 5G ProSe direct communication, the following source IP address management applies:

a)	the UE configures a link local IPv4 address to be used as the source IP address, as defined in clause 4.5.3 of TS 23.303 [3]. If it is not configured with an address, it uses Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18].

b)	the UE configures a link local IPv6 address to be used as the source IP address, as defined in clause 4.5.3 of TS 23.303 [3]. The UE may use this IP address for direct communication without sending Neighbour Solicitation and Neighbour Advertisement message for Duplicate Address Detection.

NOTE:	The destination IP address management for broadcast and groupcast modes of ProSe direct communication is left to UE implementation.

#### 5.5.1.3	IP address allocation for communication with a 5G ProSe Layer-3 ProSe UE-to-Network Relay

For communication with a 5G ProSe Layer-3 UE-to-Network Relay, the following mechanism for IP address/prefix allocation applies:

-	The PDU Session Type used for the relay traffic shall support the IP version used by the 5G ProSe Layer-3 Remote UE. If the 5G ProSe Layer-3 Remote UE initiates an allocation of IPv4 address or an IPv6 prefix when the requested IP version is not supported in the corresponding PDU Session then IP address/prefix allocation fails.

a)	When the 5G ProSe Layer-3 Remote UE uses IPv4 to access the external DN:

a1)	The IPv4 address allocation and IPv4 parameter configuration via DHCPv4 are performed according to RFC 2131 [24] and RFC 4039 [25] procedures. The IPv4 address provided to the 5G ProSe Layer-3 Remote UE from the 5G ProSe Layer-3 UE-to-Network Relay by DHCPv4 procedure shall correspond to a local IPv4 address range configured in the 5G ProSe Layer-3 UE-to-Network Relay.

a2)	The DHCPv4 request from the 5G ProSe Layer-3 Remote UE is always sent subsequent to the establishment of the one-to-one 5G ProSe Direct Communication between the 5G ProSe Layer-3 Remote UE and the 5G ProSe Layer-3 UE-to-Network Relay, see details for the IPv4 address allocation in clause 5.4.4.3 of TS 23.303 [3] with the following difference:

-	The ProSe Relay UE ID of the ProSe UE-to-Network Relay is replaced by the source Layer-2 ID of the 5G ProSe UE-to-Network Relay for PC5 unicast communication.

b)	When the 5G ProSe Layer-3 Remote UE uses IPv6 to access the external DN:

b1)	IPv6 network prefix allocation via IPv6 Stateless Address auto-configuration. Router solicitation from the 5G ProSe Layer-3 Remote UE is always sent subsequent to the establishment of the one-to-one ProSe Direct Communication between the 5G ProSe Layer-3 Remote UE and the 5G ProSe Layer-3 UE-to-Network Relay, see details for IPv6 prefix allocation in clause 5.4.4.2 of TS 23.303 [3] with the following differences:

-	The 5G ProSe Layer-3 UE-to-Network Relay shall obtain the IPv6 prefix assigned to the 5G ProSe Layer-3 Remote UE via prefix delegation function from the network as defined in clause 5.5.2.

-	The ProSe Relay UE ID of the ProSe UE-to-Network Relay is replaced by the source Layer-2 ID of the 5G ProSe UE-to-Network Relay for PC5 unicast communication.

-	PDN connection is replaced by PDU Session.

b2)	IPv6 parameter configuration via Stateless DHCPv6: The UE may use stateless DHCPv6 for additional parameter configuration.

b3)	The 5G ProSe Layer-3 UE-to-Network Relay assigns IPv6 prefixes from IPv6 prefix range that have been assigned to the PDU Session used for the relay traffic via IPv6 prefix delegation.

-	When the 5G ProSe Layer-3 Remote UE uses IPv6 to access the external DN and PDU session supporting the 5G ProSe Layer-3 UE-to-Network Relay is PDU session of SSC mode 3, the 5G ProSe Layer-3 UE-to-Network Relay shall inform the 5G ProSe Layer-3 Remote UE that new IPv6 prefix for the existing PC5 connection to the 5G ProSe Layer-3 Remote UE is available if PDU session Anchor changes.

#### 5.5.1.4	IP address allocation for communication with a 5G ProSe Layer-3 UE-to-UE Relay

For communication with a 5G ProSe Layer-3 UE-to-UE Relay, the following mechanism for IP address/prefix allocation applies:

-	IP address allocation mechanisms of unicast mode of 5G ProSe direct communication as described in clause 5.5.1.1 can be reused on each hop between a 5G ProSe End UE and the 5G ProSe Layer-3 UE-to-UE Relay.

-	The 5G ProSe Layer-3 UE-to-UE Relay may provide the IP address of the target 5G ProSe End UE to the source 5G ProSe End UE in the Direct Communication Accept message, if the target 5G ProSe End UE IP address is available at the time.

-	The 5G ProSe End UE may obtain the IP address of other 5G ProSe End UEs via the 5G ProSe Layer-3 UE-to-UE Relay using DNS query.

NOTE:	5G ProSe Layer-3 UE-to-UE Relay may support 5G ProSe End UEs of different IP versions by performing IP conversions.

### 5.5.2	IPv6 Prefix Delegation via DHCPv6 for 5G ProSe Layer-3 UE-to-Network Relay

IPv6 Prefix Delegation via DHCPv6 for 5G ProSe Layer-3 UE-to-Network Relay is defined in clause 5.8.2.2.4 of TS 23.501 [4].

# * * * * End of change * * * *