3GPP TSG-SA WG2 Meeting #159	S2-2311199

# Xiamen, P.R. China,  09th Oct 2023 – 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0069 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | v | Radio Access Network |  | Core Network | v |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Network based SL positioning for UE without NAS connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For NW based SL positioning for UE without NAS connection, each Located UE may report the result and the routing identifier from Target UE to the LMF via the serving AMF of the Located UE. The routing identifier shared from Target UE is used to indicate an LMF that manages the Target UE positioning and . According to subclause 6.5.2, when the procedure is triggered based on an event configured by the NW while target UE is in coverage, the received deferred routing identifier from the NW may be used for this purpose but when the procedure is triggered for obtain its own location information, it is not clear how the target UE can acquire the routing identifier to indicate serving LMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For each UE, a provisioned Routing Identifier is assigned by AMF during its registration procedure. When NW based SL positioning for UE without NAS connection is triggered for its own location information, target UE share the provisioned routing identifier to located UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NW based SL positioning for UE without NAS connection may not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3, 6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | v | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | v | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | v | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.5.3	Network based SL positioning for UE without NAS connection

When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage and at least one of the discovered Located UEs can establish NAS connection for 5GC-MO-LR, the following principles are applied:

NOTE 1:	5GC-MT-LR is not applicable in this release of the specification.

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE and Located UE(s) perform Ranging/SL positioning.

-	The Target UE selects at least one of the Located UEs that can use SL-MO-LR service for SL Positioning.

-	The Target UE may transmit its ranging measurement data/results to the Located UE(s).

-	A provisioned routing identifier for Target UE can be assigned by AMF during registration. The provisioned routing identifier is used to route the messages from each located UE for the target UE to a LMF via the serving AMF of the target UE.

-	Each Located UE that can establish the NAS connection may report the Ranging/SL positioning measurement data or result and the routing identifier from Target UE to the LMF via the serving AMF of the Located UE. This may include ranging measurement data/results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

NOTE 2:	The routing identifier (i.e., a provisioned routing identifier or a deferred routing identifier) from target UE is used to indicate an LMF that manages the Target UE positioning, and also used to assist the LMF in identifying the Target UE positioning, as defined in clause 6.3.1 of TS 23.273 [8].

-	The LMF may use the received information (i.e. Ranging/SL positioning measurement or estimation result with the same routing identifier) and the location of Located UE(s) to calculate the location of the Target UE, and provide the resulting location via the Located UE to the Target UE.

-	If a LMF is aware of multiple Located UEs associated with the same Target UE based on the same routing identifier, it may perform the calculation using all information from these Located UEs.

- 	The LMF may use the received information (i.e. Ranging/SL positioning measurement data/result and the same routing identifier) and the location of Located UE(s) to calculate the location of the Target UE, and provide the resulting location via the Located UE that initiates the 5GC-MO-LR procedure to the Target UE. In case the LMF has the Target UE context, the LMF may provide the resulting location via the GMLC to the LCS Client or AF as specified in TS 23.273 [8] clause 6.3. In case the UE context was deleted, then this functionality cannot be performed.

Editor's note:	Security aspects require confirmation from SA WG3.

# * * * * Second change * * * *

### 6.5.2	Procedures for Network based SL positioning for UE without NAS connection

The procedures for Network based SL positioning for UE without NAS connection apply to the following cases:

-	when the Target UE initiates 5GC-MO-LR request to obtain its own absolute location and cannot establish a NAS connection or cannot be served by a Ranging/SL positioning capable LMF

-	when the target UE detects an event configured by the network during an initial 5GC-MT-LR procedure when the Target UE has the NAS connection

The procedures for Network based SL positioning for UE without NAS connection takes the SL-MO-LR procedure in TS 23.273 [8] as baseline with following adaptations based on the functionalities described in clause 5.5.3:

-	UEx (one of the UE2/.../UEn) is the Target UE that cannot set up a NAS connection or that cannot be served by a Ranging/SL positioning capable LMF, and UE1 is Located UE that is in coverage and can be served by a Ranging/SL positioning capable LMF.

NOTE:	UE1 as the Located UE can be multiple.

-	In step2, it is the UEx (Target UE) that performs UE1 (Located UE) discovery.

-	In step 6, when the UEx(Target UE) initiates SL-MO-LR service request to UE1(Located UE), the UEx(Target UE) requests UE1(Located UE) to return its absolute location and may indicate whether the absolute location should be transferred to an LCS client or AF. For location request to transfer its location information to an LCS Client or AF, Target UE ID and the Deferred routing ID may be included. For location request for its own location information, Target UE ID and the provisioned routing ID may be included.

-	It is the UE 1 as the Located UE that performs the SL-MO-LR with the step 8~23 with following adaptions:

-	In Step 8: UE1(Located UE) sends SL-MO-LR request which indicates that the Target UE’s absolute location is required, may indicate that the location calculation assistance by the network is needed, and may indicate that location results need to be transferred to the LCS client or AF for UEx (Target UE). For transfer to the LCS Client or AF, the Target UE ID, i.e. Application Layer ID and the Deferred routing ID may be included along with the SL-MO-LR request. For location request for its own location information, Target UE ID and the provisioned routing ID may be included with the SL-MO-LR request. The SL positioning measurement data may be included in step 8.

-	The LMF determines the network based calculation taking the obtained capability of UE in step 10 and step 11 into account.

-	The LMF requests for the location information from the UE1(Located UE) in step14.

-	The step17 for UE based calculate is not needed.

-	The UE1(Located UE) returns the SL location measurement data to the LMF in step18.

-	When SL positioning measurement data was provided in step 8, step 12~17 may be omitted.

-	The LMF calculates the location result (i.e. the absolute Location of Target UE) based on the SL location measurement data and the location of the UE1(Located UE) in step19, and provides the location of UEx(Target UE) to the UE1 (Located UE) in step 22.

-	In case the LMF has the valuable UEx(Target UE) context based on the Deferred routing ID, then the LMF provides the resulting location of the UEx(Target UE) via the GMLC to the LCS Client or AF in step 22. In case the UEx(Target UE) context was deleted, transferring location of the UEx(Target UE) to the LCS Client or AF is not proceeded.

-	The UE1 (Located UE) returns the absolute location result (i.e. the absolute Location of Target UE) to the UEx(Target UE) in step23 over PC5.

# * * * * End of change * * * *