SA WG2 Meeting #159	S2-2311211

# 9-13 October 2023, Xiamen, China	(was S2-2309319)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4896 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IAB authorization operation description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR, IABARC |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN3 had provided their agreements on the RAN node behavior when receiving the IAB Authorized IE from the core netowrk in LS S2-2306278/R3-231010. In SA2#157, several CRs were agreed to align the description regarding the AMF behaivor regarding the authorization process for the IAB-UE. However, it was also agreed that additional aligment with the authorization process of MBSR (described in clause 5.35A.4) was desired when the Rel-18 contents for VMR was finalized. Therefore, this CR provides the corresponding updates to the IAB authorization process, especially on the signalling from the AMF towards the IAB-UE, according to the agreements for MBSR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description for AMF siganlling for the authorization of IAB-UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete AMF behavior description for the IAB-UE authorization description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 5.35.2	5G System enhancements to support IAB

In IAB operation, the IAB-UE interacts with the 5GC using procedures defined for UE. The IAB-node gNB-DU only interacts with the IAB-donor-CU and follows the CU/DU design defined in TS 38.401 [42].

For the IAB-UE operation, the existing UE authentication methods as defined in TS 33.501 [29] applies. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

The following aspects are enhanced to support the IAB operation:

-	the Registration procedure as defined in clause 4.2.2.2 of TS 23.502 [3] is enhanced to indicate IAB-node's capability to the AMF;

-	The IAB-node provides an IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB and includes the IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform IAB authorization.

-	the UE Subscription data as defined in clause 5.2.3 of TS 23.502 [3] is enhanced to include the authorization information for the IAB operation;

-	Authorization procedure during the UE Registration procedure is enhanced to perform verification of IAB subscription information;

-	If the IAB operation is not authorized, the AMF can indicate to the the IAB-UE's that it is not allowed to act as an IAB-node, as part of the registration procedure. The AMF provides the indication either in a Registration Accept or in a Registration Reject message, depending on the PLMN’s choice. When Registration Accept is used, the AMF may also initiate UE Context setup/modification procedure and provide IAB authorized indication with the value set to "not authorized" to the NG-RAN, but the IAB-UE is still registered.

-	If the IAB operation is authorized, UE Context setup/modification procedure is enhanced to provide IAB authorized indication with the value set to "authorized" to NG-RAN.

-	When the IAB authorization state changes for a registered IAB-node (either "authorized", or "not-authorized"), the AMF updates the IAB-UE and the NG-RAN accordingly. Based on the operator configuration, the AMF may use either Deregistration (including the option with re-registration required indication) or the UE Configuration Update procedure to inform the IAB-UE regarding the updated authorization status. The AMF informs the NG-RAN of the new authorization status using UE Context Modification, Initial Context Setup procedure or the DOWNLINK NAS TRANSPORT message.

After registered to the 5G system, the IAB-node remains in CM-CONNECTED state if the IAB operation is authorized. In the case of radio link failure, the IAB-UE uses existing UE procedure to restore the connection with the network. The IAB-UE uses Deregistration Procedure as defined in clause 4.2.2.3 of TS 23.502 [3] to disconnect from the network. In the case of controlled IAB-node release as specified in clause 8.9.10 of TS 38.401 [42] (including the case when authorization state of the IAB-node is changed from authorized to non-authorized), the AMF may trigger the IAB-UE Deregistration based on local configuration (e.g. a timer started after IAB-node is changed from authorized to non-authorized).

# >>>>END OF CHANGES<<<<