SA WG2 Meeting #159	S2-2311248

Xiamen, China, October 09 – 13, 2023	(revision of S2-230xxxx)

Source:	Qualcomm Incorporated

Title:	Key Issue: Objectives of Store and Forward Operation for WT2

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: proposes a new KI for support of Store and Forward Operation

# 1. Introduction

The SID agreed for Study on Integration of satellite components in the 5G architecture in SP-231199 includes the following objectives for WT2:

WT2: Store and Forward WT-2.1: Study, and if applicable, define the parameters needed to characterize and support S&F Satellite operation from a data service perspective, both for NR NTN (5GS) and IOT NTN (EPS). WT-2.2: Study, and if applicable, define the control plane and user plane enhancements, including the minimum necessary set of Core Network functions to be embedded in the satellite, to support S&F Satellite data operation, both for NR NTN (5GS) and IoT-NTN (EPS). Co-ordinate with SA3 LI if needed. NOTE 2: S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

The WT2 objectives refer to aspects of solutions (e.g. parameters and control plane/user plane architecture) which need to be specified as part of these solutions but do not address requirements which these solutions need to support. These requirements exist in SA1 TR 22.865 (eventually in TS 22.261) and address particular use cases and services. Additional requirements are also needed to address service aspects not included in TR 22.865 and TS 22.261 (e.g. regulatory services) and requirements partly out of scope for SA1 such as UE and network impacts and performance. A full set of such requirements is proposed below and includes labels to allow easy reference when evaluating different solutions against the requirements.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-29.

*** Start of the change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[xx]	3GPP TR 22.865: “Study on satellite access Phase 3”.

[yy]	3GPP TS 22.261: "Service requirements for the 5G system".

*** Next change (all new text) ***

## 5.x	Key Issue #x: Support of Store and Forward Satellite Operation

### 5.x.1	General description

Store and forward satellite operation should support the requirements and use cases for store and forward operation in TR 22.865 [xx] and TS 22.261 [yy] and additional objectives related to regulatory services, performance and impact. The TS 22.261 [yy] requirements and use cases provide basic objectives for the KI, and are summarized as follows.

Kx-1	Transfer of MO and MT data from and to a UE using store and forward operation over satellite access (IoT NTN and NR/5GS).

Kx-2	Support of a per UE data retention period.

Kx-3	Support of a per UE storage quota.

Kx-4	Support of integrity protection including authentication and authorization of a served UE.

Kx-5	Support of privacy.

Kx-6	Support of configurable QoS and transfer policies for store and forward operation.

Kx-7	Support of charging of UEs and applications.

Kx-8	Support for informing a UE or user of the use of store and forward operation.

Kx-9	Support for informing a trusted application about use of store and forward operation to send MT data to a UE.

Kx-10	Support for informing UEs about how long MO data will be stored before being delivered.

Additional goals for this KI are as follows.

Ax-1 	Minimize any unavoidable new UE impacts.

Ax-2	Minimize any unavoidable new RAN and CN impacts.

Ax-3	Minimize overall delay in end to end transfer of MO and MT data. (NOTE 1)

Ax-4	Support UE data services applicable to IoT in simplex mode.

Ax-5	Support regulatory service requirements such as PLMN country or regional service restrictions and Lawful Interception. (NOTE 2)

Ax-6	Support roaming and non-roaming UEs

NOTE 1:	Delay for transfer of MO data may be assumed to start when a UE can access a satellite with store and forward capability and has MO data to send and to end when the MO data reaches the intended remote endpoint. Delay for transfer of MT data may be assumed to start when a remote endpoint has MT data to send to a UE and to end when the UE receives the MT data.

NOTE 2:	Regulatory services such as Lawful Interception are likely to be needed for store and forward operation. Solutions should consider how regulatory services may be supported with minimum impairment.