3GPP TSG-SA WG2 Meeting #159	S2-2311391

# October 09 – 13, 2023, Xiamen, China							 (revision of S2-2310514)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0331 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-path communication complement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The feature of Procedures for Multi-path communication via Uu and via 5G ProSe UE-to-Network Relay is missiing in UE function. - The current condition for the Multi-path communication via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay in clause 6.9.2 is not exact as it reads that the UE has already known that it will be the L3 remote UE. Actually, before evaluating the URSP, the UE doesn’t know that it will take the role of L3 remote UE and use the L3 relay to access to the network, therefore it is proposed to remove the condition . . |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the the Procedures for Multi-path communication via Uu and via 5G ProSe UE-to-Network Relay in UE function. - Reword the precondition of the UE to use the multipath including L3 relay case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The features of multipath support is missing in UE function. - The precondition for UE to perform multipath is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 6.9.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 4.3.1	UE

Any 5G ProSe-enabled UE may support the following functions:

-	Exchange of information for 5G ProSe Direct Discovery between 5G ProSe-enabled UE and the 5G DDNMF over PC3a reference point.

-	Procedures for 5G ProSe Direct Discovery of other 5G ProSe-enabled UEs over PC5 reference point.

-	Procedures for 5G ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-2 Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 Remote UE.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-2 End UE.

-	Procedures to act as a 5G ProSe Layer-3 End UE.

-	Procedures for communication path switching between PC5 and Uu reference points.

-	Procedures for Multi-path communication via Uu and via 5G ProSe UE-to-Network Relay.

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered 5G ProSe Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for 5G ProSe Direct Discovery;

-	Policy/parameters for 5G ProSe Direct Communication;

-	Policy/parameters for 5G ProSe Layer-2 Remote UE;

-	Policy/parameters for 5G ProSe Layer-3 Remote UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-2 End UE;

-	Policy/parameters for 5G ProSe Layer-3 End UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-UE Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay.

-	Receiving the 5G ProSe Policy from 5GC over N1 reference point.

-	Configuration of parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay (e.g. including IP addresses, ProSe Layer-2 Group IDs, see clause 5.1) and 5G ProSe UE-to-UE Relay. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

-	Reporting the following capabilities to 5GC over the N1 reference point:

-	5G ProSe Capability.

* * * * Next change * * * *

### 6.9.2	Multi-path communication via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay

When a 5G ProSe enabled UE accesses the network, if the UE decides to establish multi-path via direct Uu path and via the 5G ProSe Layer-3 UE-to-Network Relay without N3IWF based on "ProSe Multi-path Preference" in the selected RSD of the matched URSP rule for the application traffic as specified in clause 6.5.4, the following is performed:

-	The UE may establish a new PDU Session as specified in clause 4.3.2 of TS 23.502 [5] or modify an existing PDU Session as specified in clause 4.3.3 of TS 23.502 [5] for the direct Uu path.

-	The UE takes the role of 5G ProSe Layer-3 Remote UE, and either establishes 5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF as specified in clause 6.5.1.1 or modifies an existing PC5 connection with a 5G ProSe Layer-3 UE-to-Network Relay as specified in clause 6.4.3.6.

NOTE 1:	How to make use of the multi-path communication via direct Uu path and via a 5G ProSe Layer-3 UE-to-Network Relay for redundant or split traffic delivery is out of scope of this specifications.

When a 5G ProSe enabled UE accesses the network if the UE decides to establish multi-path via direct Uu path and via the 5G ProSe Layer-3 UE-to-Network Relay with N3IWF based on the "multi-access" Access Type of the selected RSD, the UE establishes a MA PDU Session as specified in clause 4.22 of TS 23.502 [5], the following is performed:

-	The UE sets up path over Uu by establishing a MA PDU session using the procedures for 3GPP access in clause 4.22 of TS 23.502 [5].

-	The UE taking the role of 5G ProSe Layer-3 Remote UE, if not connected and registered to the network via a Layer-3 UE-to-Network Relay with N3IWF, performs Relay (re)selection and performs registration with 5GC as specified in clause 6.5.1.2, and then performs MA PDU Session Establishment procedure or adds the user plane resources over the path via 5G ProSe Layer-3 UE-to-Network Relay with N3IWF using the procedures for non-3GPP access in clause 4.22 of TS 23.502 [5].

NOTE 2:	The Remote UE may establish MA PDU session over the path via 5G ProSe Layer-3 UE-to-Network Relay with N3IWF before the direct Uu path. The sequence could be arbitrary.

* * * * End of Change * * * *