3GPP TSG-WG SA2 Meeting #159	S2-2310524

# Xiamen, China, October 09 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0945 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on analytics collection from MDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) According to the LS (S5-235729) from SA5, assuming the MDAF discovery is achieved based on the supported MDA type, the NWDAF can then interact with the MDAF to obtain other detailed information (e.g., area of interest, geographical location and/or the target objects). However, the discoverable service information as part of MnSInfo IoC for MDA MnS is for future enhancements and not specified yet. Therefore, the current procedure for analytics collection from MDAF is modified to align with the LS and the relevant SA5 specifications (i.e. TS 28.537, TS 28.622 and TS 28.104),. 2) Both "MDAF" and "MDA Management Function" are used in the procedure. It is better to align the terminology. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the procedure for analytics collection from MDAF to align with SA5 MDA MnS related specifications. Change "MDA Management Function" to MDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for analytics collection from MDAF is not aligned with related SA5 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.14.1, 6.2.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[26]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[27]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[28]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[29]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[30]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[33]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[34]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[35]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[36]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[37]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[38]	GSMA TS.06: "IMEI Allocation and Approval Process".

[39]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[40]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[41]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRMs). Integration Reference Point (IRP): Information Service (IS)".

[42]	3GPP TS 32.422: "Subscriber and equipment trace: Trace control and configuration management".

[43]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[44]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[45]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[46]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[47]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[ts28622]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

* * * Next Change * * * *

6.2.14.1	General

The MDA functional overview and service framework as defined in Figure 5.1-1 of TS 28.104 [45] is used by NWDAF to trigger the MDA MnS to request analytics from the MDAF (Management Data Analytics Function).

Before NWDAF requests analytics from the MDAF, the NWDAF firstly discovers the MDAF via the MnS discovery service producer as defined in clause 5 of TS 28.537 [46].

6.2.14.2	Procedure for analytics collection from MDAF

Figure 6.2.14.2-1: Procedure for analytics collection from MDAF

Precondition: Initially MDAF(s) or MDA MnS producers register their capabilities, i.e. MnS information or MnS profile as described in clause 5 of TS 28.537 [46] to a MnS discovery service producer. The MnS discovery service producer may contain all or partial information related to the capabilities of MDA MnS producer.

1.	An analytics consumer issues a request or subscription of network analytics towards the selected NWDAF as described in clause 6.1.

2.	NWDAF discovers the MDAF from the MnS discovery service producer by sending a MnS producer discovery service operation. The service operation may include, for example, the following parameters:

-	requestedMDAType: indicates a specific MDA capability such as Slice coverage analysis, Mobility performance analysis as defined in clause 7.2 of TS 28.104 [45];

-	Area of Interest;

-	Network Slice information (i.e. NetworkSliceInfo including a DN (Distinguished Name) of the NetworkSlice managed object relating to the network slice instance associated to the S-NSSAI and NSI ID if available as defined in TS 28.541 [22]).

NOTE 1: The parameters for MDA MnS discovery are to be defined in the MnSInfo IoC in TS 28.662 [ts28622].

3.	The MnS discovery service producer provides the relevant MnS information of the MDAF to the NWDAF.

NOTE 1:	MnS information refer to the data describing a MnS producer and their capabilities, which is used by the consumer to discover the producers of specific Management Services and to derive the addresses of the Management Services as defined in TS 28.537 [46] and TS 28.662 [ts28622].

4.	If the MnS information of more than one MDAF is received, NWDAF selects the most suitable MDAF and gets the address of the selected MDAF from the MnS information. Then NWDAF sends analytics request to the MDAF by triggering a MDARequest service operation as defined in clause 9.3.2 of TS 28.104 [45]. The service operation may include the following parameters:

-	requestedMDAOutputs: MDAOutputPerMDAType(type of analytics) and MDAOutputIEFilters (which assist in indicating explicitly the analytics output requested from the set of MDA outputs and the conditions, considering i) threshold cross, for reporting numeric outputs and ii) timeout, when an output is no longer needed).

-	reportingMethod: indicates the selected reporting method for analytics output. It can only be notification-based reporting since file-based and streaming-based reporting are not supported in NWDAF.

-	reportingTarget: indicates the reporting target of the MDA outputs.

-	analyticsScope: Area of Interest, Network Slice information, etc.

-	startTime: indicates the start time of the analytics requested by the MnS consumer.

-	stopTime: indicates the stop time of the analytics requested by the MnS consumer.

NOTE 2:	Definitions and details of the parameters can be found in TS 28.104 [45].

NOTE 3:	The selection of the most suitable MDAF can be based on MnS information of the MDA MnS, e.g. MDA type as one MDA capability, and the request/subscription received from analytics consumer in step 1. The detailed information for MDA MnS discovery and selection is to be defined in the MnSInfo IoC in TS 28.662 [ts28622].

5.	The MDAF provides the analytics to the NWDAF by triggering an MDAReporting service operation.

6.	NWDAF provides the analytics response back to the analytics consumer after processing the analytics provided by the MDAF together with other data receives from NF sources according to the Analytics ID defined in clause 6.

* * * End of Changes * * * *