3GPP TSG-WG SA2 Meeting #159	S2-2311480

# Xiamen, China, Oct 9 – 13, 2023		(revision of S2-2310292)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0026 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Procedures of Ranging/Sidelink Positioning control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In clause 5.7.2, Ranging/SL Positioning QoS information is used for determination of Ranging/SL Positioning method. It proposes to update the flow to align with the principle. - It is proposed to update the figure and associating procedures in clause 6.8 to include 5GC-MO-LR procedure if the location is unknow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SL Positioning Server UE determines Ranging/SL positioning method based on the Ranging/SL Positioning QoS information, UE’s Ranging/Sidelink Positioning capabilities of all involved UEs. - Update the figures and step description in clause 6.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear who and how to determine Ranging/SL positioning method, and how to determine the required positioning QoS of each Located UE. Figure is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6, 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.6	Procedure of UE only Sidelink Positioning for Target UE using Located UE

This procedure is used to acquire absolute location of Target UE using Located UE, where LMF is not involved for SL Positioning/Ranging. The Located UE is assumed to already know its absolute location or can get its absolute location via the legacy 5GC-MO-LR Procedure, and its privacy profile allows its location to be shared with the Target UE.

Procedures for Ranging/Sidelink Positioning control (UE-only operation) defined in clause 6.8 applies with the following adaptations:

-	UE1 is the Target UE, and UE2 is the Located UE

-	Step1, the service request from Target UE application layer indicates that absolute location of the Target UE is required

-	Before step 6, target UE sends a scheduled location time (Time T) to each Located UE. If the absolution location of Located UE is not known, 5GC-MO-LR procedure is triggered, as specified in clause 6.2 of TS 23.273 [8], with the Time T included in the 5GC-MO-LR Request. The QoS requirement for Target UE's positioning in step 1, may be used by the Target UE to derive the required QoS for Located UE(s) positioning. Target UE sends the required QoS for Located UE positioning to each Located UE, and the required QoS for Located UE positioning is included in the 5GC-MO-LR Request.

-	Ranging/Sidelink positioning between target UE and located UE is scheduled with the Time T.

-	Step 7, absolute location of the Target UE is calculated.

# * * * * Second change * * * *

## 6.8	Procedures of Ranging/Sidelink Positioning control

Either UE-only Operation or Network-based Operation is applied in the Ranging/Sidelink Positioning control procedures.

UE-only Operation as specified in this clause is applied for the following cases:

-	Neither Target UE nor SL Reference UE is served by NG-RAN.

-	Network-based Operation is not supported by the 5GC network:

-	When Network-based Operation is not supported by the 5GC network, indication on whether the UE is allowed to use UE-only operation to perform Ranging/ SL Positioning is included in the Policy/Parameter provisioned to UE as defined in clause 5.1.1.2, and is provisioned to the UE as defined in clause 5.1.1.1. The Target UE will take it into account to initiate UE-only operation procedure.

-	SL-MO-LR request is rejected by the network.

For any other cases, Network-based Operation as specified in clauses 6.20 of TS 23.273 [8] is applied.

Figure 6.8.1-1 Procedures for Ranging/Sidelink Positioning control (UE-only operation)

1.	UE1 (i.e. Target UE) may receive a Ranging/SL Positioning Service request from:

1a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.6.1.1.

For absolute location, the service request includes the SL Positioning Client UE's user info and Target UE's user info, and required positioning QoS.

For relative location or ranging information, the service request includes the SL Positioning Client UE's user info, Target UE's user info, SL Reference UE's user info(UE2/.../UEn), and Ranging/SL Positioning QoS information.

1b.	RSPP application layer.

The service request includes type of the result (i.e. absolute location, relative location or ranging information) and the required QoS.

2.	UE1 discovers UE2/.../UEn (i.e. SL Reference UEs/Located UEs) as defined in clause 6.4, if needed.

NOTE 1:	Details of security related procedures during UE discovery are developed by SA WG3.

3.	If none of UE1/.../UEn are served by NG-RAN or the serving network does not support Ranging/SL Positioning, , UE-only Operation is applied.

4.	UE1 and UE2/.../UEn perform capability exchange. Step 4 may be performed during step 5 and step 6 with coordination of SL Positioning Server UE.

5.	If UE1 does not support SL Positioning Server functionalities, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE, or operated by a separate UE) is discovered (if not yet discovered in step 2) and selected. If a SL Positioning Server UE that is co-located with a SL Reference UE/Located UE or operated by a separate UE, UE1 discovers and selects the SL Positioning Server UE as described in clause 6.4 and requests SL Positioning Server UE to participate in the Ranging/Sidelink positioning.

NOTE 2:	Details of security and privacy related procedures during SL Positioning Server UE discovery and operation are developed by SA WG3.

5a.	If the absolution location of Located UE is not known, 5GC-MO-LR procedure is triggered, as specified in clause 6.2 of TS 23.273 [8].

6.	Sidelink Positioning assistant data is transferred among UE1/ .../UEn and the SL Positioning Server UE.

7.	SL-PRS measurement is performed between UE1 and UE2/.../UEn and possibly also amongst UE2/.../UEn.

8.	SL-PRS measurement data is transferred to the SL Positioning Server UE or is transferred to UE1 if it supports SL Positioning Server functionalities, in order to perform result calculation. Based on the type of the result received in step 1, absolute location, relative location or ranging information is calculated at the UE.

NOTE 3:	Details of step 4-8 are developed by RAN WGs.

9.	Ranging/SL Positioning result is transferred to:

9a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.6.1.1;

9b.	RSPP application layer.

# * * * * End of changes * * * *