3GPP TSG-WG SA2 Meeting #159	S2-2311486

# Xiamen, China, October 9 – 13, 2023		(revision of S2-2310154)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4255 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UPF anchored Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Ericsson , E |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The call flow in clause 4.24.4 contains the following errors: 1. Steps 2d and 2h assume that the AMF exhibits almost the same behaviour for UE in MICO mode vs UE configured for extended idle mode DRX, which is incorrect. - For UE in MICO mode the procedure ends in this step, whereas - For UE configured for extended idle mode DRX the AMF should store the information received from the SMF and then autonomously continue with the remainder of the procedure after the AMF determines that the UE has again become reachable. 2. Step 8e 3. Steps 7a, 7b and 7c describe only failure scenarios. For the case with buffering in the SMF there is currently no step describing a successful outcome whereby the AMF notifies the SMF that the UE has responded to paging (note that step 8a describing such a successful outcome is currently enclosed in the “Buffering configured in the UPF” box). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarified that, when UE is not reachable, the following happens: - If UE is in MICO mode, the procedure ends in step 2h. - If UE is configured for extended idle mode DRX, the AMF stores the information received in the SMF request and proceeds with the rest of the procedure when the UE becomes reachable. Note that a similar clarification was made in SA2#157 in the context of the NR_RedCap work item (refer to S2-2307731). In steps 2d/2h the AMF implicitly subscribes the SMF for notification when the UE becomes reachable. 2. Step 8a has been taken out of the “Buffering configured in the UPF” box, as it is used to indicate a successful outcome for both buffering in UPF and buffering in SMF. The service operation in step 8a is now a notification (Namf_EventExposureNotify), in response to the implicit subscription in steps 2d/2h. 3. Step 8a has been replaced with steps 8aa and 8ab: - step 8aa (Namf_MT-EnableUEReachability Response) applies when the AMF has determined in step 2c that the UE was immediately reachable - step 8ab (Namf_EventExposure Notify) applies when the AMF has implicitly subscribed the SMF in step 2d or 2h. 4. Step 8e has been taken out of the “Buffering configured in UPF” box because it applies to both buffering in UPF and buffering in SMF (in the latter case only if the AMF has implicitly subscribed the SMF for notification in step 2h). 5. Steps 7a, 7b and 7c are enclosed in a “Failure cases” box. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect call flow on UPF anchored Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.24.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The approved NR_RedCap CR in S2-2307731 clarifies additional aspects (e.g. UPF sending additional Data Notification for higher priority traffic leading the AMF to overwrite any stored information with the latest one). These aspects are not covered by the present CR. This CR has the following noteworthy stage 3 impacts: 1) alignment of the Namf_Communication_N1N2MessageTransfer service operation and the Namf_MT_EnableUEReachability service operation; 2) use of UE reachability notification in step 8a. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.24.2	UPF anchored Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation

This clause describes the procedures for Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation where the PDU Session is terminated at a UPF.

Figure 4.24.2-1: Mobile Terminated Data Transport in Control Plane CIoT 5GS Optimisation

1.	Downlink data is received by the UPF. If buffering is configured in the UPF, then the flow continues in step 2a, otherwise the flow continues in step 2f.

2a. [conditional] If this is the first downlink packet to be buffered and SMF has instructed the UPF to report the arrival of first downlink packet to be buffered, then the UPF sends a Data Notification to the SMF.

2b.	[conditional] The SMF sends a Data Notification ACK to the UPF.

2c.	[conditional] The SMF sends a Namf_MT_EnableUEReachability request to the AMF.

The SMF determines whether Extended Buffering applies based on local policy and the capability of the UPF. If Extended Buffering applies, the SMF includes "Extended Buffering support" indication in Namf_MT_EnableUEReachability request.

2d.	[conditional] If the UE is considered reachable, step 3 is executed immediately.

If the AMF determines the UE is unreachable due to the UE being in MICO mode, then the AMF rejects the request from the SMF with an indication that the UE is not reachable. If the SMF included Extended Buffering support indication, the AMF indicates the Estimated Maximum Wait time in the response message. If the UE is in MICO mode, the AMF determines the Estimated Maximum Wait time based on the next expected periodic registration timer update expiration or by implementation.

If the AMF determines the UE is unreachable due to the UE being configured for extended idle mode DRX, then the AMF stores the information received in the Namf_MT_EnableUEReachability request and sends a reply to the SMF with an indication that the AMF has accepted the request, but the UE is not immediately reachable. If the SMF included Extended Buffering support indication, the AMF indicates the Estimated Maximum Wait time in the response message. The AMF determines the Estimated Maximum Wait time based on the start of next PagingTime Window. The AMF implicitly subscribes the SMF for a notification when the UE becomes reachable. The continuation to step 3 is postponed until the UE becomes reachable.

If the AMF detects that the UE context contains Paging Restriction Information, the AMF may block the paging for this UE based on the stored Paging Restriction Information (see clause 5.38.1 of TS 23.501 [2]). If the AMF blocks paging, the AMF sends MT_EnableUEReachability response to the SMF with an indication that its request has been rejected due to restricted paging.

2e.	[conditional] If the SMF receives an "Estimated Maximum Wait time" from the AMF and Extended buffering applies, the SMF sends a failure indication to the UPF with an Extended Buffering time and optionally a DL Buffering Suggested Packet Count. The Extended Buffering time is determined by the SMF and should be larger or equal to the Estimated Maximum Wait time received from the AMF. The DL Buffering Suggested Packet Count parameter is determined by the SMF and if available, the Suggested Number of Downlink Packets parameter may be considered. The procedure stops after this step.

2f.	[conditional] If buffering is not configured in the UPF, then the UPF forwards the downlink data to the (V-)SMF in non-roaming and LBO cases. In the home-routed roaming case, the H-UPF forwards the data to the V-UPF and then to the V-SMF.

2g.	[conditional] The SMF determines whether Extended Buffering applies based on local policy and the capability of the SMF.

If user data is received in step 2f and Extended buffering is not configured for the SMF, then (V-)SMF compresses the header if header compression applies to the PDU session and creates the downlink user data PDU that is intended as payload in a NAS message. The (V-)SMF forwards the downlink user data PDU and the PDU session ID to the AMF using the Namf_Communication_N1N2MessageTransfer service operation. If Extended Buffering applies, then (V-)SMF keeps a copy of the downlink data.

If user data is received in step 2f and Extended Buffering applies, the SMF includes "Extended Buffering support" indication in Namf_Communication_N1N2Message Transfer.

2h.	[conditional] AMF responds to SMF.

If AMF determines that the UE is reachable for the SMF, then the AMF informs the SMF. Based on this, the SMF deletes the copy of the downlink data.

If the AMF determines the UE is unreachable for the SMF due to the UE being in MICO mode, then the AMF rejects the request from the SMF. If the SMF included Extended Buffering support indication, the AMF indicates the Estimated Maximum Wait time, in the reject message, for the SMF to determine the Extended Buffering time. The AMF determines the Estimated Maximum Wait time based on the next expected periodic registration timer update expiration or by implementation. The AMF stores an indication that the SMF has been informed that the UE is unreachable. The procedure ends here.

If the AMF determines the UE is unreachable for the SMF due to the UE being configured for extended idle mode DRX, then the AMF stores the information received in the Namf_Communication_N1N2MessageTransfer request and sends a reply to the SMF with an indication that the AMF has accepted the request, but the UE is not immediately reachable. If the SMF included Extended Buffering support indication, the AMF indicates the Estimated Maximum Wait time, in the response message, for the SMF to determine the Extended Buffering time. The AMF determines the Estimated Maximum Wait time based on the start of next PagingTime Window. The AMF implicitly subscribes the SMF for a notification when the UE becomes reachable. The continuation to step 3 is postponed until the UE becomes reachable. If the UE is considered reachable, step 3 is executed immediately.

If the AMF detects that the UE context contains Paging Restriction Information, the AMF may block the paging for this UE, based on the stored Paging Restriction Information (see clause 5.38.1 of TS 23.501 [2]). If the AMF blocks paging, the AMF sends Namf_Communication_N1N2MessageTransfer response to the SMF with an indication that its request has been rejected due to restricted paging.

If the SMF receives an "Estimated Maximum Wait time" from the AMF and Extended Buffering in SMF applies, the SMF store the DL Data for the Extended Buffering time. The Extended Buffering time is determined by the SMF and should be larger or equal to the Estimated Maximum Wait time received from the AMF. The SMF does not send any additional Namf_Communication_N1N2MessageTransfer message if subsequent downlink data packets are received.

3.	[Conditional] If the UE is in CM-IDLE, when the AMF determines that the UE is reachable, the AMF sends a paging message to NG-RAN. If available, the AMF may include the WUS Assistance Information in the N2 Paging message(s).

4.	[Conditional] If NG-RAN received a paging message from AMF and UE and NG-RAN support WUS, then:

-	if the NGAP Paging message contains the Assistance Data for Recommended Cells IE (see TS 38.413 [10]), then NG-RAN shall only broadcast the UE's Wake Up Signal in the last used cell;

-	else (i.e. the Assistance Data for Recommended Cells IE is not included in the NGAP Paging message) NG-RAN should not broadcast the UE's Wake Up Signal.

NG-RAN performs paging. If the WUS Assistance Information is included in the N2 Paging message, the NG-eNB takes it into account when paging the UE (see TS 36.300 [46]).

5.	[Conditional] If the UE receives paging message, it responds with a NAS message sent over RRC Connection Establishment.

If the UE is in CM-IDLE state in 3GPP access and is using the Multi-USIM Paging Rejection feature (see clause 5.38 of TS 23.501 [2]), upon reception of paging request and if the UE determines not to accept the paging, the UE attempts to send a Reject Paging Indication via the UE Triggered Service Request procedure (clause 4.2.3.2) unless it is unable to do so, e.g. due to UE implementation constraints.

5a.	[Conditional] In the NB-IoT case, during Step 5, the NG-RAN, based on configuration, may retrieve the NB-IoT UE Priority and the Expected UE Behaviour Parameters from the AMF, if not previously retrieved. Based on such parameters, the NG-RAN may apply prioritisation between requests from different UEs before triggering step 6 and throughout the RRC connection. The NG-RAN may retrieve additional parameters (e.g. UE Radio Capabilities).

6.	[Conditional] The NAS message is forwarded to the AMF. If the AMF receives any Paging Restrictions information in the Control Plane Service Request, the AMF updates the UE context with the received Paging Restrictions information. If no Paging Restriction information is provided, no paging restrictions apply.

7a.	[Conditional] AMF to SMF: Namf_MT-EnableUEReachability Response.

If the SMF used the MT_EnableUEReachability request in step 2c and the UE has not responded to paging then the AMF sends a response to the SMF indicating that the request failed.

If the SMF used the MT_EnableUEReachability service operation in step 2c and the UE has responded with a Control Plane Service Request NAS message including Reject Paging Indication in step 5, the AMF notifies the SMF using the MT_EnableUEReachability response that the UE rejected the page and no user plane connection will be established. The UE remains reachable for future paging attempts.

7b.	[Conditional] SMF to UPF: If the SMF has received a Namf_MT-EnableUEReachability response from the AMF indicating that the request failed or the UE rejected the paging, the SMF indicates to the UPF to discard the buffered data. . If the AMF indicated that the UE rejected the paging, steps 7c-12 are skipped. Otherwise the procedure stops after this step.

7c.	[Conditional] AMF to SMF: Namf_Communication_N1N2Transfer Failure Notification.

If the SMF used the Namf_Communication_N1N2MessageTransfer service operation in step 2g and the UE has not responded to paging, the AMF sends a failure notification to the SMF based on which the SMF discards the buffered data. The procedure stops after this step.

If the SMF used the Namf_Communication_N1N2MessageTransfer service operation in step 2g and the UE has responded with a Control Plane Service Request NAS message including Reject Paging Indication in step 5, the AMF notifies the SMF using the Namf_Communication_N1N2MessageTransfer Failure Notification that the UE rejected the page and no user plane connection will be established. The UE remains reachable for future paging attempts. Steps 9-12 are skipped.

8aa.	[Conditional] AMF to SMF: Namf_MT-EnableUEReachability Response.

If the SMF used the MT_EnableUEReachability request in step 2c and steps 2d-2e were skipped, then the AMF indicates to the SMF that the UE is reachable.

8ab.	[Conditional] AMF to SMF: If the AMF has implicitly subscribed the SMF for a notification when the UE becomes reachable in step 2d or 2h, the AMF uses the Namf_EventExposure Notify service operation indicating to the SMF that the UE is reachable.

8b.	[Conditional] SMF to UPF: N4 Session Modification Request.

If the SMF received an indication from the AMF that the UE is reachable, then the SMF indicates to the UPF to deliver buffered data to the SMF.

8c.	[Conditional] UPF to SMF: N4 Session Modification Response.

8d.	[Conditional] Buffered data is delivered to the SMF.

8e.	[Conditional] (V-)SMF compresses the header if header compression applies to the PDU session and creates the downlink user data PDU that is intended as payload in a NAS message. The (V-)SMF forwards the downlink user data PDU and the PDU session ID to the AMF using the Namf_Communication_N1N2MessageTransfer service operation.

When buffering is not configured in the UPF, this step is executed only if the AMF has implicitly subscribed the SMF for a notification when the UE becomes reachable in step 2h.

9.	The AMF creates a DL NAS transport message with the PDU session ID and the downlink user data PDU received from the SMF. The AMF ciphers and integrity protects the NAS transport message.

10.	The AMF sends the DL NAS transport message to NG-RAN.

11.	NG-RAN delivers the NAS payload over RRC to the UE.

12.	While the RRC connection is established further uplink and downlink data can be exchanged. In order to send uplink data, the procedure continues as per steps 1-10 of the UPF anchored Mobile Originated Data Transport in Control Plane CIoT 5GS Optimisation procedure (clause 4.24.1).

13.	If the AMF has paged the UE to trigger the NAS procedure as in step 3-6, the AMF shall initiate the UE configuration update procedure as defined in clause 4.2.4.2 to assign a new 5G-GUTI. If the UE response in the Control Plane Service Request NAS message includes a Reject Paging Indication, the AMF triggers the release of the UE as specified in clause 4.2.3.2.

14.	[Conditional] If no further activity is detected by NG-RAN, then NG-RAN triggers the AN release procedure.

15.	[Conditional] The UE's logical NG-AP signalling connection and RRC signalling connection are released according to clause 4.2.6.

NOTE:	The details of the NGAP messages to be used for this procedure are specified in TS 38.413 [10].

# * * * * End of changes * * * *