SA WG2 Meeting #159	S2-2311501

# Xiamen, PRC, October 9-13th, 2023        	  	  			       		          (revision of S2-2310300)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2113 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting Miscellaneous mistakes Non related with AUN3 devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Clause 9.9 Policy control subscription information management reads To support that the *PCF of a UE* may, as described in clause 4.10, take the TNAP ID into account in policy decisions This should be the PCf of the PDU Session - § 4.10b reads “NAUN3 devices may be associated with Connectivity Group IDs" which gives the wrong understanding that some configuration is required for these devices to use a Connectivity Group ID. Actually it is the usage of a 5R RG port or SSID, etc.. that associates an NAUN3 device with a Connectivity Group ID |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 9.9: To support that the *PCF of a PDU Session* may, as described in clause 4.10, take the TNAP ID into account in policy decisions - § 4.10b updating text “NAUN3 devices may be associated with Connectivity Group IDs" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect speoifications that may confuse BBF and CableLabs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ; 9.9 ; 4.10b ; 4.10d ; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Remove changes to § 4.10 ; |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 9.9	Policy control subscription information management

This clause specifies the delta related to policy control subscription information defined in clause 6.2.1.3 of TS 23.503 [4] for 5G-RG and FN-RG.

To support that the PCF of a PDU Session may, as described in clause 4.10, take the TNAP ID into account in policy decisions when the UE connects via trusted non-3GPP access over wireline access, following information may be supported in PDU Session policy control subscription information for the UE that is defined in Table 6.2-2 of TS 23.503 [4]:

Table 9.9-1: Extract of Table 6.2-2 of TS 23.503 [4]

| Information name | Description | Category |
| --- | --- | --- |
| List of TNAP ID(s) | The list of identifiers of TNAP collocated with 5G-RG associated with the subscriber | Optional |

NEXT CHANGE

## 4.10b	Differentiated services for NAUN3 devices behind 5G-RG

NAUN3 devices cannot be authenticated by 5GC but may e.g. be locally authenticated by the 5G-RG using e.g. pre-shared secret. Differentiated services (QoS, network slicing) may be provided for NAUN3 devices as defined in this clause.

"Connectivity Group IDs" may be defined on the 5G-RG where each Connectivity Group ID corresponds to a separate physical or virtual port on the 5G-RG. These ports could, for example, refer to separate physical ethernet ports and/or to separate WLAN SSIDs and/or to a separate VLAN. The devices that connect to a certain logical port are considered part of the same Connectivity Group ID. How this configuration on the 5G-RG is done is out of scope of this specification.

Each Connectivity Group ID is then mapped to a separate PDU Session that is established by the 5G-RG based on the procedures defined in clause 7. The overall architecture is illustrated in Figure-4.10b-1.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with the (virtual) port information (e.g. VLANs and SSIDs) based on TR-69 [18], TR-360 and TR-181 [46]. URSP rules can be provided to the 5G-RG to indicate how to map Connectivity Group ID to the parameters of the PDU Session used to carry the traffic of corresponding devices e.g. DNN, S-NSSAI, etc.

NOTE:	In addition, the mapping between a "virtual port" and DNN/S-NSSAI can be configured via TR-69 [18]/TR-181 [46].

Whether and How the NAUN3 devices are configured to use a specific SSID or connect to a certain Ethernet port on the 5G-RG is out of scope of this specification.

Differentiation of charging and QoS may be provided via PCC rules (for different service flows) related with dedicated PDU Sessions for NAUN3 devices. Isolation of devices using a specific Connectivity Group ID into a specific network slice, i.e. with separate S-NSSAI may also be provided.

NEXT CHANGE

## 4.10d	Support of NSWO for 3GPP UE behind a RG

NSWO as defined in clauses 4.2.15 and 5.42 of TS 23.501 [2] may be supported for UE(s) connected via a 5G-RG, and/or for UE(s) connected via a FN-RG.

When this feature is supported, the RG and the W-5GAN need to support the WLAN Access functionality defined in clauses 4.2.15 and 5.42 of TS 23.501 [2]. The WLAN Access functionality includes the support of the SWa' interface to NSWOF. The SWa' support in Wireline access network has no impact on 3GPP specifications.

NOTE:	W-5GAN specifications and deployments can ensure that a AAA proxy is used to support SWa' interface with NSWOF(s) on behalf of RG(s). This can be used for FN-RG that do not support SWa'. This AAA proxy does not need to support the functionalities of a 3GPP AAA proxy defined in TS 23.402 [45].

When NSWO applies, the user plane traffic of the UE is not traversing the UE's 5GC.

The specification of functionalities to support NSWO in the wireline access network is out of 3GPP scope including specifications on how the offloaded traffic is carried in W-5GAN and bypass the 5GC of the UE.

The UE can also connect to 5GC using 5GS credentials as defined in clause 5.42 of TS 23.501 [2].

A 5G RG shall not issue authentication request over SWa' for the UE if it is itself not registered to 5GC.

END OF CHANGES