SA WG2 Meeting #S2-159	S2-2311566

October 09 - 13, Xiamen, China	(revision of S2-2311454)

Source:	Futurewei, InterDigital Inc., China Mobile, Google

Title:	XRM Ph 2 Architectural Assumptions for Encrypted Media

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM Ph2 /Rel-19

Abstract of the contribution: This paper proposes architectural assumptions for encrypted media in FS_XRM_Ph_2.

# 1	Discussion

The proposal below identifies some architectural assumptions for the XRM Ph2 study.

Many of the assumptions are self-explanatory and hence more discussions are not provided here.

Assumptions on the scenarios for WT#1.2 may be useful:

“WT#1.2 Support QoS control and PDU Set identification for XR stream with e2e encryption (e.g. fully encrypted header, partially encrypted header). This is applicable for PDUs received at N6 for DL.”

Media applications may use different protocols between the media server and UE for fully or partially encrypted transport from end-to-end including:

- RTP cryptex (RFC 9335),

- Media-over-QUIC (draft-ietf-moq-transport),

- RTP over QUIC, or

- SRTP with RTP header extensions that are encrypted (RFC 6904)

In addition to different protocols, different business scenarios may exist in practice. For example,

- the CDN or application network that provides the application may be able to deploy the media relay at the UPF or in proximity and the media relay is able to provide classification parameters.

- But in many other cases, the application provider may not have agreements to share keys for application-level/content level metadata directly to the mobile network operator.

- In yet other cases, the application and 5G providers are the same operator (e.g., a private network) that operate as a trusted domain or conversely are separated by untrusted networks in between.

This discussion above is not intended to enumerate the different use cases for encrypted media packets that need to be classified. The aim is to outline that many different scenarios need to be supported and that may in turn use different protocols, extensions, and other mechanisms to derive classification information. For this study, the assumption should be that the UPF should be able to map and classify information using possibly multiple protocols and business scenarios corresponding to several application domains.

Proposal for architectural assumption based on the above are in the 2nd change below.

# 2	Proposal

It is proposed to adopt the following changes into TR 23.700-70.

* * * * 1st Change (revision marked) * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", STD 64, July 2003.

[6]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[7]	IETF RFC 8285: A General Mechanism for RTP Header Extensions

[8]	IETF RFC 6904: Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)

[9]	IETF RFC 9335: Completely Encrypting RTP Header Extensions and Contributing Sources

[10]	IETF draft: RTP over QUIC (RoQ), draft-ietf-avtcore-rtp-over-quic

[11]	IETF draft: Media over QUIC Transport, draft-ietf-moq-transport

[12]	IETF experimental draft: Frame Marking RTP Header Extension, draft-ietf-avtext-framemarking

Editor’s note: [10], [11] and [12] cannot be formally referenced until published as RFC.

* * * * 2nd Change (revision marked) * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Fully Encrypted Media Flow: A media flow where both the media header and media payload are encrypted from end-to-end. Fully encrypted headers and payload are not visible in the network. Examples include RTP cryptex [9], RTP over QUIC (RoQ) [10] and Media over QUIC (MoQ) [11].

Partially Encrypted Media Flow: A media flow where some media headers (e.g., base header) are not encrypted. Other media headers (e.g., extension header) and media payload are encrypted from end-to-end. The payload and headers that are encrypted from end-to-end are not visible in the network. Examples include SRTP [6] with partially encrypted header extensions [8] [12].

* * * * 3rd Change (revision marked) * * * *

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

- End-to-end media flows may be fully or partially encrypted.

* * * * End of Changes * * * *