3GPP TSG-WG SA2 Meeting #159	S2-2311613

Xiamen, China, October 09 – 13, 2023

Title:	LS Reply on Data rate monitoring

Response to:	S2-2310108/ C4-233573

Release:	Rel-18

Work Item:	XRM

Source:	3GPP SA2

To:	3GPP CT4

Cc:	3GPP CT3

Contact person:	Chunshan Xiong

chushan.xiong@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description

SA2 thanks CT4 WG for the liaison (S2-2310108/ C4-233573) on Data rate monitoring for XR service.

Regarding the questions raised by CT4, SA2 would like to provide the following replies:

Question 1:	the data rate information that the UPF shall expose. 
For instance, if the (PFCP) Measurement Period IE (indicating the period to generate periodic measurement reports) is set to 10 seconds and the monitoring averaging window is 2 seconds, should the UPF measure the data rate during 2 seconds only once within the 10 seconds and report this measurement, or should the UPF measure the data rate up to 5 times within the 10 seconds and report the minimum, maximum and average data rate over the measurement period?

Answer:  	The UPF is required to measure and report the average data rate over the last standardized monitoring averaging window (e.g. the last 2 second).

Question 2:  what is the standardized value of the monitoring averaging window for non-GBR QoS flows?

Answer:  	The standardized monitoring averaging window is used for monitoring measurement of data rate of the GBR and Non-GBR QoS Flow. It is assumed that the value of the standardized monitoring averaging window is to be defined by CT4.

2	Actions

To CT4:

ACTION: 	SA2 kindly asks CT4 to take the above into account into their standardization work.

3	Dates of next TSG SA WG2 meetings

TSG-SA2 Meeting #160		13-17 November 2023                Chicago, US

TSG-SA2 Ad hoc Meeting	22-29 January 2024                    e-meeting