3GPP TSG-SA2 Meeting #159	S2-2311781

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023		             (revision of S2-2311498)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2117 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling devices behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provides clarity on handling of AUN3 and NAUN3 devices by 5G-RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction in Figure 4.10b-1 to clarify that the NAUN3 device(s) can connect to 5GC via 5G-RG and NG-RAN or 5G-RG and W-5GAN. 2. Clarify that a device operating as AUN3 via 5G-RG can be supported as NAUN3 if the connection between 5GC and 5G-RG via W-5GAN is unreachable, and once the connection is restored 5G-RG can support the same device as AUN3 again. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in handling of AUN3 and NAUN3 devices by 5G-RG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10b, 4.10c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.10b	Differentiated services for NAUN3 devices behind 5G-RG

NAUN3 devices cannot be authenticated by 5GC but may e.g. be locally authenticated by the 5G-RG using pre-shared secret. Differentiated services (QoS, network slicing) may be provided for NAUN3 devices as defined in this clause.

NAUN3 devices may be associated with "Connectivity Group IDs" where each Connectivity Group ID corresponds to a separate physical or virtual port on the 5G-RG. These ports could, for example, refer to separate physical ethernet ports and/or to separate WLAN SSIDs and/or to a separate VLAN. The devices that connect to a certain logical port are considered part of the same Connectivity Group ID. How this configuration on the 5G-RG is done is out of scope of this specification.

Each Connectivity Group ID is then mapped to a separate PDU Session that is established by the 5G-RG based on the procedures defined in clause 7. The overall architecture is illustrated in Figure-4.10b-1.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with the (virtual) port information (e.g. VLANs and SSIDs) based on TR-69 [18], TR-360 and TR-181 [46]. URSP rules can be provided to the RG to indicate how to map Connectivity Group ID to the parameters of the PDU Session used to carry the traffic of corresponding devices e.g. DNN, S-NSSAI, etc.

NOTE:	In addition, the mapping between a "virtual port" and DNN/S-NSSAI can be configured via TR-69 [18]/TR-181 [46].

How the NAUN3 devices are configured to use a specific SSID or connect to a certain Ethernet port on the 5G-RG is out of scope of this specification.

Differentiation of charging and QoS may be provided via PCC rules (for different service flows) related with dedicated PDU Sessions for NAUN3 devices. Isolation of devices using a specific Connectivity Group ID into a specific network slices, i.e. with separate S-NSSAIs may also be provided.

# * * * * Second change * * * *

## 4.10c	Authenticable Non-3GPP devices behind 5G-RG

This clause defines the support of AUN3 devices, i.e. Authenticable Non-3GPP devices (AUN3) as defined in clause 3.1, behind a 5G-RG. This clause applies only to 5G-RG connected via wireline access.

Figure 4.10c-1 shows the architecture for support of AUN3 device.

Figure 4.10c-1: AUN3 device behind 5G-RG

Differentiated services for AUN3 devices behind 5G-RG are provided as specified below:

-	Each AUN3 device has its own UDM/UDR subscription data including its own SUPI and policy control subscription data.

-	The interface between 5G-RG and AUN3 devices is out of scope of 3GPP.

-	In order to serve the AUN3 device in 5GC, a 5G-RG issues a NAS register and handles RM and CM related signalling on behalf of an AUN3 device that it is requesting to be served and relays EAP signalling between the AUN3 device and the 5GC.

-	A 5G-RG serving an AUN3 device establishes a single PDU Session on behalf on this AUN3 device.

-	The AMF and the 5G-RG maintain a separate NAS connection per AUN3 device. This includes maintaining a GUTI and NAS (RM, CM, security, etc.) context per AUN3 device.

-	A 5G-RG shall be connected to the 5GC (be in RM-REGISTERED and CM-CONNECTED mode) over Wireline access to serve an AUN3 device: the 5G-RG shall not issue a NAS register or service request on behalf of an AUN3 device if it is itself not registered and connected to the 5GC.

-	The 5G-RG is configured with URSP for each AUN3 devices it serves. The UE PCF selected by the AMF at the registration of an AUN3 device sends this URSP to 5G-RG via the AMF and the NAS connection of the AUN3 device.

-	The AUN3 devices and the 5G-RG belong to the same PLMN.

-	A 5G-RG uses default values, which are the same for all AUN3 devices it serves, to populate the parameters in the Registration Request message built on behalf of an AUN3 device. For example, the 5G-RG issues the Registration Request with no S-NSSAI and the AMF selects the default S-NSSAI in the subscription of the AUN3 device.

-	There shall be a separate N2 connection per AUN3 device that is in state CM-CONNECTED.

-	The W-CP and W-UP protocols shall be able to manage multiple separate Registrations and PDU Sessions for different SUPIs between the same pair of 5G-RG and W-AGF. In particular, W-CP needs to be able to differentiate NAS messages related to a 5G-RG and to each different AUN3 device served by this 5G-RG and W-UP needs to distinguish between user plane packets for a 5G-RG and each different AUN3 device served by this 5G-RG.

-	When the registration of an AUN3 device has successfully completed, the 5G-RG establishes a PDU Session on behalf of the AUN3 device. This PDU Session is handled by 5GC as part of the AUN3 subscription and is associated with the SUPI of AUN3 device. An AUN3 device can at a given time only use a single PDU Session. The parameters to establish this PDU session are based on the URSP (if any) for the AUN3 device.

-	Different QoS parameters may apply to PDU sessions of different AUN3 devices.

-	Roaming is not applicable to subscriptions for AUN3 devices.

If W-AGF detects that a 5G-RG is unreachable, then W-AGF triggers the N2 UE context release. The W-AGF identifies if there exists any AUN3 device connected to the 5G-RG through the W-AGF. For each identified AUN3 device, the W-AGF invokes step 5 and 6 of Figure 7.2.8.3-1 which releases the PDU sessions of these AUN3 devices. If the 5G-RG is still registered to the 5GC via NG-RAN, based on configuration, the 5G-RG  may allow  the AUN3 device to reconnectto the network as an NAUN3 device via the 5G-RG and NG-RAN as defined in clause 4.10b. Once the 5G-RG connection to 5GC via W-5GAN is restored and the 5G-RG is registered to the 5GC via W-5GAN, the 5G-RG may disconnect the device acting now as an NAUN3 device via NG-RAN and allow the device to reconnect to the network as an AUN3 device via W-5GAN as defined in clause 4.10c.

NOTE: How the 5G-RG supports the switching of device between AUN3 and NAUN3 based service depending on 5G-RG registration status on W-5GAN is to be defined by BBF and CableLabs. It is assumed that the procedure has no additional impact on 5GS.

# * * * * End of changes * * * *