SA WG2 Meeting #S2-160AH-e	S2-2400027

22 - 29 January, 2024, Electronic meeting

SA WG2 Meeting #S2-160	S2-2313236

13 - 17 November, 2023, Chicago, USA

3GPP TSG-SA3 Meeting #113 	S3-235078

Chicago, US, 6 - 10 November 2023

Title:	Reply LS on security aspects for Ranging/Sidelink Positioning

Response to:	LS S3-234481/S2-2310025 on Reply LS on key and security materials used for Ranging_SL from SA2

Release:	Release 18

Work Item:	Ranging_SL

Source:	SA3

To:	SA2, RAN2

Cc:

Contact person:	Wei Lu

Luwei10(at)xiaomi(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-235029

# 1	Overall description

SA3 thanks SA2 for the LS in S2-2310025. SA3 has discussed on the questions in S2-2310025 as well as in S2-2305727, and would like to provide the following answers:

Q4:	For Ranging/SL positioning service exposure to the SL positioning client UE, either through PC5 or through the 5GC network, privacy aspects of exposing the location of the Target UE/SL Reference UE may need to be studied by SA3.

A4:	The requirement on privacy protection of the Target UE/SL Reference UE for Ranging/SL Positioning service exposure is already specified in TS 33.533 clause 6.3.2.

The procedure on privacy check of the Target/Reference UEs for location exposure to SL positioning client UE through 5GC network is specified in the approved S3-235029 as attached.

For privacy check of the Target/Reference UEs for location exposure to SL positioning client UE through PC5 link, there are two proposals for network-based operation and UE-only operation respectively.

-	For network-based operation, it was proposed that the Target/Reference UE includes in the SL-MO-LR towards the LMF with Client UE's user info that is received in the Ranging/SL Positioning service request from the Client UE. The LMF triggers privacy check of Target UE and Reference UE towards GMLC.

Question1 to SA2: SA3 would like to request confirmation from SA2 whether the UE already supports adding user info of the Client UE within SL-MO-LR and the GMLC already supports to provide the required service to the LMF triggering privacy check or can be supported in Rel-18 timeframe.

-	For UE-only operation, it was proposed that UE1 includes in the SL service request towards UE2 with Client UE's user info that is received in the Ranging/SL Positioning service request from the Client UE. With such info, UE2 can determine whether the location related information can be exposed to Client UE.

Question2 to SA2/RAN2: SA3 would like to request confirmation from SA2/RAN2 whether PC5 message or SLPP message already supports to carry the required information between the UEs or can be supported in Rel-18 timeframe.

SA3 has not agreed on the answers to Q3 and Q5.

# 2	Actions

To SA2, RAN2

ACTION: 	Take SA3 answers as above into consideration and provide feedback on the questions raised in Answer 4.

# 3	Dates of next TSG SA WG 3 meetings

SA3#114	26 Feb. - 01 Mar. 2024	Athens, GR

SA3#115	20 - 24 May 2024		Korea, KR