3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400117

# E-meeting, January 22 – 29, 2024		(revision of S2-2312313 and S2-2310489)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0046 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update service exposure via control plane. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA3 has concluded the security mechanisms for Ranging and Sidelink positioning in TS 33.533. For SL Positioning Client UE using Ranging/SL Positioning service exposure through 5GC C-plane, the authorization reuses the authorization for MO-LR as specified in TS 23.273. The MO-LR authorization is performed by the AMF by checking the subscriber information detailed in clause 7.1 in TS 23.273. After the AMF authorized the SL Positioning Client UE, the AMF send a LCS service request to the GMLC which triggers the SL-MT-LR procedures. The SL-MT-LR includes the privacy check. The Privacy check is done by the GLMC by retrieving the UE LCS Privacy Policy Information from the UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the ENs related to authorization and privacy handling to align with SA3 specification and add the corresponding text needed. Correct a wrong reference. Change GPSI to Application Layer ID Move subbullet in step 4 related to AMF selection earlier as the GMLC selects the AMF in an earlier step in the referred SL-MT-LR procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authorization of a SL Positioning Client UE is not complete. Wrong UE ID is used in the procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.4, 6.7.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 23.273 CR 0445 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR 0445 for TS 23.273 adds the information used in clause 5.6.2.4 and 6.7.1.2.3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.6.2.4	Ranging/SL Positioning service exposure through 5GC C-plane

When Ranging/SL Positioning service exposure to the SL Positioning Client UE is provided through the control plane of 5GC, the following procedures are carried out:

-	The SL Positioning Client UE invokes the Ranging/SL Positioning service exposure via the SL-MO-LR procedure with Sidelink Positioning as defined in clause 6.7.1.2.3.

-	The AMF check SL Positioning Client UE authorization in the subscription information as specified in TS 33.533 [13].

-	The AMF based on the request selects a GMLC and forwards the Ranging/SL positioning request to the GMLC for the Ranging/SL Positioning service with the SL Positioning Client UE ID, i.e. SUPI. The GMLC checks the UE LCS privacy profile of the UEs as specified in TS 23.273 [8] clause 5.4.

-	The AMF sends the Ranging/SL positioning result to the SL positioning client UE when it receives the result from the GMLC.

NOTE:	In this release, for the control plane-based procedure, the SL Positioning Client UE and the UE who provides the exposure service are registered in the same PLMN.

# * * * * Next change * * * *

##### 6.7.1.2.3	Procedure for Ranging/SL Positioning service exposure through 5GC network via control plane

Figure 6.7.1.2.3-1: Exposure through 5GC network via control plane

1.	The application layer of the SL Positioning Client UE triggers the SL Positioning Client UE to request the network to perform the Ranging/SL positioning between two more Ues, e.g. between the UE1 and UE2. If the SL Positioning Client UE in CM-IDLE state, it firstly triggers the service request procedure to enter CM-CONNECTED state.

2.	The SL Positioning Client UE sends the SL-MO-LR request to the AMF1 in the UL NAS TRANSPORT message for the Ranging/SL Positioning operation.

The SL-MO-LR request includes the parameters as defined in clause 5.6.1 for the Ranging/SL positioning service requirement: UE-1 ID (e.g. UE1's Application Layer ID), UE-2 ID (e.g. UE2's Application Layer ID), Ranging/SL Positioning service type, Requested Quality, etc. The Ranging/SL Positioning service type indicates the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. SL Reference UE) via PC5 interface. The Requested Quality may include delay, accuracy, etc.

3.	The AMF1 check SL Positioning Client UE authorization in the subscription information as specified in TS 33.533 [13]. Based on the request type, the AMF1 may select a GMLC supporting Ranging/SL Positioning using NRF query or configuration in AMF1, and forwards the SL-MO-LR to the GMLC for Ranging/SL Positioning result via Ngmlc_Location_ProvideRanging_Request, the AMF includes the SL Positioning Client UE ID, i.e. SUPI, in the request.

4.	SL-MT-LR procedure defined in clause 6.20.3 of TS 23.273 [8] is performed, with the following adaptations:

-	step 2: The GMLC checks the UE LCS privacy profile of UE1 and UE2, whether the SL Positioning Client UE is allowed to retrieve location information of UE1 and UE2.

-	step 3: GMLC selects the AMF (i.e. AMF2 if the serving AMF is not the same as serving the SL Positioning Client UE) serving either UE1 or UE2.

-	step 4: it is indicated in step 4 that relative positioning or ranging information is required.

5.	The GMLC returns the result to the AMF1 via Ngmlc_Location_ProvideRanging_Response.

6.	The AMF1 returns the result to the SL Positioning Client UE via DL NAS TRANSPORT message.

# * * * * End of changes * * * *