3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400291

E-meeting, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Samsung

Title:	New KI for WT#1.1: Reduce impact on central NFs for Edge Computing

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: This pCR proposes a new KI for WT#1.1.

1. Introduction/Discussion

This KI is proposed to address the WT #1.1.

WT#1) Whether and how to support more efficient Edge Hosting Environment information management and related EAS Discovery:

- WT#1.1: Handle the edge network and EAS related information (e.g. UPF and EAS deployment information, DNAIs) locally with less impact to 5GC central NFs (e.g. central SMF) to address more flexible EAS (re)discovery/(re)selection, local UPF (re)selection.

2. Text Proposal

It is proposed to introduce the following changes in TR 23.700-49.

# * * * * First change * * * *All new

5.X	Key Issue #X: Reduce impact on central NFs for Edge Computing

5.X.1	Description

The purpose of this key issue is to investigate whether and how to reduce impact on central NFs for Edge Computing by supporting localized Control Plane, e.g., localized management of edge hosting environment (EHE)-related information and control of EAS (re)discovery/relocation and UP path management.

In Rel-18 and 19, the support of Edge Computing imposes burdens concentrated on a few central Control Plane NFs. For example, the information for EAS (re)discovery/relocation and UP path management is handled and stored by the SMF whenever the EHE-related information (e.g., EAS Deployment Information) or UE location varies. This may also cause signalling overheads across operator’s data centers for a deployment scenario where Control Plane NF(s) such as SMF is deployed in the central data center while other involved NFs such L-NEF, AF or EASDF are located in the local data center. To reduce these overheads, some procedures such as EAS (re)discovery/relocation and local UPF selection can be supported via assistance of localized Control Plane NF(s) with reducing impacts to the central NF(s).

The following aspects shall be studied:

-	How EAS (re)discovery/relocation and local UPF (re)selection can be assisted by localized Control Plane NF(s), including how to decide whether to apply such localized supports for a given UE, PDU Session or region

-	How EHE-related information (e.g., EAS deployment information) provisioned from external parties can be locally managed and utilized

# * * * * End of changes * * * *