3GPP TSG-WG SA2 Meeting #160AH-e	S2-2400944

22 - 29 January, 2024, Electronic meeting	(is revision of S2-2311471)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5067 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on delay in deregistration procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Samsung], Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Change-1: Below text is giving an impression that UE context modification message sent to NG-RAN should be delayed but rather actual intention is deregistration procedure should be delayed. “the AMF sends the UE Context Modification message to NG-RAN before triggering the deregistration towards MBSR after a certain period (e.g. based on the expiration of a timer configured on the AMF) Thus above text is clarified as below: the AMF sends the UE Context Modification message to NG-RAN and after a certain period (e.g. based on the expiration of a timer configured on the AMF) the AMF triggers the deregistration procedure with MBSR and releases the NAS signalling connection. Change-2: When MBSR status changes from authorised to not authorised The AMF delays the de-registration procedure to allow the IAB-donor gNB to move all connected UEs via MBSR to other cells as specified in clause 8.9.10 of TS 38.401 [42] The above behaviour of AMF should not be restricted to the case of authorization change only but rather for all the reasons when MBSR needs to be deregistered. Change-3: Moved NOTE 3 to right place under deregistration delay related text. It was wrongly placed. Change-4: When MBSR status changes from authorised to not authorised, the AMF delays the de-registration procedure to make sure the UEs served by the MBSR can be moved to other cells. So, there is a time interval between the AMF sending UE Context Modification message to NG-RAN and executing Deregistration procedure. According to the description, the time interval is determined by a timer configured on the AMF. However, there may be some issues with this. - If the timer is longer than the actual action time in RAN, the MBSR(IAB-DU) may attempt to re-establish F1. - If the timer is shorter than the actual action time in RAN, the UE service may be interrupted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change-1: Clarify the text on delay in dereg procedure. Change-2 : In general AMF will delay deregistration procedure to allow the IAB-donor gNB to move all connected UEs via MBSR to other cells Change-3: Moved NOTE 3 to right place. Change-4: To align with RAN3, the AMF executes deregistration procedure after receiving the NGAP UE CONTEXT RELEASE REQUESTmessage due to the NG-RAN node having completed the operation (e.g. all the UEs served by the MBSR have been moved out) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Ambiguous text has a chance of misinterpretation. - There can be service continuity issues for the UEs connected to MBSR. - Misalignment with RAN3. (The MBSR(IAB-DU) will attempt to re-establish F1 even the authorization status changed into “not-authorized” or the UE service may be interrupted) for a non-authorized MBSR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods.

NOTE 1:	For non-roaming MBSRs, the operator local policy can be taken into consideration for MBSR authorization, e.g. based on the network status, or a limit on the number of MBSRs operating in a certain area.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to the MBSR node over NAS and NG-RAN over NGAP as described in the registration procedure in TS 23.502 [3]. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation upon the reception of MBSR authorization indication (authorized).

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place. The AMF can make use of the subscription data for authorization of the MBSR in the V-PLMN.

MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into the serving PLMN to provide service.

NOTE 2:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.

If the MBSR operation is not authorized (e.g. due to location or time limitation), the AMF of the MBSR can indicate to the MBSR IAB-UE that it is not allowed to act as an MBSR, i.e. the MBSR authorization indication (not authorized), as part of registration procedure. In this case, the AMF includes the MBSR authorization indication (not authorized) to donor-gNB. The AMF may provide the indication either in a Registration Accept (if the PLMN allows the MBSR IAB-UE to be registered in the PLMM) or in a Registration Reject (if the PLMN does not allow the MBSR IAB-UE to be registered in the PLMN) message. The MBSR provides the authorization indication (not authorized) to its AS layer. The AMF may provide additional information to the MBSR regarding the authorization indication (not authorized) to assist the MBSR behaviour control.

Editor's note:	The details of the additional information and corresponding MBSR behaviour will be added.

Editor's note:	Whether the MBSR needs to provide the authorization indication (not authorized) to its AS layer will be further synch with RAN WGs.

When the MBSR authorization state changes for a registered MBSR node (either authorized, or not authorized), the AMF updates the MBSR and the NG-RAN accordingly. Based on the operator configuration, the AMF may use either Deregistration (including the option with re-registration required indication) or the UE Configuration Update procedure to inform the MBSR regarding the updated authorization status:

-	When Deregistration procedure is used, AMF provides the new authorization indication and the additional information, e.g. proper cause code, when the MBSR performs Registration procedure after the deregistration.

-	When UE Configuration Update procedure is used, the AMF provides the new authorization indication and additional information to the MBSR in the UE Configuration Update Command. The MBSR provides the new authorization indication to its AS layer.

The AMF informs the NG-RAN of the new authorization status using UE Context Modification, Initial Context Setup procedure or the DOWNLINK NAS TRANSPORT message, with the following principles:

-	If the authorization state changes from authorized to not authorized and AMF uses the UE Configuration Updated procedure to update the MBSR, the AMF updates the NG-RAN with the new authorization indication (not authorized) by including this information in the DOWNLINK NAS TRANSPORT message. The NG-RAN completes handover of the UEs served by the MBSR before releasing the F1 connection to the MBSR IAB-DU.

-	If the authorization state changes from authorized to not authorized and the AMF uses the Deregistration procedure to update the MBSR, the AMF sends the UE Context Modification message to NG-RAN and after receiving the NGAP UE CONTEXT RELEASE REQUEST message due to the NG-RAN node having completed the operation (e.g. all the UEs served by the MBSR have been moved out) for a non-authorized MBSR,  the AMF executes the deregistration procedure with MBSR and releases the NAS signalling connection before continuing with the AN Release procedure..

If the Network-initiated Deregistration procedure is triggered for MBSR IAB-UE that is registered with authorization to act as MBSR, the AMF sends the UE Context Modification message to NG-RAN and updates the NG-RAN with the authorization indication as not authorized and after receiving the NGAP UE CONTEXT RELEASE REQUEST message due to the NG-RAN node having completed the operation (e.g. all the UEs served by the MBSR have been moved out), the AMF executes the deregistration procedure with MBSR and releases the NAS signalling connection before continuing with the AN Release procedure.

NOTE 3:	The AMF delays the MBSR de-registration to allow the IAB-donor gNB to move all connected UEs via MBSR to other cells as specified in clause 8.9.10 of TS 38.401 [42].

If a PDU session is used to provide OAM access for MBSR when it is not authorized but remains registered, the AMF may notify the SMF serving the PDU session for O&M access to trigger the release of the PDU Session.

NOTE 3:	The AMF delays the MBSR de-registration to allow the IAB-donor gNB to move all connected UEs via MBSR to other cells as specified in clause 8.9.10 of TS 38.401 [42].

NOTE 4:	The mechanism applies to both roaming and non-roaming MBSR operations.

*************************** End of Changes ****************************