3GPP TSG-SA2 Meeting #160-Ad Hoc-e	S2-2401356

# Online, , 22nd Jan 2024 - 29th Jan 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4615 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data collection for Location Accuracy Analytics via AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Location Accuracy Analytics require that a mode is trained with data derived by two meassurement methods. It is unclear how to obtain input data with different meassurement methods from the GMLC. One solution proposed in S2-2312622 is that the data collection AF can inquire global navigation satelite system location information from the UE and provide it to the NWDAF. A related event for the AF Naf_EventExposure service needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An event for the Naf_EventExposure service to request data collection of global navigation satelite system location information from UE(s) is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Limited possibilities for the data collection for model training for the location accuracy analytics.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.19.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.19.2	Naf_EventExposure service

##### 5.2.19.2.1	General

Service description: This service enables consumer NF to subscribe and get notified of events.

The events can be subscribed by a NF consumer are described in TS 23.288 [50].

The following service operations are defined for the Naf_EventExposure service:

-	Naf_EventExposure_Subscribe.

-	Naf_EventExposure_Unsubscribe.

-	Naf_EventExposure_Notify.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	Service Experience information, as defined in clause 6.4.2 of TS 23.288 [50].

-	Performance Data information, as defined in clauses 6.4.2 and clause 6.14.2 of TS 23.288 [50].

-	Collective Behaviour information, as defined in clause 6.5.2 of TS 23.288 [50].

-	UE Mobility information, as defined in clause 6.7.2.2 of TS 23.288 [50].

-	UE Communication information, as defined in clause 6.7.3.2 of TS 23.288 [50].

-	Exceptions information, as defined in clause 6.7.5.2 of TS 23.288 [50].

-	User Data Congestion information, as defined in clause 6.8.2 of TS 23.288 [50].

-	UE Data volume Dispersion information, as defined in clause 6.10.2 of TS 23.288 [50].

-	DN Performance information, as defined in clause 6.14.2 of TS 23.288 [50].

-	Global Navigation Satellite System (GNSS) UE location as defined in clause 6.17.2 of TS 23.288 [50].

-	E2E data volume transfer time information, as defined in clause 6.18.2 of TS 23.288 [50].

-	Relative Proximity information, as defined in clause 6.19.2 of TS 23.288 [50].

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AF exposure.

Table 5.2.19.2.1-1: Example of Event Filters for AF exposure events

| Event ID for AF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Exceptions information | <Parameter Type = Exception ID, Value = Exception ID1> |
| Service Experience information | <Parameter Type = TAI, Value = TAI1> |
| Service Experience information | <Parameter Type = geographical area, Value = civic address1 or shape1> |
| Collective Behaviour information | <Parameter Type = collective attribute, Value = collective value1> |
| Collective Behaviour information | <Parameter Type = data processing type, Value = data processing type1> |