SA WG2 Meeting #S2-160AH-e	S2-2400012

22 - 29 January, 2024, Electronic meeting

SA WG2 Meeting #S2-160	S2-2311959

13 - 17 November, 2023, Chicago, USA

3GPP TSG-CT WG4 Meeting #118	C4-234414

# Xiamen, P.R. China; 09th– 13th October 2023

# Title:	Reply LS on Enhancement on the attribute for 5GLAN management

# Response to:	LS S5-235780 (C4-234376) on Enhancement on the attribute for 5GLAN management from SA5

# Release:	Release 18

# Work Item:	5GLAN_Mgt (990027)

Source:	3GPP CT4

To:	3GPP SA5

Cc:	3GPP SA2

Contact Person:	Rong Wang

Name:	Rong Wang

Tel. Number:	+86 13810948370

E-mail Address:	wangrongyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

CT4 thanks SA5 for their liaison S5-235780 (C4-234376) in which SA5 asks CT4 two questions.

Q1: Please clarify which identifiers is used to support 5G LAN-type services, as stated in TS 29.510, the attribute of internalGroupId has already defined and supported by the trusted AF in Table 6.1.6.2.96-1: Definition of type TrustAfInfo. However, 5g-vn-groups is used in TS 29.503.

A: SA2 specified that the External Group ID and Internal Group ID are used to identify the 5G VN group (refer to clause 5.29.2 of 3GPP TS23.501). For Internal Group ID, CT4 defined the internalGroupIdentifier to uniquely identify a 5G VN group in UDM (Please refer to clause 6.5.6.2.6 of 3GPP TS29.503).

Besides, here are some additional information for reference:

-	The External Group ID is mainly used by the AF/NEF to enable the dynamic 5G VN group management (E.g., add/delete/modify). After the NEF provides the External Group ID to UDM, the UDM will map the External Group ID to Internal Group ID. For a newly created 5G VN Group, an Internal Group ID is determined by the UDM based on implementation specific means. (Please refer to clause 5.29.2 of 3GPP TS23.501).

-	The AmfInfo/SmfInfo/NrfInfo/TrustAfInfo definition in TS29.510 is used for node selection, but not used for identifying a 5G VN group. Since there is a 1:1 mapping between 5G VN group and the combination of SNSSAI and DNN, the SNSSAI and DNN will be used for the SMF/NRF selection in serving a 5G VN group member. That is why CT4 didn’t define any 5G VN group related information in SmfInfo/NrfInfo. AMF node selection in serving a 5G VN group member is based on SNSSAI. The AmfInfo is only used by other SBI based NF instances (e.g., SMF) to select AMF. So, CT4 does not need to define any 5G VN group related information in AmfInfo.

-	The 5g-vn-groups used in TS 29.503 is not the identifier to a 5G VN group, it is a part of the path in the resource URI of 5GVnGroupConfiguration.

Q2: SA5 would kindly ask CT4 to make the necessary updates to their specifications to enable 5G LAN-type services (e.g., enhancement for the AmfInfo/SmfInfo/NrfInfo).

A: CT4 didn’t see the need to define the 5G LAN-type service in NRF for AmfInfo/SmfInfo/NrfInfo, since such information is not used for node selection.

2. Actions:

To SA5 group.

ACTION: 	CT4 asks SA5 to take the above into consideration.

3. Date of Next CT4 Meetings:

CT4#119	November 13th 2023 – November 17th 2023	Chicago, USA

CT4#120	January 22th 2024 – January 26th 2024		Online (Under discussion)

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm