3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400099

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Tencent, Tencent Cloud

Title:	KI#6, New Sol: Support of L4S mechanism for non-3GPP access networks and intermediate 5GS nodes

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract: This pCR proposes the solution to support L4S mechanism for non-3GPP access networks and intermediate 5GS nodes.

1. Introduction/Discussion

In Key Issue#6 of XRM_Ph2, the following key issue is captured:

The objective of this Key Issue is to extend the L4S mechanism to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes.

The following aspects should be studied:

-	How to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S.

-	Support L4S in untrusted/trusted access (e.g., N3IWF, TNGF).

-	Support L4S in wireline access (e.g. W-AGF).

NOTE: 	It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

## 6.X	Solution #X: Support L4S mechanism for non-3GPP access networks and intermediate 5GS nodes

### 6.X.1	Key Issue mapping

This solution applies to Key Issue #6 i.e. L4S for non-3GPP access networks and intermediate 5GS nodes.

### 6.X.2	Description

This solution takes the architecture models for non-3GPP access networks as basis which is specified in Clause 4.2.8 as basis which includes the following aspects.

- Non-roaming architecture for 5G Core Network with untrusted non-3GPP access (Figure 4.2.8.2.1-1 of TS 23.501)

- Non-roaming architecture for 5G Core Network with trusted non-3GPP access (Figure 4.2.8.2.1-2 of TS 23.501)

- Non- roaming architecture for 5G Core Network for 5G-RG with Wireline 5G Access network and NG RAN (Figure 4.2.8.4-1 of TS 23.501)

### 6.x.3 Procedures and Impacts to support L4S

### 6.X.3.1 Support of L4S in Non-roaming architecture for 5G Core Network with untrusted non-3GPP access

Referring to Figure 4.2.8.2.1-1 of TS 23.501, impacts to NFs are listed as follows:

N3IWF:

-    N3IWF supports PDU set QoS handling like NG-RAN/gNB as N2/N3 termination point and support ECN marking for L4S

SMF:

- Configure QoS profiles to N3IWF as to NG-RAN for PDU set based QoS handling

UPF, AF:

-  Same functions as L4S mechanisms from NR-RAN case

### 6.X.3.2 Support of L4S in Non-roaming architecture for 5G Core Network with trusted non-3GPP access

Referring to Figure 4.2.8.2.1-2 of TS 23.501, impacts to NFs are listed as follows:

TNGF:

-    TNGF supports PDU set QoS handling like NG-RAN/gNB as N2/N3 termination point and support ECN marking for L4S

SMF:

- Configure QoS profiles to TNGF as to NG-RAN for PDU set based QoS handling

UPF, UE, AF:

-  Same functions as L4S mechanisms from NR-RAN case

### 6.X.3.2 Support of L4S in Non-roaming architecture for 5G Core Network for 5G-RG with Wireline 5G Access network and NG RAN

Referring to Figure 4.2.8.4.-1 of TS 23.501, the impacts are listed as follows:

5G-RG:

- 5G-RG support ECN marking for L4S functions in 5G UE as specified in TS 23.501

W-AGF:

- W-AGF support the ECN marking for L4S in NG-RAN node as specified in TS 23.501

# * * * * Second change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #6> |  |
| #X | x |  |

# * * * * End of changes * * * *