SA WG2 Meeting #160-Ad Hoc-e	S2-2400125

# January 22 – 29, 2024, Elbonia

Source: 	NTT DOCOMO

Title: 	KI#2: New Sol: Energy-aware data transfer policy negotiation

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a solution for KI#2 where AF can negotiate for an energy-aware data transfer policy.

# 1	Discussion

5GC provides “Background Data Transfer” and “Planned Data Transfer with QoS Requirement” policy negotiation services that allow AF to negotiate with 5GC to transfer data; however, these policies do not consider the energy related aspects (e.g., energy consumption). It is proposed to introduce a new policy negotiation services to consider energy as well.

# 2 Proposal

It is proposed to update TR 23.700-66 according to the following text.

********** First Change (All new text) **********

## 6.x	Solution #X: Energy Aware Data Transfer Policy Negotiation

### 6.x.1	Key Issue mapping

The proposed solution maps to the following aspect of KI#2:

‐	Whether and how to enhance the existing subscription and policy control framework to support energy related information as service criteria.

-	Whether and what new energy related policies are to be defined, and how to perform energy related policy control, e.g. to determine, provision and enforce energy related policies.

### 6.x.2	Functional Description

Currently, PCF provides two services “Npcf_BDTPolicyControl” and “Npcf_PDTQPolicyControl” that allow AF to negotiate with 5GC for “Background Data Transfer” (BDT) and “Planned Data Transfer with QoS requirements” (PDTQ), respectively. However, these policies do not consider the energy-related aspects. To support energy related information as service criteria, it is proposed to extend 5GC functionalities to provide a service similar to BDT and PDTQ that allows AF to negotiate “Energy Aware Data Transfer” (EADT).

The overall procedure of EADT is similar to that of BDT and PDTQ policy negotiation procedures. AF initiates the negotiation by specifying its desired settings (e.g., desired time window or desired QoS profile). PCF takes this request and considering the AF preferences and also the status of the network (which can be predicted by NWDAF for time windows in future) creates a number of policies and shares them with AF; then AF selects a policy and inform the PCF. Later, AF requests the QoS flow in the time window specified in the selected policy. In the proposed solution, in the EADT policy negotiation, in comparison to the BDT and PDTQ policy negotiations, (i) the AF can share its adaptability in terms of QoS level and transfer time and (ii) PCF also takes the energy-related aspect of service provision into account and consequently, the derived policies consider energy-related information as service criteria. The details of these two aspects are explained in the following paragraphs.

To consider energy related information as service criteria, AF may adapt the transfer of the data in two aspects:

- Adaptability in QoS requirements: On one hand, providing different QoS levels by 5GS may consume different amounts of energy; on the other hand, similar to the “Alternative QoS Profile” concept, different level of QoS profiles might be acceptable for the AF. Hence, for the energy aspects of the service (e.g., energy consumption, and carbon emission), the AF can indicate its adaptability in terms of QoS requirements.

- Adaptability in transfer time: Energy related aspect of a service (e.g., renewable energy consumption ratio) may differ in different time windows, consequently, to consider energy related information as service criteria, similar to the BDT and PDTQ policies, AF may adapt the transfer time of the data.

Based on these observations, in the proposed solution, in addition to the parameters that existing in the BDT and PDTQ policies like “data volume”, “Desired time window”, “Desired QoS profile”, the AF can also indicate its degree of adaptability in terms of QoS requirements and/or transfer time.

PFC is the NF responsible for deriving the policies and responding to the AF. The policies are derived based on the AF desired settings and also considering network status. To support energy related information as service criteria, in deriving the policies, PCF considers not only availability of the network resources but also energy related aspect of service provisioning, that may include:

- Energy related information of RAN segment of the service

- Energy related information of the 5GC segment of the service

- Energy consumption of the service

- Energy efficiency of the service

- Carbon emission of the service

- Renewable energy consumption ratio of the service

The overall procedure of proposed solution is composed of the following steps. At the first step, according to the desired transmission time, desired QoS level and the degrees of adaptability provided by AF, PCF derives a number of “Transmission Opportunity” options where each option is composed of (i) transmission time and (ii) QoS level. Each option is time window and QoS level proposal for the AF to transmit the data. In the second step, PCF needs to evaluate each option considering both the network resources and the mentioned energy aspect. The PCF is already capable of evaluating network resources for example in PDTQ. Obtaining these energy related information would require additional communication and computation; hence, to facilitate it, two enhancements in 5GC are proposed:

- Introducing a new NF named EMF (Energy Manager Function)

- Extending the functionality of NWADF

EMF is responsible for providing (an estimate) of the energy related aspect of each option to PCF. For this purpose, it may use analytics provided by NWDAF. The functionality of NWDAF is enhanced to provide Energy Analytics to predict the status of the network from energy perspective in future time windows.

NOTE 1: EMF may include other functionalities to address the other KIs as well,

NOTE 2: Enhancing NWDAF functionalities to provide energy analytics is discussed in KI#3.

Once, PCF has the energy related information of a transmission opportunity, it derives the corresponding policy considering resource availability and operator’s policy. Finally, PFC send the list of the policies to the AF. AF evaluates the polices and selects one of them and informs the PCF. Later, AF request the QoS flow at the time specified in the selected policy.

Editor’s Note: Charging aspect of EADT is FFS.

### 6.x.3	Procedures

The procedure depicted in Figure 6.x.3-1 allows AF to negotiate an energy aware time window with specified QoS level to transmit data.

Figure 6.x.3-1: Negotiation for energy aware data transfer

Prior to the transfer of the data, the AF negotiates with the 5G Core for the EADT policies. At first, AF discovers its serving NEF before starting the following procedure.

1a.	The AF invokes the Nnef_EADTPolicyNegotiation_Create Request (ASP Identifier, target UE, Desired time windows, Desired QoS Profile, Transfer time adaptability, QoS level adaptability).

1b-1c. The NEF may authenticate the AF and authorize the EADT request from the AF. If the authentication/authorization of the AF's request has failed, the NEF will respond to the AF's request through the Nnef_EADTPolicyNegotiation_Create Response with a failure result and the following steps are skipped.

2.	Based on an AF request, the NEF may translate the information provided by the AF and invokes the Npcf_EADTPolicyControl_Create with the PCF to authorize the creation of the policy.

3.	PCF queries the UDR to retrieve all existing EADT polices using Nudr_DM_Query service operation.

4.	The UDR provides all the stored EADT policies and corresponding related information to the PCF.

5.	Based on the desired QoS level and transmission time and also the corresponding degree of adaptabilities provided by the AF, the PCF derive a number of transmission opportunity options. Then steps 6-9 are performed for each option.

6.	For the given option, PCF requests the energy related information of the option from the EMF.

7-8. EMF collects the current and future energy related information from the OAM and NWDAF and estimates the energy related parameters (e.g., energy consumption, carbon emission, etc.) of the option.

9.	EMF provides the estimated energy related information of the transmission opportunity option to the PCF.

10.	By referring to the information provided by the EMF, the PCF determines one or more EADT policies. Each EADT policy includes a recommended time window and a recommended QoS profile for the traffic transfer for each of the AF sessions for each of the UEs involved.

11.	The PCF sends one or more EADT policies to NEF in Npcf_EADTPolicyControl_Create Response including the EADT Reference ID.

12.	The NEF sends a Nnef_EADTPolicyNegotiation_Create response to the AF to provide one or more EADT policies together with the EADT Reference ID.

13.	The AF selects one of the EADT policies and notifies NEF for the selected EADT policy via Nnef_EADTPolicyNegotiation_Update request together with the EADT Reference ID. The AF stores the EADT Reference ID for the future interaction with the PCF.

14-16. The NEF notifies PCF about the selected EADT policy by the AF. The PCF acknowledges NEF. The NEF responds to the AF request with a Nnef_EADTPolicyNegotiation_Update Response.

17-18. The PCF stores the EADT Reference ID together with the new EADT policy in the UDR by invoking Nudr_DM_Update. The UDR sends a response to the PCF as acknowledgement.

### 6.x.4	Impacts on existing services, entities and interfaces

The solution has the following impacts:

EMF:

-	New network functionalities to provide the energy-related information.

-	New network functionalities to gather energy and performance related information from OAM based on the requirements from the AF.

NEF:

-	Extending service to negotiate EADT policy.

PCF:

-	Extending service to negotiate EADT policy.

NWDAF:

-	Extending functionalities to provide energy analytics.

********** End of Changes **********