SA WG2 Meeting #160-Ad Hoc-e	S2-2400139

E-Meeting, January 22-29, 2024	(was S2-24XXXX)

Source:	Ericsson

Title:	KI 1: Update to Solution 1

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes an update to solution to KI #1.

# 1. Proposal

It is proposed to accept the following changes to TR 23.700-77 rev 0.2.0

FIRST CHANGE

## 6.1	Solution #1: Architectural Enhancements in Support of exposure of IMS events using HSS Subscription

### 6.1.1	Description

In this solution, DCSF subscribes to Data Channel Session Control events defined in Table AA.2.4.2.2-1 in Annex AA.2.4.2 of TS 23.228 related to a given IMS user of interest via HSS. When the event is fulfilled, the IMS AS notifies the event to DCSF directly or via HSS.

Additionally, in this solution, it is also proposed that a similar approach is used for trusted and untrusted Application Servers to subscribe to IMS events of interest via the NEF. When the IMS exposure event is related to a given IMS user, the NEF subscribes to the corresponding IMS AS allocated to the IMS user via the HSS. When the event is fulfilled, the IMS AS notifies the IMS exposure event to the NEF.

Editor’s Note:	Definition of IMS events which can be exposed by this solution is FFS.

To support the above, once an IMS subscriber successfully registers in IMS and if at least one IMS AS is selected for this IMS subscriber, the IMS AS register in HSS the IMS AS instance allocated for the subscriber. using an enhanced Nhss_imsUECM service over N71 for that purpose.

Editor’s Note:	Which IMS AS(s) register in HSS for the IMS subscriber is FFS.

Editor’s Note:	The use of Diameter for registering the IMS AS instance in HSS is FFS.

For DC Session Control related events specified in Annex AA.2.4.2 of Ts 23.228 [5], DCSF subscribes with HSS for such an event using a new service Nhss_imsEE over N72. HSS subscribes with IMS AS using Nimas_SessionControEvent Subscribe Request over N71.

For other type of IMS events, an AF subscribes to NEF for an IMS event for a subscriber via a new service Nnef_imsEE, over N33. The NEF in turns forwards such a subscription request to HSS via a new service Nhss_imsEE over Nxx.

HSS subscribes to the event with the identified IMS AS instance via a new service Nimsas_imsEE, over N71. For unregistered UEs, the HSS keeps the subscription request and subscribes to the requested IMS event when the UE registers in IMS.

Upon detection of the requested IMS event, the IMS AS notifies the event directly to the NEF/DCSF via a new service Nimsas_imsEE, over DC1/Nyy.

The architecture depicting the solution is shown in Figure 6.1.1-1 below.

Figure 6.1.1-1 : IMS Event Framework

Figure 6.1.1-1includes the following reference points:

- N33 is enhanced to enable an AF to subscribe to, and receive notifications about the requested IMS event of interest via the NEF.

- A new reference point between NEF and HSS is introduced to enable NEF to subscribe to HSS for IMS related events for a specific subscriber. This reference point is labelled Nxx.

- N72 is enhanced to allow the DCSF to subscribe to DC Session Control related events via the HSS.

NOTE Y: DCSF can also subscribe to other IMS events of interest.

- N71 is enhanced to enable HSS to receive registration of an AS instance allocated for a specific subscriber upon initial IMS registration and to subscribe to the IMS AS instance of a specific subscriber for any IMS event of interest.

NOTE Z: IMS AS supports subscription to DC Session Control related events as well as other IMS events of interest. The existing Subscribe service operation place holder in TS 23.228 [5] in Nimsas_SessionEventControl for Session Event Control is dedicated only to DC Session Control related events.

- A new reference point between IMS AS and NEF is introduced to enable IMS AS to send notifications regarding directly to the NEF. This reference point is labelled Nyy.

- DC1 is enhanced to allow the DCSF to receive notifications for DC Session Control related events.

Editor’s Note:	Handling of exposure of IMS events for a group of IMS subscribers is FFS.

### 6.1.2	Procedures

#### 6.1.2.1 Subscription to DC Events Via HSS

To subscribe to DC Session Control Event specified defined in Table AA.2.4.2.2-1 in Annex AA.2.4.2 of TS 23.228 [5], DCSF subscribes to HSS for that purpose by sending the Nhss_ImsEE-Subscribe Request and includes the DC Session Control Event, as well as the subscriber Identity (e.g IMPU) in the request. When any of the defined events in Table AA.2.4.2.2-1 is fulfilled, a notification is generated including the relevant information for the event as defined in Annex AA.2.4..2.2 of TS 23.228 [5].

Figure 6.1.2.1-1 depicts a typical call flow for DCSF subscribing to DC Session Control events via HSS.

6.1.2.2-1 DCSF Subscription to Data Channel Session Control Events

1) UE performs initial IMS Registration.

2) S-CSCF performs third party Registration with the IMS AS instance allocated to the registering UE.

3)	IMS AS registers in HSS using Nhss_ImsUECM_Registration operation the IMS AS instance allocated to the UE.

4) 	DCSF subscribes to DC Session Control event by sending to HSS a Nhss_ImsEE_Subscribe Request. The requested event is DC Session Control event. The request includes the subscriber IMPU.

5) 	HSS returns to DCSF Nhss_ImsEE_Subscribe Response including the Correlation ID.

6) HSS in turn locates the IMS AS instance serving the UE. Subsequently, HSS sends to the IMS AS an Nimsas_SessionControlEvent Subscribe Request and includes the subscriber IMPU.

7) IMS AS returns to HSS Nimsas_SessionControlEvent Subscribe Response including the Correlation ID.

There are two options in this call flow. In option 1, DCSF requests direct Notification from IMS AS. In the second option the notification is received from HSS. For option 1, the received notification request would be Nimsas_SessionControlEvent Notify. In the second option, the notification received from HSS would be Nhss_ImsEE_Notfiy.

8a) When the requested event is fulfilled, IMS AS returns to DCSF an Nimasas_SessionControlEvent_Notifty Request including the the relevant event information and the Correlation ID.

9a) DCSF returns Nimasas_SessionControlEvent_Notifty Request

8b) When requested event is fulfilled, IMS AS returns to HSS an Nimasas_SessionControlEvent_Notifty Request including the received the relevant event information and the Correlation ID.

9b) HSS returns to DCSF an Nhss_ImsEE_Notify Request including the received the relevant event information and the Correlation ID.

10b) DCSF returns to HSS Nhss_ImsEE_Notify Response.

11b) DCSF returns to HSS Nimsas_SessionControlEvent_Notify Response.

#### 6.1.2.2 Subscription to IMS Generic events via NEF

6.1.2.2-1 AF Subscription to IMS events via NEF

Figure 6.1.2.2-1 depicts a typical call flow for an AF subscribing to an IMS event for a UE.

- UE performs initial IMS Registration.

- S-CSCF performs third party Registration with the IMS AS instance allocated to the registering UE.

- IMS AS registers in HSS using Nhss_ImsUECM_Registration operation the IMS AS instance allocated to the UE.

- AF subscribes to NEF initiating the Nnef_ImsEE_Subscribe Request for a specific IMS event.

- NEF subscribes to HSS using the Nhss_ImsEE_Subscribe Request for a specific IMS event.

- An NF, e.g. DCSF, may also subscribe for an IMS event for a subscriber via HSS.

- HSS locates the IMS AS instance(s) serving the UE.

- HSS subscribes to the IMS AS instance(s) serving the UE using Nimsas_ImsEE_Subscribe Request for the requested IMS event.

- IMS AS returns to HSS the Nimsas_ImsEE_Subscribe Response

- HSS returns to NEF (or to the NF, e.g. DCSF) the Nhss_ImsEE_Subscribe Response.

- NEF returns to AF the Nnef_ImsEE_Subscribe Response

- At some point, the requested event for the UE is detected by the IMS AS. The IMS AS sends an Nimsas_ImsEE_Notify Request to NEF

- NEF sends to AF Nnef_ImsEE Notify Request.

#### 6.1.2.3 Additional Services and Operations in Support of IMS Event subscription

##### 6.1.2.3.1 Enhancements to hss Services

Nhss_imsUECM service operations are modified to support registering a UE IMS AS instance.

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nhss_ImsUE Context | Register | Request/Response | S-CSCF, IMS AS instance |
| Management (Nhss_ImsUECM) | Deregister | Request/Response | S-CSCF, IMS AS instance |

Nhss_ImsUECM service

Nhss_ImsUECM Register service operation

Service operation name: Nhss_ImsUECM_Register

Description: This service operation registers the serving NF instance for an initial IMS registering subscriber.

Inputs, Required: Public Identity, Registering Identity representing the identity of the registering NF.

The list of allowed Identities to register their instance are configured based on operator policy.

Outputs, Required: Result indication.

Nhss_ImsUECM Deregister service operation

Service operation name: Nhss_ImsUECM Deregister

Description: This service operation deregisters the serving NF instance for an IMS de-registering subscriber

Inputs, Required: Public Identity, Deregistering Identity representing the identity of the deregistering NF.

Outputs, Required: Result indication.

Outputs, Optional: None.

Services provided by HSS are enhanced to enable NEF and DCSF to subscribe for non-Data Channel session control events of interest related to a given IMS user. To that effect, a new service is introduced for IMS Event Exposure as depicted below.

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nhss_imsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | NEF, DCSF |
|  | Unsubscribe | Subscribe/Notify | NEF, DCSF |

##### 6.1.2.3.1 Enhancements to Nimsas Services

Service provided by IMS AS are also enhanced to enable HSS to subscribe to an IMS AS instance for non-Data Channel session control events of interest related to a given IMS user. To that effect, a new service is introduced for IMS Event Exposure as depicted below.

| Service | Service Operations |  | Operation Semantics |  | Example Consumer(s) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nimsas_imsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | HSS |
|  | Unsubscribe |  | Subscribe/Notify |  | HSS |  |
|  | Notify |  | Subscribe/Notify |  | NEF, DCSF |  |

Nimsas_Ims_Event_Exposure (Ims_EE ) service

Nimsas_Ims_EE Subscribe service operation

Service operation name: Nimsas_Ims_EE Subscribe

Description: The NF consumer subscribes to an IMS event of interest.

Inputs, Required: Public Identity, Event name.

Editors Note: The list of events are FFS.

Editors Note: Additional input is FFS

Outputs, Required: When the subscription is accepted: Subscription Correlation ID

Nimsas_Ims_EE Unsubscribe service operation

Service operation name: Nimsas_Ims_EE Unsubscribe

Description: The NF consumer unsubscribes to an IMS event

Inputs, Required: Subscription Correlation ID.

Outputs, Required: None.

Outputs, Optional: None.

##### 6.1.2.3.2 Enhancements to Nnef Services

A new service is introduced for IMS Event Exposure as depicted below:

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| imsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | HSS |
|  | Unsubscribe | Subscribe/Notify | HSS |
|  | Notify | Subscribe/Notify | NEF |

### Nnef_Ims_Event_Exposure (Ims_EE ) service

#### Nnef_Ims_EE Subscribe service operation

Service operation name: Nnef_Ims_EE Subscribe

Description: The NF consumer subscribes to an IMS event of interest.

Inputs, Required: Public Identity, Event name.

Editors Note: The list of events are FFS.

Editors Note: Additional input is FFS

Outputs, Required: When the subscription is accepted: Subscription Correlation ID

#### Nnef_Ims_EE Unsubscribe service operation

Service operation name: Nnef_Ims_EE Unsubscribe

Description: The NF consumer unsubscribes to an IMS event

Inputs, Required: Subscription Correlation ID.

Outputs, Required: None.

Outputs, Optional: None.

### 6.1.3	Impacts on existing services, entities and interfaces

Editors Note:	Additional impacts are FFS.

HSS impacts:

- Stores IMS AS instance serving a UE at initial IMS registration.

- Receive subscription from NEF for a UE for IMS related events.

- Subscribe to an IMS AS instance for a UE for IMS related events on behalf of the NEF.

IMS AS:

- Registering the IMS AS instance serving a UE at IMS initial registration

- Receive subscription from HSS for a UE for IMS related events.

- Send notifications of requested IMS events to the NEF.

NEF:

- Receive subscription from AFs for a UE for IMS related events.

- Subscribe to HSS for a UE or a group of UEs for IMS related events.

- Receive notifications of requested IMS events from the IMS AS.

- Send notifications of requested IMS events to the AF.