SA WG2 Meeting #S2-160-Ad Hoc-e	S2-2400149

22-29 January, 2024

# Title: Key issue on enhancement of UE direct or assisted AI/ML positioning

# Source: ZTE

# Document for: Approval

Agenda Item: 19.15

Abstract:	This is a key issue proposal for WT#1.4 case 1 and 2a in R19 AIML_CN

1 Discussion

The key issue is related to WT#1.4 in R19 AIML_CN:

WT1.4: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning considering the conclusions in 3GPP TR 38.843.

There are different cases to solve the problem, as defined by RAN:

- Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning:

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

START OF CHANGE

- x Key Issue #x: Enhancement of UE direct or assisted AI/ML positioning

This is the key issue corresponds to case 1 and 2a(UE direct or assisted AI/ML positioning) in WID_for NR_AIML_Air of RAN WG. In case 1, LMF sends some location assistance parameters to UE, the UE-side model output the UE location. In case 2a, the UE send some measurements data collected or generated by UE-side model to LMF, the LMF side model output UE location. To be specific, the following may be in the scope of this key issue:

-What assistance parameter(s) are needed to be generated by LMF side model and/or collected from RAN and send to UE

-What assistance parameter(s) are needed to be generated by UE side model and/or measured by UE and send them to LMF

-Whether a new analytic ID of NWDAF is needed.

-How to enhance procedures of data exchange between LMF and NWDAF. For example, the NWDAF collect necessary information and generate location assistance parameter and send to LMF, then LMF provide the assistance information to UE.

-Whether more model output performance indicators related to positioning are needed.

-How to compare the positioning accuracy with AI/ML and without AI/ML.

-When the system uses AI/ML positioning and when the system uses traditional positioning method.

- NOTE X: UE data collection for UE-side model training, model delivery/transfer to the UE and model identification/management between SA2 and RAN are not within the scope of is not in scope of this key issue.

- NOTE Y: For the core network side, the model training,validation and testing or generation of statistical result, shall be done by NWDAF.

END OF CHANGE