SA WG2 Meeting #S2-160-Ad Hoc-e	S2-2400150

22-29 January, 2024

# Title: Key issue on enhancement of NG-RAN assisted positioning

# Source: ZTE

# Document for: Approval

Agenda Item: 19.15

Abstract:	This is a key issue proposal for WT#1.4 3a in R19 AIML_CN

1 Discussion

The key issue is related to WT#1.4 in R19 AIML_CN:

WT1.4: Study whether and how to consider enhancements to LCS to support AI/ML based Positioning considering the conclusions in 3GPP TR 38.843.

There are different cases to solve the problem, as defined by RAN:

- Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- AI/ML assisted positioning:

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

START OF CHANGES

# 5.x Key Issue #x: Enhancement of NG-RAN assisted positioning

This is the key issue corresponds to case 3a in WID for NR_AIML_Air of RAN WGs, NG-RAN assisted positioning. In this key issue, the assistance data are generated by NG-RAN model, and transport to LMF or UE to assist positioning. The following may include in the scope of this key issue:

-What location assistance data need to be generated by NG-RAN model and how to send to UE for positioning.

-What location assistance data need to be generated by NG-RAN model and how to send to LMF for positioning.

-Whether more model output performance indicators related to positioning are needed.

-How to compare the positioning accuracy with AI/ML and without AI/ML.

-When the system uses AI/ML positioning and when the system uses traditional positioning method.

- NOTE X: UE data collection for UE-side model training, model delivery/transfer to the UE and model identification/management between SA2 and RAN are not within the scope of is not in scope of this key issue.

- NOTE Y: For the core network side, the model training,validation and testing or generation of statistical result, shall be done by NWDAF.

END OF CHANGES