3GPP WG2 Meeting #160AHE	S2-2400171

# 22 - 29 January, 2024, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | XXXX | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Replication and elimination for redundancy transmission for FRER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarify how the 5GC support the replication and elimination for redundancy, protocol aspects to replicate/eliminate for FRER.​ |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of replication and elimination for redundancy for FRER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GS is not supported for replication and elimination for redundancy for FRER. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.33.2.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change all the text are new* * * *

5.33.2.X	Support FRER based on Redundant User Plane Paths

When the 5GS supports TSC communication, UE(s) may establish two redundant PDU Sessions over the 5G network, such that the 5GS sets up the FRER capability for high reliability. The DS-TT and the NW-TT support the Talker and Listener functionality to enable replication and elimination for redundancy transmission based FRER, as defined in IEEE 802.1CB [XX].

In order to support IEEE 802.1CB features related to FRER, the DS-TT and NW-TT supports the following functions as defined in Clause 7.3 in IEEE 802.1CB [XX]:

-	Sequencing function;

-	Stream splitting function;

-	Individual recovery function;

-	Sequence encode/decode function;

-	Stream identification.

Plan A (by using PMIC/UMIC to enable the FRER capability):

For single UE case, support of redundant PDU Sessions for FRER include:

-	A UE initiates two redundant PDU Sessions, respectively, and provides PMIC via PDU Session Establishment Request (including the same DS-TT port MAC address, and the FRER capability of DS-TT, etc). The SMF should select the same UPF to ensure FRER based redundancy transmission.

-	The TSN AF/TSCTSF gets the PMIC/UMIC from NW-TT during PDU Session Establishment  (including the FRER capability of NW-TT, etc);

-	The TSN AF/TSCTSF maintains an association between the port number for the PDU Session and the DS-TT port MAC address (with Ethernet type PDU session) of the UE, as described in Clause 5.28.3.1. The 5GC determines whether these two PDU Session is to be handled for redundancy transmission for FRER, based on the PMIC/UMIC or local policy configuration.

-	The TSN AF/TSCTSF shall provide the FRER configurations in PMIC/UMIC to the UE/DS-TT and UPF/NW-TT for the two redundant PDU Sessions during PDU Session establishment procedure.

TSN System

TSN Bridge/End Station

DS-TT

UE

UPF

NW-TT

PDU Session1

PDU Session2

Figure 5.33.2.X-1: Single UE based FRER transmision

For two UE case, support of redundant PDU Sessions for FRER include:

-	Two UE initiate two redundant PDU Sessions, respectively, and provide PMIC via PDU Session Establishment Request  (including two different DS-TT port MAC addresses, and the FRER capability of DS-TT, etc). The SMF should select the same UPF to ensure FRER based redundancy transmission.

-	The TSN AF/TSCTSF gets the PMIC/UMIC from NW-TT during PDU Session Establishment  (including the FRER capability of NW-TT, etc);

-	The TSN AF/TSCTSF  maintains an association between the port number for the PDU Session and the DS-TT port MAC address (with Ethernet type PDU session) of the UE, as described in Clause 5.28.3.1. The 5GC determines whether these two PDU Session is to be handled for redundancy transmission for FRER, based on the PMIC/UMIC or local policy configuration .

-	The TSN AF/TSCTSF shall provide the FRER configurations in PMIC/UMIC to the UE/DS-TT and UPF/NW-TT for the two redundant PDU Sessions during PDU Session establishment procedure.

TSN System

PDU Session 2

TSN Bridge/End Station

DS-TT

UE1

UPF

NW-TT

PDU Session 1

Port1

Port2

UE2

Figure 5.33. 2.X-2: Two UEs based FRER transmission

Plan B (by using a new indication to enable the FRER capability):

For single UE case, support of redundant PDU Sessions for FRER include:

-	A UE initiates two redundant PDU Sessions, respectively, and provides the FRER capability of DS-TT via PDU Session Establishment Request. The SMF should select the same UPF to ensure FRER based redundancy transmission.

-	The SMF gets the FRER capability of NW-TT from UPF during PDU Session Establishment;

-	The SMF determines whether the PDU Session is to be handled for redundancy transmission for FRER. The determination is based on the presence of the UE capability in the PDU Session Establishment Request or the determination is based on local policy configuration.

-	The SMF shall provide the FRER indicator to the UE(s) and UPF(s) for the two redundant PDU Sessions during PDU Session establishment procedure (i.e. to enable the FRER function of DS-TT/NW-TT).

-	Operator configuration of UPF selection ensures the appropriate UPF selection for FRER based redundancy transmission.

TSN System

TSN Bridge/End Station

DS-TT

UE

UPF

NW-TT

PDU Session1

PDU Session2

Figure 5.33.2.X-2: Single UE based FRER transmission

For two UE case, support of redundant PDU Sessions for FRER include:

-	Two UE initiate two redundant PDU Sessions, respectively, and provide the FRER capability of DS-TT via PDU Session Establishment Request. The SMF should select the same UPF to ensure FRER based redundancy transmission.

-	The SMF gets the FRER capability of NW-TT from UPF during PDU Session Establishment;

-	The SMF determines whether the PDU Session is to be handled for redundancy transmission for FRER. The determination is based on the presence of the UE capability in the PDU Session Establishment Request or the determination is based on local policy configuration.

-	The SMF shall provide the FRER indicator to the UE(s) and UPF(s) for the two redundant PDU Sessions during PDU Session establishment procedure (i.e. to enable the FRER function of DS-TT/NW-TT).

TSN System

PDU Session 2

TSN Bridge/End Station

DS-TT

UE1

UPF

NW-TT

PDU Session 1

Port1

Port2

UE2

Figure 5.33.2.X-2: Two UEs based FRER transmission

# * * * * End of changes * * * *