SA WG2 Meeting #160-Ad-Hoc-e	S2-2400300

# January 22th – 29th, 2024; Elbonia               		       (revision of S2-240xxxx)

Source: 	OPPO

Title: 	23.700-45: New KI: 5GS support interworking between CAG and CSG cells.

Document for: 	Approval

Agenda Item: 	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: The intent of this proposed KI is to study how the 5GS support interworking between CAG and CSG cells.

1	Discussion

The proposed KI is applicable to WT#2 i.e.,

WT#2: How to enable interworking between CAG and CSG cells.

NOTE 2: It is expected that EPC (e.g., MME), E-UTRAN and NG-RAN impacts are avoided.

NOTE 3: It is assumed that the existing CAG concept defined for PNI-NPN is re-used for Femto access control without impacts to PNI-NPN.

Deploy Femto cell will bring many benefits for the 5G network. For example, 5G NR Femto extends e.g., coverage using higher frequency bands (e.g., FR2 bands), also improves 5G indoor coverage, offloads traffic from the macro network, enables better voice quality, and better supports enterprise mobility.

In order to support 4G/5G femto deployment, interworking should also be considered when a UE in the 5G CAG femto cell can be moved to a 4G CSG femto cell, assuming that the CAG cell is connected to the 5GC and the CSG cell is connected to the EPC in the same premise.

Based on the information described above, the following aspects need to be studied:

- How to support UE capable of both CAG and CSG access

- Whether and what are the control signalling procedure enhancements to support interworking between CAG and CSG cells

2. Proposal

It is proposed to agree the following key issue into TR 23.700-45

Start of Changes

5	Key Issues

5.X	Key Issue #X: 5GS support interworking between CAG and CSG cells.

5.X.1	Description

In order to support 4G/5G femto deployment, interworking should also be considered when a UE in the 5G CAG femto cell can be moved to a 4G CSG femto cell, assuming that the CAG cell is connected to the 5GC and the CSG cell is connected to the EPC in the same premise.

Based on the information described above, the following aspects need to be studied:

- How to support UE capable of both CAG and CSG access

- Whether and what are the control signalling procedure enhancements to support interworking between CAG and CSG cells

NOTE X: It is expected that EPC (e.g., MME), E-UTRAN and NG-RAN impacts are avoided.

NOTE Y: It is assumed that the existing CAG concept defined for PNI-NPN is re-used for Femto access control without impacts to PNI-NPN.

End of Changes