3GPP TSG-WG SA2 Meeting #160-AH	S2-2400325

# Online, January 22 - 29, 2024

Source: 	China Mobile

Title: 	New key issue: Reducing impact on central 5GC NFs to support EAS (re)discovery and UPF (re)selection

Document for: 	Approval

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: Propose a key issue about WT#1.1: Reducing impact on central 5GC NFs to support EAS (re)discovery and UPF (re)selection.

1	Discussion

This paper is to propose a new key issue according to the WT#1.1 of FS_eEDGE_5GC_ph3.

The FS_eEDGE_5GC_ph3 objectives for WT#1.1:

- WT#1.1. Handle the edge network and EAS related information (e.g. UPF and EAS deployment information, DNAIs) locally with less impact to 5GC central NFs (e.g. central SMF) to address more flexible EAS (re)discovery/(re)selection, local UPF (re)selection.

2 Proposal

It is proposed to include the below changes into TR 23.700-49 v0.0.0.

- FIRST CHANGE (all the text is new)

5.X	Key Issue #X: Reducing impact on central 5GC NFs to support EAS (re)discovery and UPF (re)selection

5.X.1	Description

Support of Rel-17/18 defined Edge Computing has heavy impact on central 5GC Control Plane NFs. For example, the edge application information(FQDN, or IP address) or local UPF information (deployment information, or load) may change dynamically or frequently, while need to be updated at the central SMF.

A method help to reduce impact on central 5GC NF to support EAS (re)discovery and UPF (re)selection need to be studied.

This key issue is to study reducing impact on 5GC NFs for edge computing handling. The following aspects should be studied:

- Whether and how the local 5GC NFs can be designed/introduced to support the edge network information control, e.g. EAS (re)discovery and UPF (re)selection.

- What edge computing related information can be locally handled to support EAS (re)discovery and UPF (re)selection

- End of CHANGE