SA WG2 Meeting #160-Ad Hoc-e	S2-2400405

E-meeting, January 22 – 29, 2024

Source:	CableLabs, Charter Communications

Title:	KI#7: New Sol: Support PDU Set QoS in wireline access

Document for:	Approval

Agenda Item:	19.3

Work Item / Release: 	FS_XRM_Ph2 / Rel-19

Abstract: This paper proposes a solution for KI #7: Support for PDU Set in non-3GPP access.

# 1. Introduction

TR 23.700-70 includes Key Issue #7: Support for PDU Set in non-3GPP access. The description of the KI states:

Support for PDU Set mechanisms are specified in TS 23.501 [2] clause 5.37.5. 5G system supports PDU Set based QoS handling in NG-RAN with the PSA UPF identifying PDUs that belong to PDU Sets based on Protocol Description for PDU Set identification and providing PDU Set Information to the RAN in the GTP-U header. Rel-18 5GS support for PDU Set based handling is limited to NG-RAN access. However, the interaction between the application and 5GS via non-3GPP access is also necessary to enhance efficiency and promote user experience. The user may be serviced by the 5GC via non-3GPP accesses such as trusted, untrusted, or wireline access, such as a device behind RG.

The objective of this Key Issue is to support PDU set based QoS Handling to non-3GPP access networks and the potential impacts of such extension on the non-3GPP access-specific intermediate nodes.

The following aspects should be studied:

- How PDU Set QoS Control mechanisms can be extended to non-3GPP access networks:

-	Support PDU set QoS in untrusted/trusted access (e.g., N3IWF, TNGF).

-	Support PDU set QoS in wireline access (e.g., W-AGF).

NOTE: 	It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Additional parameters are not precluded to support non-3GPP nodes. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

This pCR proposes a solution for KI #7.

# 2. Proposal

This contribution proposes to include Solution X in TR 23.700-70 V0.2.0.

Start of changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control framework for the 5G System; Stage 2".

[5]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)", March 2004.

[6]	IETF RFC 6904: "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)".

[7]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[8]	IETF draft-ietf-avtcore-rtp-over-quic: "RTP over QUIC (RoQ)".

[9]	IETF draft-ietf-moq-transport: "Media over QUIC Transport".

[10]	IETF experimental draft-ietf-avtext-framemarking: "Frame Marking RTP Header Extension".

[11]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[x]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

Editor's note:	References [8], [9] and [10] cannot be formally referenced until published as RFC.

Next change

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | Key Issue #7 |
| #1 |  |  |  |
| #2 |  |  |  |
| #X |  |  | X |

Next change

6.X	Solution #X: PDU Set handling in wireline access

6.X.1	Key Issue mapping

This solution proposes a solution for the following issue in KI #7:

- Support PDU set QoS in wireline access (e.g., W-AGF).

6.X.2	Description

6.X.2.1	General

To support PDU Set based QoS handling in the wireline access, PDU Set QoS Parameters and PDU Set Information need to be provided to the wireline nodes (W-AGF and RG).

The proposed solution has the following principles:

- PDU Set QoS Parameters specified for NG-RAN may be supported in wireline access.

- PDU Set related assistance information (i.e. Protocol Description) specified for NG-RAN may be supported in wireline access.

- PDU Set Information specified for NG-RAN may be supported in wireline access.

6.X.2.2	Supporting PDU Set based QoS handling in W-5GAN

The PDU Set based QoS handling by the W-5GAN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow and PDU Set information provided by the PSA UPF.

For QoS Flows with PDU Set based QoS handling enabled, PDU Set QoS Parameters are determined by the PCF (based on information provided by AF and/or local configuration) and provided by SMF to the W-AGF as part of the QoS profile. Alternatively, the SMF may be configured to support PDU Set based QoS handling without receiving PCC rules from PCF. When the W-AGF receives N2 requests related to PDU Session resources, the W-AGF maps the QoS profile(s) received from the 5GC to W-UP level QoS.

The following PDU Set QoS Parameters are used to support PDU Set based QoS handling in the W-5GAN. At least one PDU Set QoS Parameter shall be sent to the W-5GAN to enable PDU Set based QoS handling.

- PDU Set Delay Budget

The PDU Set Delay Budget (PSDB) defines an upper bound for the delay that a PDU Set may experience for the transfer between the RG and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the RG for UL) and the time when all PDUs of a PDU Set have been successfully received (at the RG for DL or N6 termination point for UL). PSDB applies to the DL PDU Set received by the PSA UPF over the N6 interface, and to the UL PDU Set sent by the RG.

If the 5G-RG supports providing of the non-3GPP delay budget for a specific QoS flow, the 5G-RG may provide a non-3GPP delay budget to SMF by using the 5G-RG requested PDU Session Modification procedure. The SMF may compensate for this delay by adjusting the dynamic CN PDB for the 3GPP network by the non-3GPP delay.

- PDU Set Error Rate

The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. W-UP or L-W-UP of a W-5GAN) but that are not successfully delivered by the corresponding receiver to the upper layer.

- PDU Set Integrated Handling Indicator

The PDU Set Integrated Handling Information (PSIHI) indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side.

The SMF may additionally signal Non-3GPP QoS Assistance Information (N3QAI) for each QoS flow to the 5G-RG. Based on the N3QAI together with QoS rule information, the 5G-RG may reserve resources in the non-3GPP network behind the 5G-RG. N3QAI may include the PDU Set QoS Parameters.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the W-AGF in the GTP-U header. The PDU Set Information comprises:

- PDU Set Sequence Number.

- Indication of End PDU of the PDU Set.

- PDU Sequence Number within a PDU Set.

- PDU Set Size in bytes.

- PDU Set Importance.

For the uplink direction, the 5G-RG identifies PDU Sets based on the Protocol Description sent by the SMF with the QoS rule.

6.X.3	Procedures

Fig. 6.X.3-1 High-level procedure of the solution for W-AGF and 5G-RG

- Steps 1-2a specified in TS 23.316 [x] clause 7.3.1.1.

- The N2 SM information carries information that the AMF shall forward to the W-AGF which includes: at least one PDU Set QoS parameter to activate PDU Set QoS handling for a given QoS flow.

The N1 SM container that the AMF shall provide to the 5G-RG contains the: For each QoS Flow for which UL PDU Set QoS handling needs to be enabled, Protocol Description for identifying the PDU Set Information.

- The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message to W-AGF to establish the access resources for this PDU Session.

- Based on the QoS flows and QoS parameters received in the previous step, W-AGF determines the corresponding PDU Set wireline QoS resource needed for the PDU session.

- The W-AGF sets up the W-UP resources for the PDU session.

- After all W-UP resources are established, the W-AGF shall forward to 5G-RG via the W-CP signalling connection the PDU Session Establishment Accept message (including the Protocol Description associated with the QoS Rule) received in step 2b.

- The W-AGF shall send to AMF an N2 PDU Session Resource Setup Response including: the PDU Set Based Handling Support Indication in N2 SM information.

- All steps specified in TS 23.502 clause 4.3.2.2.1 after step 14 are executed according to the PDU Session Establishment procedure over 3GPP access.

Fig. 6.X.3-2 High-level procedure of the solution for W-AGF and FN-RG

- Steps 0-2a specified in TS 23.316 [x] clause 7.3.4.

- The N2 SM information carries information that the AMF shall forward to the W-AGF which includes: at least one PDU Set QoS parameter to activate PDU Set QoS handling for a given QoS flow.

- The AMF shall under request of the SMF send a N2 PDU Session Resource Setup Request message to W-AGF to establish the access resources for this PDU Session.

- Based on its own policies, configuration and based on the QoS flows, QoS parameters received in the previous step, the W-AGF shall determine what W-UP resources are needed for the PDU session.

The W-AGF may perform Access specific resource reservation with the AN, that is, it sets up the W-UP resources for the PDU session.

- The W-AGF shall send to AMF an N2 PDU Session Resource Setup Response including: the PDU Set Based Handling Support Indication in N2 SM information.

- All steps specified in TS 23.502 clause 4.3.2.2.1 after step 13 are executed according to the PDU Session Establishment procedure over 3GPP access.

6.X.4	Impacts on services, entities and interfaces

SMF:

- Sending Protocol Description associated with QoS Rule to 5G-RG over N1

W-AGF:

- Mapping of PDU Set-enabled QoS profile to corresponding W-UP resource

5G-RG:

- Mapping of PDU Set-enabled QoS rule to corresponding W-UP resource

- PDU Set identification based on Protocol Description sent by SMF over N1

End of changes