3GPP TSG-WG SA2 Meeting #160AHE	S2-2400408

Online, Jan 22 – 29, 2024

Source:	Huawei, HiSilicon

Title:	Key issue for WT1 of 5G ProSe multi-hop U2N/U2U Relay support

Document for:	Approval

Agenda Item:	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract: Key issue of 5G ProSe multi-hop U2N/U2U Relay support is proposed in this contribution.

# 1. Introduction

In the agreed WT#1 of SP-231798, as shown below, the objective is to study how to support multi-hop U2N/U2U Relay.

WT#1: Enhance ProSe to support multi-hop over NR PC5 reference point

WT-1.1: for UE-to-Network Relay

WT-1.2: for UE-UE Relay

NOTE 1:	The UE-to-Network Relay include both Layer-3 and Layer-2 Relays. UE-to-UE Relay include L3 Relay only.

NOTE 2:  This study requires coordination with RAN WGs.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-03.

# * * * * First change * * * *

# 5	Key Issues

Editor's note: This clause will describe the key issues for the study item.

## 5.X	Key Issue #X: Support of multi-hop 5G ProSe UE-to-Network Relay

### 5.X.1	General description

This key issue intends to support multi-hop UE-to-Network Relay for unicast as illustrated in figure 5.X.1. The multi-hop UE-to-Network Relays include both Layer-3 and Layer-2 Relays.

This key issue addresses the following:

-	How to discover UE-to-Network Relay(s) in proximity, at least including how to:

-	whether and how to control maximum number of hops.

-  how to select UE-to-Network Relay(s).

-	How to manage multi-hop PC5 links, at least including how to establish, modify and release Layer-2 link over PC5 reference point for multi-hop UE-to-Network Relays.

-	Whether and how to satisfy End-to-End QoS when the remote UE is connecting with network via multi-hop UE-to-Network Relays.

Figure 5.X.1 Example scenario of support of multi-hop 5G ProSe UE-to-Network Relay

# * * * * Second change * * * *

## 5.Y	Key Issue #Y: Support of multi-hop 5G ProSe UE-to-UE Relay

### 5.Y.1	General description

This key issue intends to support multi-hop UE-to-UE Relay for unicast as illustrated in figure 5.Y.1. The multi-hop UE-to-UE Relay include L3 Relays only.

This key issue addresses the following:

-	How to discover UE-to-UE Relay(s) in proximity, at least including how to:

-	whether and how to control maximum number of hops.

-  how to select UE-to-UE Relay(s).

-	How to manage multi-hop PC5 links, at least including how to establish, modify and release Layer-2 link over PC5 reference point for multi-hop UE-to-UE Relays.

-	Whether and how to satisfy End-to-End QoS when two End UEs are communicating via multi-hop UE-to-UE Relays.

Figure 5.Y.1 Example scenario of support of multi-hop 5G ProSe UE-to-UE Relay

# * * * * End of changes * * * *