3GPP TSG|WG-2 Meeting #160 Ad hoc-e 	S2-2400448

22-29 Jan 2024, Emeeting	(revision of S2-xxxxxxx)

# Source:	Xiaomi

# Title:	New WID on Robust Paging Notification for NR-NTN

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WI on Robust Paging Notification for NR-NTN

Acronym:	TEI19_ Robust Notification_NR-NTN

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 80048 | Stage 1 of 5GSAT | Service requirements of satellite access in 5G |
| 960016 | Stage 1 of 5GSAT: FS_5GSAT_Ph3 Study on satellite access - Phase 3 | Defines new services requirements for R19 |
| 860005 | Integration of satellite systems in the 5G architecture (5GSAT_ARCH) | 5GS NTN functionality to be used as baseline for IoT NTN in EPS |
| 980014 | 5GC/EPC enhancement for satellite access Phase 2 (5GSAT_Ph2) | SA2 Rel-18 WID for 5G System enhancement for satellite access |
| 1010033 | Study on Integration of satellite components in the 5G architecture Phase III (FS_5GSAT_ARCH_Ph3) | SA2 Rel-19 SID for 5G System enhancement for satellite access |
|  | Non-Terrestrial Networks (NTN) for NR Phase 3 (NR_NTN_Ph3) | RAN Rel-19 WID for NR_NTN supporting Robust Paging Notification |

3	Justification

Satellite access as one of the 3GPP access is supported by 5GC/EPC. Using satellite access typically has very limited power margin and usually requires Line of Sight (LOS) for best transmission performance. For Mobile Originated service, the UE can be aware of the channel quality and can achieve LOS communication by adjusting its location if experiencing poor LOS in the current location. However, for Mobile Terminated service, the CM-IDLE UE may experience Non-Line of Sight (NLOS), i.e. locating in area with poor reception channel conditions, which might miss important paging message (e.g. paging for emergency call).

According to the study in TR 38.811, it is conclusion that at 30 degrees elevation, about 10% of users in rural areas and 50% of users in urban areas will experience NLOS. It is important to deliver paging message to those UEs under poor reception channel conditions, as this will greatly increase the success rate of MT service and improve user experience via satellite access.

To address this issue, it's noted that robust paging notification feature was proposed and discussed in RAN#102. Furthermore, RAN plenary also sends a Liaison (RP-234075/S2-2400036) to SA2 on this subject to request some inputs, guidance and requirements to this feature. In order to move forward the feature of robust notification/paging proposed by RAN WGs, it is desirable to identify potential impact on the network side.

4	Objective

Based on the above justification, it is proposed to support robust paging notification alert in 5GC with the following enhancements:

- AMF determines to apply robust paging notification for a UE after paging to the UE fails in a duration time.

- AMF sends the robust paging notification to NG-RAN in order to trigger the NG-RAN to allocate a dedicated paging alert channel to the UE.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Adding a new sub-clause to state: - Wether and how to determine the robust paging notification for a UE. - The AMF provides robust paging notification to NG-RAN in order to notify UE to move to a location with better reception condition | TBD | This will be handled as “TEI19”. |
| 23.502 | Extension in relevant clause to support determining and sending robust paging notification by AMF in the case of paging failure | TBD |  |

This TEI19_WID requires 1 TU

6	Work item Rapporteur(s)

Yuxin MAO, Xiaomi, maoyuxin1@xiaomi.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

NGAP enhancement in 38.413 by RAN3 for sending robust paging notification to NG-RAN.

9	Supporting Individual Members

Supporting IM name

Xiaomi