SA WG2 Meeting #160-Ad-Hoc-e	S2-2400523

# January 22th – 29th, 2024; Elbonia               		       (revision of S2-24xxxxx)

Source: 	CATT

Title: 	23.700-03: New KI for FS_5G_ProSe_Ph3 WT#1.1

Document for: 	Approval

Agenda Item: 	19.7

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper is to propose new key issue for FS_5G_ProSe_Ph3.

1. Proposal

It is proposed to agree the new key issue into TR 23.700-03 based on the following WT#1.1 of FS_5G_ProSe_Ph3 SID.

- WT-1: Enhance ProSe to support multi-hop over NR PC5 reference point

  - WT-1.1: for UE-to-Network Relay

  - WT-1.2: for UE-UE Relay

NOTE 1:	The UE-to-Network Relay include both Layer-3 and Layer-2 Relays. UE-to-UE Relay include L3 Relay only.

Start of Changes

5.x	Key Issue #x: Support of multi-hop over NR PC5 reference point for UE-to-Network Relay

5.x.1	Description

This key issue aims to study how to support multi-hop over NR PC5 reference point for UE-to-Network Relay based on the 5G ProSe UE-to-Network Relay as defined in TS 23.304 [xx]. The example scenario of multi-hop over NR PC5 reference point for UE-to-Network Relay is illustrated in Figure 5.x.1-1.

Figure 5.x.1-1: Example scenario of multi-hop over NR PC5 reference point for UE-to-Network Relay

The following aspects need to be studied:

-	How the network can control multi-hop over NR PC5 reference point for UE-to-Network Relay operation, at least including how to:

-	Authorize a UE to be a UE-to-Network Relay with support of multi-hop over NR PC5 reference point.

-	Authorize a UE to access 5GC via a UE-to-Network Relay with support of multi-hop over NR PC5 reference point.

-	Provisioning policy and parameters for UE-to-Network Relaying service with support of multi-hop over NR PC5 reference point.

-	How to discover and (re)-select UE-to-Network Relay(s) with support of multi-hop over NR PC5 reference point.

-	How to establish a connection between a Remote UE and a UE-to-Network Relay with support of multi-hop over NR PC5 reference point to support connectivity to the network for the Remote UE.

-	How to support end-to-end requirements between a Remote UE and the network via multiple UE-to-Network Relays, including QoS (such as data rate, reliability, latency) and the handling of PDU Session related attributes (e.g. S-NSSAI, DNN, PDU Session Type and SSC mode).

-	How the network allows and controls the QoS requirement for UE-to-Network Relay with support of multi-hop over NR PC5 reference point.

-	How to transfer data between the Remote UE and the network over multiple UE-to-Network Relays.

NOTE x:	This KI covers both Layer-2 and Layer-3 UE-to-Network Relay cases.

End of Changes